| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0243-v1 |
| --- | --- |

| Title: | AHG8: On inter-layer reference for scalability support |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Muhammed Coban, Adarsh K. Ramasubramonian, Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes modifications to the reference picture list construction and access unit definition for scalability support in VVC.

# Introduction

In this contribution, it is assumed that reference picture resampling (RPR) is already supported in VVC, it brings the ability to include reference pictures of different sizes and includes the resampling process, which can be used in the spatial scalability.

In the current VVC, it is not allowed to reference pictures from other layers for reference following the constraint that “there is a reference picture picA in the DPB with the same NuhLayerId as the current picture”. This layer Id constraint should be relaxed for inter-layer prediction, additionally a decision has to be made as to whether continuous POC or same POC should be used for the pictures in the same access unit. Two possibilities are considered in this document: pictures of all layers have continuous (unique) POC values and pictures of the same access unit have the same POC value, as for example, is the case in HEVC.

Picture marking process should also be modified considering NuhLayerId of the pictures. In this contribution, it is proposed that:

- a previously decoded picture can be marked as “unused for reference” when a picture of the same layer Id is decoded.

- a picture with a layer Id A may mark pictures with a layer Id B if B > A as “unused for reference”, when the layer with layer Id B is dependent (direct on indirect) on the layer with the layer ID A. In one example, a picture with the layer Id A can be an IDR picture (at a cross layer random access point), in this case the picture of the dependent layer Id B (that are previously decoded) can be marked as “unsued for reference”.

# VPS indication of the layer dependency

To indicate the dependency across layers, it is proposed to add this information into VPS.

| video_parameter_set_rbsp( ) | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| if( i > 0 ) { |  |
| vps_independent_layer_flag[ i ] | u(1) |
| if(!vps_independent_layer_flag[ i ]) |  |
| for( j = 0; j < i; j++ ) |  |
| vps_direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| vps_reserved_zero_bit | u(1) |
| } |  |
| … |  |

vps_independent_layer_flag[ i ] equal to 1 specifies that the layer with index i does not use inter-layer prediction. vps_independent_layer_flag[ i ] equal to 1 specifies that the layer with index i may use inter-layer prediction and vps_layer_dependency_flag[ i ] is present in VPS.

vps_direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. vps_direct_dependency_flag [ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When vps_direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to MaxLayersMinus1, inclusive, it is inferred to be equal to 0.

The variable DependencyIdc[ i ][ j ] is derived as follows:

for( i = 1; i <= MaxLayersMinus1; i -- )

if( !vps_independent_layer_flag[ i ] )

for( j = MaxLayersMinus1 – 1, k = 0; j >= 0; j -- )

if( vps_direct_dependency_flag[ i ][ j ] )

DependencyIdc[ i ][ k++ ] = j

The variable LayerIdc[ i ][ j ] is derived as follows:

for( i = 0, j = 0; i  <=  vps_max_layers_minus1; i++, j++ ) {

LayerIdc[ vps_included_layer_id[ i ] ][ j ] = j
}

When reference picture list is constructed, it should be consistent with the VPS indication, and no pictures of the layers not specified as dependent layers shall be present in the reference picture list or being used for inter-layer prediction.

# Continuous POC values

In this section, all pictures have continuous POC numbering regardless of the layer Id as shown on the Figure 1.

Figure 1. Continuous POC numbering across layers.

There are no reference pictures having the same POC values, so the reference picture structure (RPS) and reference picture list (RPL) construction can stay unchanged.

However, access unit (AU) definition should be modified, since in the current VVC the new AU starts when a different POC value in VCL NAL unit is encountered. Layer Id is proposed to be added into the AU definition, so the new AU starts when the smaller or equal NuhLayerId is encountered in the next VCL NAL unit.

In one example, the proposed modification is highlighted in yellow below.

When there is none of the above NAL units preceding firstVclNalUnitInAu and succeeding the last VCL NAL preceding firstVclNalUnitInAu, if any, firstVclNalUnitInAu starts a new access unit if NuhLayerId is smaller or equal to the NuhLayerId of the previous coded picture.

# Same POC values

In this section, the alternative to the continuous POC numbering is proposed for enabling scalability.

Figure 2. The same POC numbering across layers.

In this case, AU definition may stay unchanged, since the new POC value VCL NAL unit indicates the next AU. However, adding the same POC value reference picture with a different layer Id should be supported.

Similar to the inter reference picture case, it is proposed to add a number of the inter-layer reference pictures to the ref_pic_list_struct. Inter-layer reference picture is marked as long term reference picture (LTRP), the inter-layer refence picture flag is added to the LTRP and the layer Id infomration is signaled if the flag indicates inter-layer refernece picture.

The SPS level flag indicating whether inter-layer prediction is applied is added.

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( sps_interlayer_ref_pics_present_flag ) |  |
| num_inter_layer_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| numILEntries = 0 |  |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ] + num_inter_layer_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| if( numILEntries < num_inter_layer_ref_entries[ listIdx ][ rplsIdx ] ) { |  |
| il_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( il_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| layer_dependency_idc[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| numILEntries++ |  |
| } |  |
| } |  |
| if( !il_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j ] | u(v) |
| j++ |  |
| } |  |
| } |  |

sps_inter_layer_ref_pics_present_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the pictures that refer to the SPS. Sps_inter_layer_ref_pics_present_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the pictures that refer to the SPS.

num_inter_layer_ref_entries[ listIdx ][ rplsIdx ] specifies the number of direct inter-layer entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The value of num_inter_layer_ref_entries[ listIdx ][ rplsIdx ] shall be in the range of 0 to sps_max_dec_pic_buffering_minus1 + 14 – num_ref_entries[ listIdx ][ rplsIdx ], inclusive. When num_inter_layer_ref_entries[ listIdx ][ rplsIdx ] is not present, num_inter_layer_ref_entries[ listIdx ][ rplsIdx ] is inferred to be equal to 0.

il_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 1 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an inter-layer reference picture. il_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 0 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is not inter-layer refernce picture entry. When not present, the value of il_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to 0.

layer_dependency_idc[ listIdx ][ rplsIdx ][ i ] specifies the dependency layer index of the i-th entry in in ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The value of layer_dependency_idc[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to the current layer idc LayerIdc[nuh_layer_id] minus 1, inclusive.

The reference picture list is constructed by allowing a LTRP with the layer Id different from the current picture layer Id be present.

The reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are constructed as follows:

for( i = 0; i < 2; i++ ) {
		for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ] + num_inter_layer_ref_entries[ listIdx ][ rplsIdx ]; j++) {
			if( il_ref_pic_flag [ i ][ RplsIdx[ i ] ][ j ] ) {
				refNuhLayerId = DependencyIdc[ LayerIdc[NuhLayerId] ][ layer_dependency_idc[ i ][ RplsIdx[ i ] ][ j ] ][ j ] ]
					if( there is a reference picA in the DPB with the same PicOrderCntVal and the NuhLayerId equal to refNuhLayerId )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
			} else {
					if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
					RefPicPocList[ i ][ j ] = pocBase − DeltaPocSt[ i ][ RplsIdx[ i ] ][ j ]
					if( there is a reference picture picA in the DPB with the same NuhLayerId as the current picture and
									PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"					(8-5)
					pocBase = RefPicPocList[ i ][ j ]
				} else {
					if( !delta_poc_msb_cycle_lt[ i ][ k ] ) {
						if( there is a reference picA in the DPB with the same NuhLayerId as the current picture and
									PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) equal to PocLsbLt[ i ][ k ] )
							RefPicList[ i ][ j ] = picA
						else
							RefPicList[ i ][ j ] = "no reference picture"
					} else {
						if( there is a reference picA in the DPB with the same NuhLayerId as the current picture and 									PicOrderCntVal equal to 	FullPocLt[ i ][ k ] )
							RefPicList[ i ][ j ] = picA
						else
							RefPicList[ i ][ j ] = "no reference picture"
					}
					k++
			}
		}
	}
}

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).