| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0245-v1 |
| --- | --- |

| Title: | AHG17: On TemporalId and NuhLayerId in APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Muhammed Coban, Adarsh K. Ramasubramonian, Nan Hu, Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to modify TemporalId assignment for APS NAL unit and to exclude the usage of other layer APS. In the current VVC draft, TemporalId of APS NAL unit shall be equal to that of the layer access unit containing the NAL unit and it is allowed to use other layer APS.

# Introduction

In the current VVC draft, TemporalId of APS NAL unit shall be equal to the TemporalId of the layer access unit (AU) containing the NAL unit. However, in low latency encoder when ALF filter coefficients are derived from the current picture but are applied to the next picture, APS may be signalled after the VCL NAL unit of the AU from which the coefficients are derived. Following the current TemporalId assignment for APS, it shall be equal to the TemporalId of the next AU, which may have higher TemporalId than the VCL NAL unit from which this APS is derived, so it cannot be used for pictures having that lower TemporalId.

Also, there is no restriction on the APS usage having different NuhLayerId, for example there can be multiple layers and APS can be referenced across the layers, so when a layer is extracted the APS NAL units of other layers could be removed from the bitstream.

# APS TemporalId assignment

To solve the problem with APS TemporalId assignment, it is proposed to decouple APS TemporalId assignment from the AU TemporalId, so it can be an encoder choice. In this case, the following language is removed from the specification draft.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If NalUnitType is equal to SPS_NUT, TemporalId is equal to 0 and the TemporalId of the layer access unit containing the NAL unit shall be equal to 0.

–	Otherwise, if NalUnitType is equal to APS_NUT, TemporalId shall be equal to that of the layer access unit containing the NAL unit.

–	Otherwise, when NalUnitType is not equal to EOS_NUT and not equal to EOB_NUT, TemporalId shall be greater than or equal to the TemporalId of the layer access unit containing the NAL unit.

In the alternative solution, TemporalId assignment is modified to include the possibility to assign lower TemporalId than the current AU TemporalId.

The value of TemporalId for non-VCL NAL units is constrained as follows:

–	If NalUnitType is equal to SPS_NUT, TemporalId is equal to 0 and the TemporalId of the layer access unit containing the NAL unit shall be equal to 0.

–	Otherwise, if NalUnitType is equal to APS_NUT, TemporalId shall be less or equal to that of the layer access unit containing the NAL unit.

–	Otherwise, when NalUnitType is not equal to EOS_NUT and not equal to EOB_NUT, TemporalId shall be greater than or equal to the TemporalId of the layer access unit containing the NAL unit.

# NuhLayerId constraint in APS

To disable the usage of other layer APS, which can be removed during the sub-bitstream extraction process, it is proposed to add a constraint similar to TemporalId contstraint but applied to NuhLayerId in the APS syntax semantics.

It is also proposed to extend the APS id restriction to slice_alf_aps_id_chroma, which is already applied to slice_alf_aps_id_luma.

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the slice refers to. The TemporalId of the ALF APS NAL unit having adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the coded slice NAL unit.

The NuhLayerId of the ALF APS NAL unit having adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to the NuhLayerId of the coded slice NAL unit.

When multiple ALF APSs with the same value of adaptation_parameter_set_id are referred to by two or more slices of the same picture, the multiple ALF APSs with the same value of adaptation_parameter_set_id shall have the same content.

For intra slices and slices in an IRAP picture, slice_alf_aps_id_luma[ i ] shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id that the chroma component of the slice refers to. When slice_alf_aps_id_chroma is not present, it is inferred to be equal to slice_alf_aps_id_luma[ 0 ]. The TemporalId of the ALF APS NAL unit having adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be less than or equal to the TemporalId of the coded slice NAL unit.

When multiple ALF APSs with the same value of adaptation_parameter_set_id are referred to by two or more slices of the same picture, the multiple ALF APSs with the same value of adaptation_parameter_set_id shall have the same content.

The NuhLayerId of the ALF APS NAL unit having adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be equal to the NuhLayerId of the coded slice NAL unit.

For intra slices and slices in an IRAP picture, slice_alf_aps_id_chroma shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

slice_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slice refers to. The TemporalId of the LMCS APS NAL unit having adaptation_parameter_set_id equal to slice_lmcs_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit.

When multiple LMCS APSs with the same value of adaptation_parameter_set_id are referred to by two or more slices of the same picture, the multiple LMCS APSs with the same value of adaptation_parameter_set_id shall have the same content.

The NuhLayerId of the LMCS APS NAL unit having adaptation_parameter_set_id equal to slice_lmcs_aps_id shall be equal to the NuhLayerId of the coded slice NAL unit.

If inter-layer prediction is allowed, then the proposed NuhLayerId constraints can be relaxed to include direct dependent NuhLayerId.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).