| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0246-v1 |
| --- | --- |

| Title: | AHG17: Separate luma and chroma ALF APS types |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Nan Hu, Muhammed Coban, Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current VVC, there are two APS types: ALF and LMCS. In the ALF APS, luma and chroma filter coefficients are signalled. It is possible to have only luma or only chroma coefficients in an APS. However, when an APS replaces the old APS with the same Id, it is possible to replace the APS having both luma and chroma coefficients with just an APS having coefficients for only one color component. In such case, the coefficients of other component in the old APS are lost. This contribution proposes to split ALF APS type into luma and chroma ALF APS types.

# Introduction

In the current VVC, ALF APS contains filter coefficients for both luma and chroma color components and the same APS Id is used to refer to those filter coefficients. However, it is also possible to have coefficients for only one color component, which is controlled by alf_luma_filter_signal_flag and alf_chroma_filter_signal_flag. When an APS is referred to, the same APS Id is used for luma and chroma components, so when only one component filter coefficients are present in the new APS with the same Id, the other component filter coefficients in the old APS will be lost, but potentially could be used.

This contribution proposes to split ALF APS type into luma and chroma APS types, i.e. aps_params_type represents ALF_LUMA_APS, ALF_CHROMA_APS, and LMCS_APS.

Then, alf_luma_filter_signal_flag and alf_chroma_filter_signal_flag are no longer needed and are removed from the alf_data().

When an APS is referred to, then both adaptation_parameter_set_id and aps_params_type are used to identify the APS. The total APS storage memory stays unchanged.

# Specification change

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 7-2.

Table 7-2 – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_LUMA_APS | Luma ALF parameters |
| 1 | ALF_CHROMA_APS | Chroma ALF parameters |
| 1 2 | LMCS_APS | LMCS parameters |
| 2 3..7 | Reserved | Reserved |

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_LUMA_APS ¦¦ aps_params_type = = ALF_CHROMA_APS) |  |
| alf_data( adaptation_parameter_set_id, aps_params_type ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| alf_data( adaptation_parameter_set_id, aps_params_type ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag aps_params_type = = ALF_LUMA_APS ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | tb(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | tb(v) |
| } |  |
| alf_luma_use_fixed_filter_flag | u(1) |
| if( alf_luma_use_fixed_filter_flag ) { |  |
| alf_luma_fixed_filter_set_idx | tb(v) |
| alf_luma_fixed_filter_pred_present_flag | u(1) |
| if( alf_luma_fixed_filter_pred_present_flag ) { |  |
| for( i = 0; i < NumAlfFilters; i++ ) |  |
| alf_luma_fixed_filter_pred_flag[ i ] | u(1) |
| } |  |
| } |  |
| alf_luma_coeff_delta_flag | u(1) |
| if( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| alf_luma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if( alf_luma_coeff_delta_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_delta_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_delta_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_delta_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| else if( alf_chroma_filter_signal_flag aps_params_type = = ALF_CHROMA_APS ) { |  |
| alf_chroma_clip_flag | u(1) |
| alf_chroma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] | u(1) |
| } |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v) |
| } |  |
| } |  |
| } |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | tb(v) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(5) |
| slice_alf_chroma_idc | tu(v) |
| if( slice_alf_chroma_idc && ( slice_type != I ¦ ¦ slice_num_alf_aps_ids_luma != 1) ) |  |
| slice_alf_aps_id_chroma | u(5) |
| } |  |
| } |  |
| … |  |

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th luma ALF APS that the slice refers to. The TemporalId of the luma ALF APS NAL unit having adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the coded slice NAL unit.

When multiple luma ALF APSs with the same value of adaptation_parameter_set_id are referred to by two or more slices of the same picture, the multiple luma ALF APSs with the same value of adaptation_parameter_set_id shall have the same content.

For intra slices and slices in an IRAP picture, slice_alf_aps_id_luma[ i ] shall not refer to a luma ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id that the chroma component of the slice refers to.When slice_alf_aps_id_chroma is not present, it is inferred to be equal to slice_alf_aps_id_luma[ 0 ]. The TemporalId of the chroma ALF APS NAL unit having adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be less than or equal to the TemporalId of the coded slice NAL unit.

For intra slices and slices in an IRAP picture, slice_alf_aps_id_chroma shall not refer to a chroma ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).