| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0248 |
| --- | --- |

| Title: | CE8-related: dedicated IBC reference buffer without bitstream restrictions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Alexander Karabutov Semih Esenlik Sergey Ikonin Biao Wang Anand Meher Kotra Elena Alshina | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# WD Compared with JVET-N1001-v8

### 8.6.1 General decoding process for coding units coded in IBC prediction mode

…

- It is a requirement of bitstream conformance that the chroma motion vector mvC[ xSbIdx ][ ySbIdx ] shall obey the following constraints:

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ), yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) ) as inputs, the output shall be equal to TRUE.

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + cbWidth / SubWidthC − 1, yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + cbHeight / SubHeightC − 1 ) as inputs, the output shall be equal to TRUE.

  - One or both of the following conditions shall be true:

    - ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + xSbIdx * 2 + 2 is less than or equal to 0.

    - ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + ySbIdx * 2 + 2 is less than or equal to 0.

…

### 8.6.2	Derivation process for motion vector components for IBC blocks

### 8.6.2.1	General

…

It is a requirement of bitstream conformance that the luma motion vector mvL shall obey the following constraints:

- The motion vector mvL shall point to a reference block that is available, i.e. for ( xRefTL, yRefTL ) and ( xRefBR, yRefBR ), all of the following conditions shall be true:

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xRefTL, yRefTL ) as inputs, the output shall be equal to TRUE.

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xRefBR, yRefBR ) as inputs, the output shall be equal to TRUE.

- The motion vector mvL shall point to a reference block that is outside (left and/or above) and not overlapping with the current coding bock, i.e. one or both the following conditions shall be true:

  - The value of ( mvL[ 0 ] >> 4 ) + cbWidth is less than or equal to 0.

  - The value of ( mvL[ 1 ] >> 4 ) + cbHeight is less than or equal to 0.

- The motion vector mvL shall point to a reference block that is fully contained in the same coding tree block as the current block or fully contained in a block to the left with the same height of the current coding tree block and a width equal to 128 luma samples, i.e. all of the following conditions shall be true:

yRefTL >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-893)

yRefBR >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-894)

xRefTL >> CtbLog2SizeY >= ( xCb >> CtbLog2SizeY ) + Min( 1, 7 − CtbLog2SizeY )	(8-895)
														− ( 1 << ( (7 − CtbLog2SizeY ) << 1) ) )

xRefBR >> CtbLog2SizeY <= ( xCb >> CtbLog2SizeY )	(8-896)

- When the motion vector mvL points to a reference block where the top-left sample is located inside the neighbouring coding tree block to the left, i.e. ( xRefTL ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) − 1, and CtbSizeY is equal to 128, the motion vector mvL shall not point to a reference block where the top-left sample is located left to or inside the same 64x64 area as the current block but inside the neighbouring coding tree block to the left, i.e. the following conditions shall be true:

  - The derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( ( xRefTL + 128 ) / 64 * 64, yRefTL / 64 * 64 ) as inputs, and the output shall be equal to FALSE.

  - The luma location ( ( xRefTL + 128 ) / 64 * 64, yRefTL / 64 * 64 ) shall not be equal to ( xCb, yCb ).

…

### 8.6.3 Decoding process for ibc blocks

#### 8.6.3.1 General

…

Let predSamplesL0L, and predSamplesL1L be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr and predSamplesL1Cr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

The reference indices refIdxL0 and refIdxL1 are set equal to −1.

The prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] are set equal to 1 and predFlagL1[ xSbIdx ][ ySbIdx ] are set equal to 0 with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

The bi-prediction weight index bcwIdx is set equal to 0,

…

The current decoded picture consists of a pic_width_in_luma_samples by pic_height_in_luma_samples array of luma samples currPicL and two PicWidthInSamplesC by PicHeightInSamplesC arrays of chroma samples currPicCb and currPicCr. The current decoded picture sample arrays currPicL, currPicCb and currPicCr correspond to decoded sample arrays SL, SCb and SCr derived in clause 8.8 for the current decoded picture.

- If cIdx is equal to 0, the following applies:

  - The array predSamplesL0L is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth, the coding subblock height sbHeight in luma samples, the motion vector offset mvOffset set equal to ( 0, 0 ), the luma motion vector mv[ xSb ][ ySb ], the reference array currPicL, bdofFlag, and cIdx as inputs.

- Otherwise if cIdx is equal to 1, the following applies:

  - The array predSamplesL0Cb is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the motion vector offset mvOffset set equal to ( 0, 0 ), the chroma motion vector mv[ xSb ][ ySb ], the reference array currPicCb, bdofFlag, and cIdx as inputs.

- Otherwise (cIdx is equal to 2), the following applies:

  - The array predSamplesL0Cr is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the motion vector offset mvOffset set equal to ( 0, 0 ), the chroma motion vector mv[ xSb ][ ySb ], the reference array currPicCr, bdofFlag, and cIdx as inputs.

- The array predSamples of prediction samples is derived as follows:

- If cIdx is equal to 0, the prediction samples inside the current luma coding subblock, predSamples[ xL + xSb ][ yL + ySb ] with xL = 0..sbWidth − 1 and yL = 0..sbHeight − 1, are derived by invoking the weighted sample prediction process as specified in clause 8.5.6.5 is invoked with the luma coding subblock width sbWidth, the luma coding subblock height sbHeight and the sample arrays predSamplesL0 L and predSamplesL1L, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs, and predSamples[ xL + xSb ][ yL + ySb ] as outputs.

- Otherwise, if cIdx is equal to 1, the prediction samples inside the current chroma component Cb coding block, predSamples [ xC + xCb / 2 ][ yC + yCb / 2 ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process specified in clause 8.5.6.5 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesL0Cb and predSamplesL1Cb, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs.

- Otherwise (cIdx is equal to 2), the prediction samples inside the current chroma component Cr coding block, predSamples[ xC + xCb / 2 ][ yC + yCb / 2 ] with xC = 0..cbWidth / 2 − 1 and yC = 0..cbHeight / 2 − 1, are derived by invoking the weighted sample prediction process specified in clause 8.5.6.5 with the coding block width nCbW set equal to cbWidth / 2, the coding block height nCbH set equal to cbHeight / 2, the sample arrays predSamplesL0Cr and predSamplesL1Cr, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, bcwIdx, and cIdx as inputs.

- When cIdx is equal to 0, the following assignments are made for x = 0..sbWidth − 1 and y = 0..sbHeight − 1:

MvL0[ xSb + x ][ ySb + y ] = mv[ xSbIdx ][ ySbIdx ]	(8-916)

MvL1[ xSb + x ][ ySb + y ] = 0		(8-917)

RefIdxL0[ xSb + x ][ ySb + y ] = refIdxL0		(8-918)

RefIdxL1[ xSb + x ][ ySb + y ] = refIdxL1		(8-919)

PredFlagL0[ xSb + x ][ ySb + y ] = predFlagL0[ xSbIdx ][ ySbIdx ]	(8-920)

PredFlagL1[ xSb + x ][ ySb + y ] = predFlagL1[ xSbIdx ][ ySbIdx ]	(8-921)

BcwIdx[ xSb + x ][ ySb + y ] = bcwIdx		(8-922)

The variable ibcBufWidthInCtb is equal to 128*128/CtbSizeY/CtbSizeY.

The ibc reference buffer consists of a ibcBufWidthInCtb*CtbSizeY by CtbSizeY array of luma samples ibcRefL and two ibcBufWidthInCtb *CtbWidthC by CtbHeightC arrays of chroma samples ibcRefCb and ibcRefCr.

If xCb is equal to 0 and yCb is equal to ( yCb >> CtbLog2SizeY  ) << CtbLog2SizeY

ibcRefM [x] [y] = 1 << (internalBitDepth - 1).

for x=0,1,…, ibcBufWidthInCtb* CtbSizeY -1 and y=0,1,…, CtbSizeM -1

where M is presented for L, Cb or Cr

If CtbLog2SizeY  is equal to 7 and xCb is equal to  (xCb >> MaxTbSizeY ) << MaxTbSizeY and yCb is equal to  (yCb >> MaxTbSizeY ) << MaxTbSizeY

ibcRefM [xCb + x] [yCb  + y] = 1 << (internalBitDepth - 1).

for x=0,1,…, (cbWidth  == CtbSizeY? 2:1) * MaxTbSizeY -1 and y=0,1,…, (cbHeight  == CtbSizeY? 2:1) * RefreshSizeY -1

where M is presented for L, Cb or Cr

if cIdx is equal to 0

predSamples[xL+xSb][yL+ySb] =

ibcRefL[(xL+xSb+(mv[xSb][ySb][0]>>4))%(ibcBufWidthInCtb*CtbSizeY)][(yL+ySb+(mv[xSb][ySb][0]>>4))%CtbSizeY]

for xL=0,…,sbWidth-1 and yL=0,…,sbHeight-1

otherwise, if cIdx is equal to 1

predSamples[xL+xSb][yL+ySb]= ibcRefCb[(xL+xSb+(mv[xSb][ySb][0]>>4))%(ibcBufWidthInCtb*CtbSizeY)][(yL+ySb+(mv[xSb][ySb][0]>>4))%CtbSizeY]

otherwise, if cIdx is equal to 2

predSamples[xL+xSb][yL+ySb]=

ibcRefCb[(xL+xSb+(mv[xSb][ySb][0]>>4))%(ibcBufWidthInCtb*CtbSizeY)][(yL+ySb+(mv[xSb][ySb][0]>>4))%CtbSizeY]

for xL=0,…,sbWidth-1 and yL=0,…,sbHeight-1

One item added to reconstruction process.

When a cbWidth*cbHeight coding block starting from (xCb, yCb) has been reconstructed, the ibc referecne buffer is updated as

If cIdx is euqal to 0

ibcRefL[(xCb+x)%(ibcBufWidthInCtb*CtbSizeY)][(yCb+y)%CtbSizeY] 
= (SL[xCb+x][yCb+y]+(1<<(bdDepth-1))>>bdShiftY

for x=0,1,…,cbWidth-1 and y=0,1,…,cbHeight-1

bdShiftY is derived as BitDepthY-8.

Otherwise if cIdx is equal to 1

ibcRefCb[(xCb+x)%(ibcBufWidthInCtb*CtbSizeC)][(yCb+y)%CtbSizeC] 
= (SCb[xCb+x][yCb+y]+(1<<(bdDepth-1))>>bdShiftC

for x=0,1,…,cbWidth-1 and y=0,1,…,cbHeight-1

Othereise if cIdx is equal to 2

ibcRefCr[(xCb+x)%(ibcBufWidthInCtb*CtbSizeC)][(yCb+y)%CtbSizeC] 
= (SL[xCb+x][yCb+y]+(1<<(bdDepth-1))>>bdShiftC

for x=0,1,…,cbWidth-1 and y=0,1,…,cbHeight-1

bdShiftC is derived as BitDepthC-8.

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).