| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0248 |
| --- | --- |

| Title: | CE8-related: Dedicated IBC reference buffer without bitstream restrictions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Alexander Karabutov Semih Esenlik Sergey Ikonin Biao Wang Anand Meher Kotra Elena Alshina | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

In the current VVC Draft 5, IBC mode reference area comprises 3 VPDUs from left CTU and the current VPDU in current CTU. However, many bitstream restrictions are necessary to make sure the block vector (BV) points to a valid area. In the CE test CE8-1.6, the concept of dedicated IBC reference buffer is introduced which reduce the number of the block vector restrictions (i.e. necessary conformance checks in the encoder). This proposal is based on CE8-1.6 and provides a simple IBC buffer concept that remove all of the BV bitstream restrictions. It is asserted that considerable simplification in draft text is achieved with no impact on coding performance. Furthermore the additional memory requirement for IBC mode (3 VPDUs) is not changed.

# Introduction

## Current VTM design

In VVC Draft 5 [1], the reference samples area of IBC is limited to a pre-defined area inside of the current CTU and left CTU as shown in Fig. 1.

Figure 1. Reference samples area of IBC VVC Draft 5

The reference sample area of IBC is defined with bitsteam restrictions. The luma component bitstream restriction of IBC reference sample area is in VVC Draft 5 [1] shown as:

It is a requirement of bitstream conformance that the luma motion vector mvL shall obey the following constraints:

- The motion vector mvL shall point to a reference block that is available, i.e. for ( xRefTL, yRefTL ) and ( xRefBR, yRefBR ), all of the following conditions shall be true:

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xRefTL, yRefTL ) as inputs, the output shall be equal to TRUE.

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xRefBR, yRefBR ) as inputs, the output shall be equal to TRUE.

- The motion vector mvL shall point to a reference block that is outside (left and/or above) and not overlapping with the current coding bock, i.e. one or both the following conditions shall be true:

  - The value of ( mvL[ 0 ] >> 4 ) + cbWidth is less than or equal to 0.

  - The value of ( mvL[ 1 ] >> 4 ) + cbHeight is less than or equal to 0.

- The motion vector mvL shall point to a reference block that is fully contained in the same coding tree block as the current block or fully contained in a block to the left with the same height of the current coding tree block and a width equal to 128 luma samples, i.e. all of the following conditions shall be true:

yRefTL >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-893)

yRefBR >> CtbLog2SizeY = yCb >> CtbLog2SizeY	(8-894)

xRefTL >> CtbLog2SizeY >= ( xCb >> CtbLog2SizeY ) + Min( 1, 7 − CtbLog2SizeY )	(8-895)
	− ( 1 << ( (7 − CtbLog2SizeY ) << 1) ) )

xRefBR >> CtbLog2SizeY <= ( xCb >> CtbLog2SizeY )	(8-896)

- When the motion vector mvL points to a reference block where the top-left sample is located inside the neighbouring coding tree block to the left, i.e. ( xRefTL ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) − 1, and CtbSizeY is equal to 128, the motion vector mvL shall not point to a reference block where the top-left sample is located left to or inside the same 64x64 area as the current block but inside the neighbouring coding tree block to the left, i.e. the following conditions shall be true:

  - The derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( ( xRefTL + 128 ) / 64 * 64, yRefTL / 64 * 64 ) as inputs, and the output shall be equal to FALSE.

  - The luma location ( ( xRefTL + 128 ) / 64 * 64, yRefTL / 64 * 64 ) shall not be equal to ( xCb, yCb ).

In summary, the following bitstream conformance checks are necessary:

- x + MVx <=0; y + MVy <=0

- Reference block should be in current or left CTU

- Reference block should be in the VPDU based updating reference area

- Reference block and current block should be in the same CTU row

- Reference block should be available (already reconstructed)

- Reference block should inside of the picture boundary

## Dedicated IBC reference buffer

A dedicated IBC reference buffer is introduced in JVET-N0472 [2] and CE8-1.6 [3] which partially solve the issue about excessive BV conformance checks. The concept of dedicated IBC buffer is explained briefly as follows.

Assume that a pixel (x,y) is coded in IBC mode with BV=(BVx, BVy), it is prediction sample in the IBC reference buffer is located at ((x+BVx)%CTUsize, (y+BVy)%(1282/CtuSize )).

- Decoding Buffer                                    (b) IBC dedicated buffer

Figure 2, IBC dedicated buffer of JVET-N0472

In its core, 2 mechanisms are introduced by dedicated buffer concept:

- A virtual memory addressing mechanism, such that a block vector coordinate is translated to a virtual address inside a 128 by 128 square block (e.g. 64 by 256 block in the case of smaller 64x64 CTU)

- Reference sample wraparound mechanism, in which a sample exceeding right or bottom boundary of dedicated buffer is wrapped around.

Both mechanisms are implemented by the modulo operation in the formula above.

With the dedicated buffer design, the bitstream restrictions of luma IBC reference area is partially removed as:

- x + MVx <=0; y + MVy <=0

- Reference block should be in current or left CTU

- Reference block should be in the VPDU based updating reference area

- Reference block and current block should be in the same CTU row

- Reference block should be available (already reconstructed)

- Reference block should be inside of the picture boundary

# Problem Statement

Too many bistream restrictions is not safe for decoder, because the decoder easily receives evil bistream. Moving the condition checks to the decoder side is also not desirable, since too many condition checks heavily burden the decoder.

The dedicated buffer design in JVET-N0472 [2] is partially solved the bitstream restrictions problem. But a 128x128 10bits dedicated buffer increases the memory requirement compared with the IBC reference area design in VTM5.0. The following are the bitstream conformance checks that can cannot be removed by CE8-1.6:

- x + MVx <=0; y + MVy <=0

- Reference block should be in current or left CTU

- Reference block should be in the VPDU based updating reference area

- Reference block and current block should be in the same CTU row

- Reference block should be available (already reconstructed)

- Reference block should be inside of the picture boundary

# Proposed Method

In the proposal, the concept of IBC dedicated buffer is improved to remove all of the bitstream restrictions.

The following operations in CE8-1.6 are used as they are in the current proposal:

- Dedicated buffer width is equal to 1282/CtuSize, and height is equal to CtuSize.

- A reconstructed block located at (x, y) before loop filter is written to (x%128, y%128) in the dedicated buffer.

- During IBC referencing, the block located at ((x + BVx)%(1282/CtuSize), (y + BVy)% CtuSize) in the dedicated buffer is used as reference block of a current block (x, y) with block vector (BVx, BVy)

In addition to the CE8-1.6 the following IBC buffer refresh rules are introduced:

- Whole dedicated buffer will be refreshed with a default value (1 << (InternalBitDepth - 1), when the current block is the first CU of a CTU row.

- When the current block is the first CU of the current VPDU,

  - The current VPDU area of the dedicated buffer is refreshed if CU is smaller than or equal to one VPDU.

  - 2 VPDU areas of the dedicated buffer are refreshed if CU is greater than one VPDU.

Finally all block vectors are considered valid according to the proposal. The introduced buffer refresh rules guarantee that there is no invalid section in the dedicated buffer. All unavailable samples are replaced with default value (1 << (InternalBitDepth - 1) according to two refresh rules.

With this proposed dedicated IBC buffer, all luma and choma IBC bistream conformance checks are removed, no decoder side checks are necessary either. The only increase in decoder complexity is application of a modulo operation.

## Example	application of the proposal

Fig.3 (a) shows the dedicated buffer introduced by CE8-1.6. Because of the remaining bistream restrictions, the block vectors that are represented by dashed lines are invalid. Those invalid BVs point one of:

- non-reconstructed area in the current CTU row.

- a position outside of the current CTU row

- a position outside of the picture boundary

Fig. 3a: Invalid BVs and the dedicated IBC buffer according to CE8-1.6

Fig 3 (b) shows the proposed dedicated buffer, all block vectors are valid since all bitstream restrictions are removed. In the figure, all 4 example block vectors point to the same location in the dedicated buffer due to the modulo operation.

Fig. 3b: All BVs are valid according to the current proposal

## Dual tree chroma operation

For dual tree case, the luma and chroma components dedicated IBC buffer is refreshed separately using the same set of rules, therefore there is no invalid chroma BV. In the case when the collocated luma block of a chroma subblock (2x2) is not coded with IBC mode, the chroma BV is set to (0,0).

## Benefits of the proposed dedicated IBC buffer

The following benefits shown in the proposed method:

- Compared with VTM5.0 or the CE8-1.6 (JVET-N0472), all of the bitstream restrictions are removed, which guarantees a safer bistream for the decoder.

- By applying the CTU row refreshing rules, no samples from the last CTU rows are involved for IBC reference, guaranteeing wavefront based encoding/decoding.

- By applying the current VPDU refreshing rules, the memory requirement for IBC referencing is same as the current VMT 5.0 design. In addition, compared with CE8-1.6, for referencing a sample in current VPDU, the dedicated IBC buffer is no need to be accessed.

- The chroma component also uses the exact same method as luma (in the case of dual tree), ensuring uniform operation.

- Almost 2 pages of specification text is removed.

# Simulation results

## Simulation results

Table 1 shows the simulations results of the proposed dedicated IBC buffer. Since the dedicated buffer has not effect of the encoder design, there is no coding performance changes compared with VTM5.0

Table 1, Dedicated IBC buffer without encoder modifications

| Cfg | Class | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| AI | Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
|  | TMG | 0.00% | 0.00% | 0.00% | 100% | 103% |
| RA | Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
|  | TMG | 0.00% | 0.00% | 0.00% | 100% | 103% |
| LDB | Class F | 0.00% | 0.00% | 0.00% | 100% | 103% |
|  | TMG | 0.00% | 0.00% | 0.00% | 100% | 103% |

## Supplementary results to demonstrate all encoder conformance checks can safely be removed

Supplementary tests are provided to demonstrate that no bitstream conformance checks are necessary in the encoder anymore. Please note that the encoder search space is extended in the supplementary tests which is actually not very desirable in terms of coding gain/encoding time point of view. The results are provided to only to show that encoder side bitstream conformance tests can now be safely removed.

- In table 2, the encoder side block vector validation checks for IBC merge mode are removed (all IBC merge candidates are searched by encoder).

- In table 3, the coder side block vector validation check for IBC chroma blocks (dual tree case) are removed. In other words a chroma block can use IBC mode even if collocated luma block is not IBC.

In both supplementary tests, same decoder (as in subsection 4.1) is used.

Table 2, proposed decoder with an encoder removed BV validation checks for IBC merge

| Cfg | Class | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| AI | Class F | -0.06% | 0.09% | 0.04% | 105% | 101% |
|  | TMG | -0.17% | -0.13% | -0.14% | 102% | 102% |
| RA | Class F | -0.06% | -0.02% | 0.01% | 110% | 103% |
|  | TMG | -0.01% | -0.08% | -0.11% | 106% | 103% |
| LDB | Class F | -0.11% | -0.27% | -0.31% | 117% | 106% |
|  | TMG | -0.15% | -0.21% | -0.15% | 111% | 105% |

\

Table 3, proposed decoder with an encoder removed BV validation checks for IBC chroma

| Cfg | Class | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| AI | Class F | 0.00% | -0.04% | 0.02% | 101% | 103% |
|  | TMG | -0.02% | -0.03% | -0.05% | 101% | 102% |
| RA | Class F | -0.01% | 0.03% | 0.01% | 102% | 103% |
|  | TMG | -0.05% | -0.07% | -0.13% | 102% | 104% |
| LDB | Class F | 0.09% | -0.06% | 0.31% | 103% | 106% |
|  | TMG | -0.03% | -0.15% | -0.07% | 103% | 105% |

# Conclusion

The proposed dedicated IBC reference buffer design removes all the IBC related bitsteam restrictions for luma and chroma. In the meantime, no additional memory is required for IBC compared with VTM5.0. The proposed method has no impact of the coding performance as shown in the simulation results. The proposed method removes almost 2 pages of draft specification and provides an intuitive design. It is proposed to be adopted into the next draft of VVC.

# References

- “Versatile Video Coding (Draft 4)”, JVET-M1001, B. Bross, J. Chen, S. Liu

[2] “Non-CE8: On IBC reference buffer design”, JVET-N0472, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang

[3] “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).