| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0249 |
| --- | --- |

| Title: | Non-CE4: Merge Modes Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Vadim Seregin, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, vseregin, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In the current VTM, there are five merge modes including: the regular merge mode, the merge mode with motion vector difference, the subblock-based merge mode, combined inter and intra prediction mode, and triangle partition mode.  The merge mode signaling follows the order described. In this contribution, two changes are proposed. First, it’s proposed to disable triangle merge prediction mode for a CU that has width or height larger than 64. Second, it’s proposed to group the merge modes in to 3 categories: 1) subblock-based merge mode, 2) regular merge mode and merge mode with motion vector difference, 3) combined inter and intra prediction mode and triangle merge prediction mode. Then a first bin is used to indicate whether the first group is used, and if the first bin is 0, a second bin is used to decide between second group and third group. For the second and third groups, one bin is used to specify the selected mode. The Y, U, V BD-rate performance for RA and LDB under common test conditions respectively are {-0.08%, -0.12%, -0.19%} and {-0.06%, 0.09%, 0.02%}. In the high QP test, the BD-rate performance for RA and LDB are {-0.24%, -0.35%, -0.39%} and {-0.38%, -0.55%, -0.65}, respectively. For the first change only, the BD-rate performance for RA and LDB are {-0.02%, 0.00%, 0.00%} and {0.03%, 0.07%, -0.10%}, respectively.

# Introduction

In the current VTM, besides the regular merge mode as in the HEVC, several new merge modes are adopted. The new merge modes include: Merge Mode with Motion Vector Difference (MMVD), Subblock-based Merge Mode (including Subblock-based Temporal Motion Vector Prediction (SbTMVP) and Affine Merge Prediction (AMP) candidates), Triangle Partition Mode (TPM), and Combined Inter and Intra Prediction (CIIP).

The regular merge mode and MMVD can be applied to all inter-coded CU. For the subblock-based merge mode, the CU size should have both width and height larger or equal to 8. For the TPM and CIIP, the CU area should be larger than 64. And for CIIP, the CU size should have both width and height less than or equal to 64, and it shouldn’t be skip mode. Signaling of different merge modes is illustrated in Fig. 1. If it’s merge mode, a regular_merge_flag is first signaled to indicate whether it’s regular merge mode or other merge modes. Then if it’s not regular merge mode, a mmvd_merge_flag is signaled to indicate whether it’s MMVD mode or not.  If not, a subblock_merge_flag is signaled to indicate whether it’s subblock-based merge mode. If not, a ciip_merge_flag is further signaled to indicate whether it’s CIIP mode or TPM. The present conditions for those flags are shown in Table 1-1. As can be observed from the table, there’re a considerable amount of checks for the signaling of each flag, which results in unnecessary complexity in the parsing process.

Regular merge

MMVD

Subblock merge

CIIP

TPM

regular_merge_flag

mmvd_merge_flag

subblock_merge_flag

ciip_merge_flag

Fig. 1 Signaling of merge modes in VTM-5.-0

Table 1-1 Present conditions for merge flags in current VTM

| syntax | present conditions |
| --- | --- |
| ciip_merge_flag | (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128 && ( cbWidth * cbHeight ) >= 64) ¦¦ (sps_triangle_enabled_flag && slice_type = = B) |
| subblock_merge_flag | MaxNumSubblockMergeCand > 0 && cbWidth > 8 && cbHeight > 8 && ((sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128) ¦¦ (sps_triangle_enabled_flag && slice_type = = B)) |
| mmvd_merge_flag | sps_mmvd_enabled_flag && ( cbWidth * cbHeight ) >= 64 && ((MaxNumSubblockMergeCand > 0 && cbWidth > 8 && cbHeight > 8) ¦¦ ((sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128) ¦¦ (sps_triangle_enabled_flag && slice_type = = B))) |
| regular_merge_flag | sps_mmvd_enabled_flag ¦¦ (( cbWidth * cbHeight ) >= 64 && ((MaxNumSubblockMergeCand > 0 && cbWidth > 8 && cbHeight > 8) ¦¦ ((sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128) ¦¦ (sps_triangle_enabled_flag && slice_type = = B)))) |

# Proposal

In the JEVT-N0483 that was adopted in Geneva meeting, it’s proposed to disable subblock partition transform for TPM due to two artificially intersecting boundaries within the CU. The intersecting boundaries also exist if a TPM  mode CU has width or height larger than the maximum TU size, which is 64. Therefore, it’s proposed to disable TPM for a CU that have width or height larger than 64.

It’s also proposed to group the merge modes into 3 categories for signaling:

- The regular merge mode group, including the regular merge mode and MMVD mode.

- The subblock based merge mode group, including SbTMVP and affine merge candidates.

- The blending merge mode group, including the TPM mode and CIIP mode.

The proposed signaling of different merge modes is illustrated in Fig. 2. A subblock_merge_flag is first signaled to indicate whether it’s the subblock group. If not, regular_merge_flag is signaled to indicate whether it’s regular merge mode group. If regular_merge_flag is true, a mmvd_merge_flag is signaled to indicate whether it’s MMVD mode or regular merge mode. If regular_merge_flag is false, a ciip_merge_flag is signaled to indicate whether it’s CIIP mode or TPM mode.

The present conditions for those merge flags in the proposed method are illustrated in Table 2-1, which is significantly simplified compares to Table 1-1.

MMVD

Subblock merge

TPM

CIIP

subblock_merge_flag

regular_merge_flag

Regular merge

ciip_merge_flag

mmvd_merge_flag

Fig. 2 Proposed signaling method for merge modes

Table 2-1 Present conditions of merge flags of proposed method

| syntax | present conditions |
| --- | --- |
| ciip_merge_flag | (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0) && (sps_triangle_enabled_flag && slice_type = = B) |
| subblock_merge_flag | MaxNumSubblockMergeCand > 0 && cbWidth > 8 && cbHeight > 8 |
| mmvd_merge_flag | sps_mmvd_enabled_flag |
| regular_merge_flag | ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 && ((sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0) ¦¦ (sps_triangle_enabled_flag && slice_type = = B)) |

# Test results

The proposed method was implemented on top of VTM-5.0. Three tests were performed under random access and low delay B test conditions. In Test 1, the common test condition is used, the Y, U, V BD-rate performances in RA and LDB are {-0.08%, -0.12%, -0.19%} and {-0.06%, 0.09%, 0.02%} respectively. In Test 2, a high QP set {32, 37, 42, 47} was used, the BD-rate performances in RA and LDB are {-0.24%, -0.35%, -0.39%} and {-0.38%, -0.55%, -0.65}, respectively. In Test 3, TPM is disabled for a CU that have width or height larger than 64, but the proposed signaling method is not applied. The BD-rate performance in RA and LDB are {-0.02%, 0.00%, 0.00%} and {0.03%, 0.07%, -0.10%}, respectively. Details of the BD-rate performance for the 3 tests are shown in Table 3-1, Table 3-2 and Table 3-3. Runtime may not be accurate.

Table 3-1 Test 1 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.31% | -0.14% | 105% | 104% |
| Class A2 | -0.09% | 0.00% | -0.16% | 98% | 100% |
| Class B | -0.05% | -0.12% | -0.29% | 101% | 103% |
| Class C | -0.07% | -0.07% | -0.10% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.12% | -0.19% | 100% | 101% |
| Class D | -0.01% | -0.08% | 0.17% | 102% | 102% |
| Class F | -0.02% | 0.06% | 0.08% | 101% | 105% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.10% | 0.26% | 98% | 105% |
| Class C | -0.02% | 0.09% | 0.14% | 99% | 100% |
| Class E | -0.15% | 0.07% | -0.54% | 96% | 112% |
| Overall | -0.06% | 0.09% | 0.02% | 98% | 105% |
| Class D | -0.11% | 0.01% | 0.99% | 100% | 103% |
| Class F | 0.03% | 0.31% | -0.71% | 102% | 104% |

Table 3-2 Test 2 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.71% | -0.17% | 102% | 100% |
| Class A2 | -0.29% | -0.25% | -0.43% | 95% | 98% |
| Class B | -0.17% | -0.32% | -0.50% | 99% | 100% |
| Class C | -0.24% | -0.21% | -0.40% | 96% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | -0.35% | -0.39% | 98% | 99% |
| Class D | -0.14% | -0.47% | -0.06% | 101% | 99% |
| Class F | -0.17% | -0.13% | -0.12% | 100% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | -0.46% | -0.09% | 96% | 103% |
| Class C | -0.34% | -0.30% | -0.78% | 97% | 101% |
| Class E | -0.77% | -1.02% | -1.44% | 94% | 113% |
| Overall | -0.38% | -0.55% | -0.65% | 96% | 105% |
| Class D | -0.71% | 0.32% | -0.36% | 100% | 103% |
| Class F | -0.04% | 0.17% | -0.60% | 98% | 103% |

Table 3-3 Test 3 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.15% | 0.06% | 104% | 103% |
| Class A2 | -0.03% | 0.12% | 0.02% | 98% | 100% |
| Class B | 0.00% | 0.03% | -0.03% | 100% | 102% |
| Class C | -0.01% | -0.04% | -0.04% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 106% | 112% |
| Class F | 0.02% | 0.02% | 0.03% | 100% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.19% | -0.03% | 97% | 104% |
| Class C | 0.04% | -0.10% | -0.07% | 99% | 99% |
| Class E | 0.07% | 0.11% | -0.26% | 97% | 107% |
| Overall | 0.03% | 0.07% | -0.10% | 98% | 103% |
| Class D | -0.01% | 0.02% | 0.11% | 106% | 113% |
| Class F | 0.03% | -0.25% | -0.23% | 99% | 104% |

# Conclusion

In this contribution, it’s proposed to disable TPM if block width or height is larger than 64 and group the merge modes into 3 categories for signaling. The number condition checks for syntax signaling is significantly reduced and -0.08% gain is achieved in RA and -0.06% in LDB under CTC. In a high QP (low bitrate) test, the propose method achieves -0.24% gain in RA and -0.38% gain in LDB. It’s recommended to adopt the propose method.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed merge data syntax and semantics

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 && ((sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0) ¦¦ (sps_triangle_enabled_flag && slice_type = = B) ) { |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( regular_merge_flag[ x0 ][ y0 ] ) { |  |
| if( sps_mmvd_enabled_flag ) |  |
| { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } else { |  |
| if(sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( ! ciip_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

regular_merge_flag[ x0 ][ y0 ] equal to 1 specifies that regular merge mode or merge mode with motion vector difference is used to generate the inter prediction parameters of the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. regular_merge_flag[ x0 ][ y0 ] equal to 0 specifies that combined inter-picture merge and intra-picture prediction or triangular shape based motion compensation is used to generate the inter prediction paramters.

When regular_merge_flag[ x0 ][ y0 ] is not present, it is inferred to 1. as follows:

- If all the following conditions are true, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_mmvd_enabled_flag is equal to 0.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- cbWidth*cbHeight is equal to 32.

- Otherwise, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

mmvd_merge_flag[ x0 ][ y0 ] equal to 1 specifies that merge mode with motion vector difference is used to generate the inter prediction parameters of the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. mmvd_merge_flag[ x0 ][ y0 ] equal to 0 specifies that the regular merge mode is used to generate the inter prediction paramters.

When mmvd_merge_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.as follows:

- If all the following conditions are true, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_mmvd_enabled_flag is equal to 1.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- cbWidth*cbHeight is equal to 32.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

ciip_flag[ x0 ][ y0 ] specifies whether the combined inter-picture merge and intra-picture prediction is applied for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. The variable MergeTriangleFlag[ x0 ][ y0 ], which specifies whether triangular shape based motion compensation is used to generate the prediction samples of the current coding unit, when decoding a B slice is set equal to ciip_flag[ x0 ][ y0 ] ? 0 : 1.

When ciip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

When ciip_flag[ x0 ][ y0 ] is equal to 1, the variable IntraPredModeY[ x ][ y ] with x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to INTRA_PLANAR.

The variable MergeTriangleFlag[ x0 ][ y0 ], which specifies whether triangular shape based motion compensation is used to generate the prediction samples of the current coding unit, when decoding a B slice is derived as follows:

- If all the following conditions are true, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 1:

- sps_triangle_enabled_flag is equal to 1.

- slice_type is equal to B.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- MaxNumTriangleMergeCand is greater than or equal to 2.

- cbWidth * cbHeight is greater than or equal to 64.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- mmvd_merge_flag[ x0 ][ y0 ] is equal to 0.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- ciip_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 0.