| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0252 |
| --- | --- |

| Title: | CE4-related: Alignment and simplification of PROF and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Vadim Seregin, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, vseregin, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In CE4-2.1, prediction refinement with optical flow for affine mode is tested. It’s reported the BD-rate performance for RA and LD respectively are {-0.48%, -0.09%, -0.07%} and {-0.32%, -0.04%, -0.37%} in test a. In this contribution, methods for alignment and simplification of PROF and BDOF are proposed. First, the padding process for gradient calculation in BDOF is aligned with that in PROF, which uses the nearest integer position. The range of MV in BDOF is modified for less bit representation. Second, the rounding operation in BDOF and PROF is removed for the refinement offset calculation. The clipping operation in PROF is also removed. Third, the motion offset in PROF is calculated from subblock MVs instead of CPMVs. It’s reported the overall BD-rate performance for RA and LD respectively are {-0.52%, -0.11%, -0.12%} and {-0.30%, -0.16%, -0.24%}.

# Introduction

In CE4-2.1, prediction refinement with optical flow (PROF) for affine mode is tested. The PROF refine the sub-block based affine motion compensated prediction with optical flow. After the sub-block based affine motion compensation is performed, luma prediction sample is refined by adding a difference derived by the optical flow equation [1]. The refinement is performed by adding an offset that is calculated for each pixel using the affine parameters and spatial gradient. In the gradient calculation, the pixels on the extended borders are copied from the nearest integer pixel position in the reference picture. However, in the BDOF, the pixels on the extended borders are copied from the integer position that is calculated by floor rounding.

# Proposal

In this contribution, methods for alignment and simplification of PROF and BDOF are proposed.

## Alignment of BDOF and PROF

It’s proposed to align padding process of gradient calculation in BDOF to that in PROF, which uses the nearest integer position. It’s also proposed to modify the delta MV range in BDOF to [-mvRefineThres, mvRefineThres-1] as in PROF instead of [-mvRefineThres, mvRefineThres] to reduce 1 bit for MV representation.

## Simplification of refinement offset calculation

The prediction refinement in BDOF is described as follows:

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

bdofOffset = Round( ( vx * ( gradientHL1[ x + 1 ][ y + 1 ] − gradientHL0[ x + 1 ][ y + 1 ] ) ) >> 1 )	(8-783)
	+ Round( ( vy * (gradientVL1[ x + 1 ][ y + 1 ] − gradientVL0[ x + 1 ][ y + 1 ] ) ) >> 1 )

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ]  + offset4 +	(8-784)
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )

The prediction refinement process of PROF is described as follows:

Inputs to this process are:

- two variables sbWidth and sbHeight specifying the width and the height of the current subblock,

- one (sbWidth + borderExtension) x (sbHeight + borderExtension) luma prediction sample arrays, predSamples

- one (sbWidth x sbHeight) motion vector difference array, diffMv

Output of this process is the (sbWidth)x(sbHeight) array pbSamples of luma prediction sample values.

Variables bitDepth and shift1 are derived as follows:

- The variable bitDepth is set equal to BitDepthY.

- The variable shift1 is set to equal to Max( 6, bitDepth − 6 ).

- The variable dILimit is set equal to 1 << (BitDepthY – 1).

For x =0.. sbWidth - 1, y =0.. sbHeight - 1, the following ordered steps apply:

  - The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] − predSamples[ x ][ y ] ) >> shift1

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] − predSamples[ x ][ y ] ) >> shift1

  - The variable dI is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]

  - Luma prediction sample value at location ( x, y ) in the subblock is derived as follows:

pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + Clip3( -dILimit, dILimit - 1, ( dI + 1 ) >> 1 ) )

It’s proposed to modify the prediction refinement in BDOF as follows:

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

bdofOffset = Round( ( vx * ( gradientHL1[ x + 1 ][ y + 1 ] − gradientHL0[ x + 1 ][ y + 1 ] ) ) >> 1 )	
	+ Round( ( vy * (gradientVL1[ x + 1 ][ y + 1 ] − gradientVL0[ x + 1 ][ y + 1 ] ) ) >> 1 )

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ]  + offset4 +	
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )

and modifiy the prediction refinement process of PROF as follows:

For x =0.. sbWidth - 1, y =0.. sbHeight - 1, the following ordered steps apply:

  - The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] − predSamples[ x ][ y ] ) >> shift1

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] − predSamples[ x ][ y ] ) >> shift1

  - The variable dI is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]

  - Luma prediction sample value at location ( x, y ) in the subblock is derived as follows:

pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + dIClip3( -dILimit, dILimit - 1, ( dI + 1 ) >> 1 ) )

Correpondingly, the horizontal and vertical motion offset in BDOF and PROF is divied by 2 before feed into the refinemenet process.

## Motion offset calculation

In PROF, the motion offset calculation is based on the CPMVs of the affine CU, which requires the storage of CPMVs for each 8x8 unit in the motion compensation stage. However, in the current VTM, the CPMVs can be stored in left/above/above left contexts for affine motion inheritance in the memory-optimized implementation, which requires much less buffer size [2]. Therefore, it’s proposed to use the subblock MVs, which is already stored for motion compensation, instead of CPMVs for the motion offset calculation.

# Test results

The tests are performed on top of CE4-2.1 under random access and low delay B common test conditions. In Test 1, the alignment described in section 2.1 is applied, the BD-rate performance is {-0.54%, -0.08%, -0.07%} in RA and {-0.32%, -0.04%, -0.37%} in LDB. In Test 2, the simplification described in section 2.2 is applied on top of Test 2, the BD-rate performance is {-0.53%, -0.10%, -0.09%} in RA and {-0.27%, 0.07%, -0.07%} in LDB. In Test 3, the subblock based motion offset calculation is applied on top of Test 2, the BD-rate performance is {-0.52%, -0.11%, -0.12%} in RA and {-0.30%, -0.16%, -0.24%} in LDB. In Test 4, the subblock based motion offset calculation is applied on top of CE4-2.1, the BD-rate performance is {-0.48%, -0.10%, -0.12%} in RA and {-0.31%, 0.03%, -0.24%} in LDB.  The detail results of the 4 tests are shown in the following tables. Runtime may not be accurate due to use of heterogeneous cluster. When compares to the results in CE4-2.1.a (results obtained from CE test), the BD-rate change of Test 1 is {-0.06%, 0.01%, 0.00%} in RA, and no change in LDB; the BD-rate change of Test 2 is {-0.05%, -0.01, -0.03%} in RA, and {0.05%, 0.11, 0.30%} in LDB; the BD-rate change of Test 3 is {-0.04%, -0.02%, -0.05%} in RA, and {0.02%, -0.12%, 0.12%} in LDB; the BD-rate change of Test 4 is {0.00%, -0.01%, -0.05%} in RA, and {0.01%, 0.07%, 0.13%} in LDB.

Table 3-1- Test 1 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.13% | 0.15% | 106% | 105% |
| Class A2 | -0.88% | 0.29% | 0.19% | 103% | 99% |
| Class B | -0.66% | -0.26% | -0.19% | 104% | 105% |
| Class C | -0.43% | -0.12% | -0.26% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.54% | -0.08% | -0.07% | 103% | 102% |
| Class D | -0.82% | -0.25% | -0.10% | 106% | 108% |
| Class F | -0.86% | -0.44% | -0.48% | 104% | 105% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -0.05% | -0.28% | 102% | 102% |
| Class C | -0.23% | 0.05% | -0.15% | 103% | 105% |
| Class E | -0.30% | -0.14% | -0.80% | 105% | 110% |
| Overall | -0.32% | -0.04% | -0.37% | 103% | 105% |
| Class D | -0.38% | -0.35% | 1.14% | 107% | 115% |
| Class F | -1.33% | -1.34% | -0.97% | 105% | 103% |

Table 3-2- Test 2 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.15% | 0.09% | 108% | 106% |
| Class A2 | -0.88% | 0.32% | 0.15% | 102% | 101% |
| Class B | -0.65% | -0.32% | -0.23% | 104% | 104% |
| Class C | -0.40% | -0.12% | -0.25% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.53% | -0.10% | -0.09% | 103% | 102% |
| Class D | -0.83% | -0.41% | -0.20% | 101% | 100% |
| Class F | -0.86% | -0.43% | -0.39% | 103% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.40% | 0.15% | -0.05% | 102% | 106% |
| Class C | -0.20% | -0.03% | -0.10% | 102% | 104% |
| Class E | -0.16% | 0.09% | -0.07% | 103% | 107% |
| Overall | -0.27% | 0.07% | -0.07% | 102% | 106% |
| Class D | -0.42% | -0.11% | -0.63% | 102% | 100% |
| Class F | -1.29% | -0.69% | -1.37% | 103% | 104% |

Table 3-3- Test 3 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.16% | 0.00% | 106% | 105% |
| Class A2 | -0.86% | 0.24% | 0.09% | 102% | 102% |
| Class B | -0.61% | -0.25% | -0.17% | 103% | 104% |
| Class C | -0.42% | -0.17% | -0.30% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -0.11% | -0.12% | 103% | 102% |
| Class D | -0.83% | -0.41% | -0.13% | 106% | 107% |
| Class F | -0.86% | -0.45% | -0.47% | 104% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.40% | -0.26% | -0.23% | 102% | 103% |
| Class C | -0.23% | 0.04% | -0.06% | 102% | 105% |
| Class E | -0.22% | -0.25% | -0.51% | 103% | 106% |
| Overall | -0.30% | -0.16% | -0.24% | 102% | 104% |
| Class D | -0.43% | -0.58% | 0.69% | 106% | 105% |
| Class F | -1.29% | -1.49% | -1.55% | 105% | 108% |

Table 3-4- Test 4 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.19% | 0.09% | 107% | 105% |
| Class A2 | -0.84% | 0.31% | 0.07% | 102% | 101% |
| Class B | -0.59% | -0.31% | -0.27% | 104% | 105% |
| Class C | -0.33% | -0.07% | -0.23% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.48% | -0.10% | -0.12% | 103% | 103% |
| Class D | -0.56% | 0.02% | 0.08% | 101% | 100% |
| Class F | -0.87% | -0.47% | -0.50% | 104% | 106% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | -0.08% | -0.17% | 102% | 102% |
| Class C | -0.22% | -0.05% | -0.15% | 102% | 105% |
| Class E | -0.30% | 0.32% | -0.48% | 103% | 109% |
| Overall | -0.31% | 0.03% | -0.24% | 102% | 105% |
| Class D | -0.38% | -0.25% | 0.57% | 103% | 102% |
| Class F | -1.26% | -1.13% | -1.13% | 105% | 106% |

Table 3-5 Test 1 compares to CE4-2.1.a

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-4-2.1.a |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | 0.04% | 85% | 80% |
| Class A2 | -0.03% | 0.01% | 0.03% | 88% | 88% |
| Class B | -0.05% | 0.03% | 0.00% | 86% | 80% |
| Class C | -0.12% | -0.04% | -0.05% | 88% | 88% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.01% | 0.00% | 87% | 84% |
| Class D | -0.28% | -0.12% | -0.14% | 92% | 81% |
| Class F | 0.00% | 0.00% | -0.01% | 85% | 77% |

Table 3-6 Test 2 compares to CE4-2.1.a

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-4-2.1.a |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.02% | -0.02% | 87% | 81% |
| Class A2 | -0.02% | 0.05% | -0.01% | 87% | 90% |
| Class B | -0.04% | -0.04% | -0.04% | 86% | 79% |
| Class C | -0.08% | -0.05% | -0.04% | 86% | 86% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.01% | -0.03% | 86% | 83% |
| Class D | -0.29% | -0.28% | -0.24% | 87% | 76% |
| Class F | 0.00% | 0.02% | 0.09% | 84% | 76% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over CE-4-2.1.a |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.20% | 0.23% | 71% | 78% |
| Class C | 0.03% | -0.08% | 0.06% | 87% | 92% |
| Class E | 0.14% | 0.23% | 0.76% | 82% | 90% |
| Overall | 0.05% | 0.11% | 0.30% | 79% | 85% |
| Class D | -0.04% | 0.24% | -1.72% | 87% | 76% |
| Class F | 0.04% | 0.68% | -0.41% | 81% | 82% |

Table 3-7 Test 3 compares to CE4-2.1.a

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-4-2.1.a |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.00% | -0.11% | 85% | 80% |
| Class A2 | -0.01% | -0.03% | -0.06% | 87% | 91% |
| Class B | 0.00% | 0.03% | 0.02% | 85% | 79% |
| Class C | -0.11% | -0.10% | -0.09% | 88% | 87% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.02% | -0.05% | 86% | 84% |
| Class D | -0.30% | -0.28% | -0.17% | 91% | 81% |
| Class F | -0.01% | 0.00% | 0.01% | 84% | 76% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over CE-4-2.1.a |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.21% | 0.05% | 71% | 76% |
| Class C | 0.00% | -0.01% | 0.09% | 87% | 93% |
| Class E | 0.08% | -0.11% | 0.29% | 82% | 89% |
| Overall | 0.02% | -0.12% | 0.12% | 79% | 84% |
| Class D | -0.06% | -0.23% | -0.43% | 90% | 80% |
| Class F | 0.04% | -0.16% | -0.60% | 82% | 84% |

Table 3-8 Test 4 compares to CE4-2.1.a

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE-4-2.1.a |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.02% | 86% | 80% |
| Class A2 | 0.01% | 0.03% | -0.08% | 87% | 90% |
| Class B | 0.03% | -0.02% | -0.08% | 86% | 80% |
| Class C | -0.02% | 0.00% | -0.02% | 87% | 87% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.05% | 87% | 84% |
| Class D | -0.02% | 0.15% | 0.04% | 87% | 75% |
| Class F | -0.01% | -0.02% | -0.02% | 84% | 77% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over CE-4-2.1.a |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.03% | 0.11% | 70% | 75% |
| Class C | 0.01% | -0.10% | 0.00% | 87% | 93% |
| Class E | 0.00% | 0.46% | 0.33% | 82% | 92% |
| Overall | 0.01% | 0.07% | 0.13% | 78% | 85% |
| Class D | -0.01% | 0.10% | -0.56% | 88% | 78% |
| Class F | 0.07% | 0.22% | -0.16% | 83% | 83% |

# Conclusion

In this contribution, it’s proposed to align the padding process in gradient calculation for BDOF and PROF, simplify the refinement process of BDOF and PROF by removing the rounding operation, and use subblock MVs for refinement offset calculation of PROF to reduce the memory requirement. The test results show some gain on the alignment and no loss for the simplifications.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- J. Luo, Y. He, “CE4-2.1: Prediction refinement with optical flow for affine mode”, JVET-O0070, Gothenburg, SE, July 2019.

- M. Zhou, B. Heng, “CE4-related: A clean up for affine mode”, JVET-L0047, the 12th JVET meeting, Oct. 2018, Macao, CN.