| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0258-v1 |
| --- | --- |

| Title: | Non-CE8: Adaptive single/dual tree with IBC simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Jizheng Xu, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

It was verified that dual tree is efficient in general, especially for chroma components. However, it may not be the case for screen content coding and for 4:4:4 format. Thus, it is proposed to adaptively switch between dual tree and single tree at CTU-level and disable chroma IBC in the dual tree coding structure. Simulation results show that the proposed scheme achieves coding gains over both single tree and dual tree anchors. Furthermore, it removes IBC for chroma CU, which simplify both the implementation and the text. Results (BD-rate for luma, encoding time change and decoding time change) are summarized as follows:

On 4:2:0 sequences (versus VTM-5.0 with dual tree on, i.e. VTM-5.0 anchor),

Class F: AI: -0.01%, 126%, 100%; RA: -0.20%, 110%, 101%; LDB: 0.05%, 104%, 98%;

Class TGM: AI: -1.31%, 143%, 98%; RA: -0.10%, 108%, 100%; LDB: -0.02%, 103%, 99%.

On 4:2:0 sequences (versus VTM-5.0 with dual tree off),

Class F: AI: -0.80%, 89%, 101%; RA: -0.59%, 102%, 107%; LDB: -0.08%, 102%, 104%;

Class TGM: AI: -0.54%, 105%, 99%; RA: -0.24%, 104%, 104%; LDB: -0.03%, 103%, 103%.

On 4:4:4 sequences (versus VTM5 with dual tree on),

Class TGM 1080p: AI: -6.35%, 150%, 88%; RA: -2.06%, 108%, 106%; LDB: -0.32%,1 05%, 101%;

Class TGM 720p: AI: -3.33%, 161%, 99%; RA: -2.34%, 114%, 111%; LDB: -0.38%, 106%, 111%;

Animation: AI: 0.17%, 102%, 102%; RA: 0.08%, 103%, 107%; LDB: 0.03, 103%, 102%;

Mixed content: AI: -1.75%, 160%, 95%; -1.24%, 112%, 104%; -0.16%, 106%, 101%;

Camera-captured content: AI: 0.02, 103%, 103%; -0.02%, 103%, 104%; 0.01%, 104%, 103%.

On 4:4:4 sequences (versus VTM5 with dual tree off)

Class TGM 1080p: AI: -0.17%,101%, 102%; RA: 0.00%, 104%, 106%; LDB: 0.00%, 105%, 103%;

Class TGM 720p: AI: -0.45%,105%,107%; RA: -0.52%,103%, 112%; LDB: -0.04%,104%,112%.

Animation: AI: 0.21%, 62%, 107%; RA: -0.17%, 97%, 108%; 0.02%, 102%, 103%;

Mixed content: AI: -0.05%, 109%, 103%; RA: -0.12%, 106%, 104%; -0.10%, 107%, 101%;

Camera-captured content: AI: -1.58%, 57%, 106%; -1.57%, 98%, 105%; -0.15%, 105%, 105%.

# Introduction

The dual tree coding structure is employed on intra slices in the design and enabled in the CTC. However, sometimes dual tree may not be efficient compared with single tree coding structure, especially for 4:4:4 sequences. In addition, 2x2 block level block vector checking and sample copying in the current chroma IBC design of intra slices introduce too much complexity for both hardware and software implementation.

# Proposed method

In this document, we propose to adaptively switch between dual tree and single tree at CTU-level for I-slice and disable chroma IBC when dual tree coding structure is used for a CTU. A flag is signalled at CTU level to indicate whether the single tree or dual tree is applied. The proposed scheme can preserve the benefits of both structures and perform better than either one. Details of the proposed scheme are described as follows.

## Decision of usage of CTU-level adaptation for an intra slice

There is a SPS flag introduced to control whether to enable the proposed method for I-slice. If the SPS flag is enabled, we use hash values to determine whether the proposed scheme is enabled on each I-slice. In intra block copy mode, hash value is generated for each 4x4 block at encoder, then we check whether there is another 4x4 block has the same hash value with it. If yes, such block is considered as a matching block. If the percentage of matching blocks is larger than a threshold (20%) for an I-slice, the proposed method is enabled, Otherwise, the proposed method is disabled, and current slice is coded with dual tree structure only. In addition, the proposed method is only applied on I slices, so the slice level flag is not signalled and inferred to 0 for P and B slices.

## Fast algorithms for CTU-level selection of coding structure

For each 4x4 block in a CTU, we check whether there is another 4x4 block has the same hash value with it. If yes, such block is considered as a matching block. If the percentage of matching blocks is larger than a threshold (75%) and the proposed method is enabled for a slice, single tree coding structure is used to code the CTU. Otherwise, both dual and single tree could be tested for a CTU. To further reduce the encoding complexity, when both dual tree and single tree are enabled, the MIP flag, MTS flag, RST index of dual tree are reused for the single tree coding.

# Simulation results

The proposed method is evaluated with VTM5. The simulation follows the CE 8 test condition [1]. Four sets of simulations have been launched, as follows:

1) Test #1: Test conditions of 4:2:0 sequences in CE8.

Anchor: VTM5 with dual tree enabled

Tested: The proposed method

2) Test #2: Test conditions of 4:2:0 sequences in CE8.

Anchor: VTM5 with dual tree disabled.

Tested: The proposed method

3) Test #3: Test conditions of 4:4:4 sequences in CE8.

Anchor: VTM5 with dual tree enabled

Tested: The proposed method

4) Test #4: Test conditions of 4:4:4 sequences in CE8.

Anchor: VTM5 with dual tree disabled

Tested: The proposed method

Summary of coding performance are given as follows. More detailed results can be found in the attached spreadsheets.

Table 1. 4:2:0 results compared with VTM-5.0 dual tree on, i.e. Test #1.

| AI | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | 0.27% | -0.60% | 127% | 95% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 99% |
| Class B | 0.00% | 0.01% | 0.03% | 103% | 96% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 93% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Overall | 0.00% | 0.05% | -0.09% | 106% | 95% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 93% |
| Class F | -0.01% | 0.90% | 1.44% | 126% | 100% |
| TGM | -1.31% | 0.42% | -0.79% | 143% | 98% |

| RA | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.02% | 0.07% | -0.22% | 106% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 104% | 97% |
| Class B | 0.00% | -0.04% | 0.01% | 103% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.04% | 103% | 96% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class F | -0.20% | 0.30% | 0.66% | 110% | 101% |
| TGM | -0.10% | 0.29% | -0.19% | 108% | 100% |

| LDB | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | -0.06% | 101% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class E | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Overall | 0.00% | 0.00% | -0.02% | 102% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 105% |
| Class F | 0.05% | -0.17% | -0.75% | 104% | 98% |
| TGM | -0.02% | 0.33% | -0.19% | 103% | 99% |

Table 2. 4:2:0 results compared with VTM-5.0 dual tree off, i.e. Test #2.

| AI | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.53% | -11.30% | -5.22% | 74% | 103% |
| Class A2 | -0.10% | -12.17% | -7.61% | 62% | 105% |
| Class B | -0.42% | -11.91% | -17.22% | 65% | 102% |
| Class C | -0.75% | -8.03% | -9.41% | 64% | 103% |
| Class E | -0.13% | -10.83% | -13.94% | 61% | 99% |
| Overall | -0.41% | -10.81% | -11.34% | 65% | 102% |
| Class D | -0.43% | -8.12% | -7.68% | 66% | 100% |
| Class F | -0.80% | -4.80% | -6.22% | 89% | 101% |
| TGM | -0.54% | -1.63% | -1.83% | 105% | 99% |

| RA | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.06% | -2.69% | -1.30% | 103% | 102% |
| Class A2 | -0.07% | -6.53% | -3.04% | 102% | 102% |
| Class B | -0.15% | -8.07% | -12.19% | 99% | 102% |
| Class C | -0.25% | -2.94% | -3.47% | 98% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -5.32% | -5.86% | 100% | 102% |
| Class D | -0.19% | -4.80% | -5.50% | 98% | 106% |
| Class F | -0.59% | -2.14% | -3.23% | 102% | 107% |
| TGM | -0.24% | -0.75% | -0.86% | 104% | 104% |

| LDB | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -2.00% | -3.20% | 101% | 101% |
| Class C | -0.03% | -0.67% | -0.58% | 101% | 106% |
| Class E | -0.22% | -5.07% | -7.17% | 103% | 106% |
| Overall | -0.08% | -2.32% | -3.32% | 101% | 104% |
| Class D | -0.02% | -1.35% | -2.85% | 101% | 104% |
| Class F | -0.08% | -0.44% | -1.03% | 102% | 104% |
| TGM | -0.03% | -0.21% | -0.31% | 103% | 103% |

Table 3. 4:4:4 results compared with VTM-5.0 dual tree on, i.e. Test #3.

| AI | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -6.35% | -7.66% | -7.87% | 150% | 88% |
| TGM 720p | -3.33% | -3.67% | -3.20% | 161% | 99% |
| Animation | 0.17% | 0.20% | 0.21% | 102% | 102% |
| Mixed content | -1.75% | -1.72% | -1.66% | 160% | 95% |
| Camera-Captured content | 0.02% | -0.02% | -0.05% | 103% | 103% |
| Overall | -2.91% | -3.34% | -3.26% | 140% | 96% |

| RA | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -2.06% | -2.85% | -2.90% | 108% | 106% |
| TGM 720p | -2.34% | -2.52% | -2.36% | 114% | 111% |
| Animation | 0.08% | 0.15% | 0.13% | 103% | 107% |
| Mixed content | -1.24% | -1.63% | -1.33% | 112% | 104% |
| Camera-Captured content | -0.02% | -0.05% | 0.01% | 103% | 104% |
| Overall | -1.41% | -1.75% | -1.65% | 109% | 107% |

| LDB | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.32% | -0.60% | -0.64% | 105% | 101% |
| TGM 720p | -0.38% | -0.47% | -0.64% | 106% | 111% |
| Animation | 0.03% | 0.08% | -0.03% | 103% | 102% |
| Mixed content | -0.16% | 0.16% | 0.04% | 106% | 101% |
| Camera-Captured content | 0.01% | 0.08% | -0.17% | 104% | 103% |
| Overall | -0.21% | -0.23% | -0.36% | 105% | 104% |

Table 4. 4:4:4 results compared with VTM-5.0 dual tree off, i.e. Test #4.

| AI | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.17% | -0.02% | 0.01% | 101% | 102% |
| TGM 720p | -0.45% | -0.24% | -0.30% | 105% | 107% |
| Animation | 0.21% | 1.76% | 2.57% | 62% | 107% |
| Mixed content | -0.05% | -0.18% | 0.12% | 109% | 103% |
| Camera-Captured content | -1.58% | -2.43% | -2.64% | 57% | 106% |
| Overall | -0.36% | -0.19% | -0.06% | 90% | 105% |

| RA | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.00% | -0.03% | -0.06% | 104% | 106% |
| TGM 720p | -0.52% | 0.49% | 0.49% | 103% | 112% |
| Animation | -0.17% | 1.69% | 2.02% | 97% | 108% |
| Mixed content | -0.12% | -0.07% | 0.24% | 106% | 104% |
| Camera-Captured content | -1.57% | -1.62% | -1.96% | 98% | 105% |
| Overall | -0.39% | 0.12% | 0.17% | 102% | 107% |

| LDB | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.00% | 0.13% | 0.02% | 105% | 103% |
| TGM 720p | -0.04% | 0.28% | 0.22% | 104% | 112% |
| Animation | 0.02% | -0.51% | -0.26% | 102% | 103% |
| Mixed content | -0.10% | 0.40% | 0.28% | 107% | 101% |
| Camera-Captured content | -0.15% | -0.16% | -0.41% | 103% | 104% |
| Overall | -0.05% | 0.10% | 0.03% | 105% | 105% |

# Conclusions

This document proposed a scheme of adaptive CTU-level dual tree. Simulation results report significant coding gain especially on 444 sequences. We recommend adopting the proposed scheme to the next version of VVC.

# Text changes

The changes on top of the draft provided by JVET- N1001-v7 [2] are marked in grey. Deleted texts are marked with strikethrough.

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| sps_mmvd_enabled_flag | u(1) |
| … |  |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| sps_asdt_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| … |  |
| } |  |

7.3.5 Slice header syntax

7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if ( slice_type != I ) { |  |
| … |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ . ( weighted_bipred_flag && slice_type = = B ) ) |  |
| … |  |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_asdt_flag ) |  |
| slice_asdt_flag | u(1) |
| slice_qp_delta | se(v) |
| … |  |
| } |  |

sps_asdt_flag equal to 1 specifies that the proposed method can be enabled on the current sequqence. It equals to 0 specifies that the proposed method cannot be enabled on the current sequqence. When sps_asdt_flag is not present, it is inferred to be equal to 0.

slice_asdt_flag equal to 1 specifies that the proposed method can be enabled on the current slice. It equals to 0 specifies that the proposed method cannot be enabled on the current slice. When slice_asdt_flag is not present, it is inferred to be equal to 0.

#### 7.3.7.2 Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| … |  |
| if(slice_asdt_flag) |  |
| ctu_ dual_tree_intra_flag | ae(v) |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag && ctu_ dual_tree_intra_flag ) |  |
| dual_tree_implicit_qt_split ( xCtb, yCtb, CtbSizeY, 0 ) |  |
| else |  |
| coding_tree( xCtb, yCtb, CtbSizeY, CtbSizeY, 1, 0, 0, 0, 0, 0, SINGLE_TREE ) |  |
| } |  |

ctu_ dual_tree_intra_flag equal to 1 specifies that the current CTU employs dual tree coding structure. It equals to 0 specifies that the current CTU employs single tree coding structure. When ctu_ dual_tree_intra_flag is not present, it is inferred to be equal to qtbtt_dual_tree_intra_flag.

#### 7.3.7.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && . !( cbWidth = = 4 && cbHeight = = 4 && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I . && !( cbWidth = = 4 && cbHeight = = 4 ) ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ . ( slice_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ . ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && . sps_ibc_enabled_flag && ( cbWidth != 128 ¦ ¦ cbHeight != 128 ) && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| … |  |
| } |  |

## 8.6 Decoding process for coding units coded in IBC prediction mode

### 8.6.1 General decoding process for coding units coded in IBC prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs.

The decoding process for coding units coded in ibc prediction mode consists of the following ordered steps:

- The motion vector components of the current coding unit are derived as follows:

    - If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the following applies:

- The derivation process for motion vector components as specified in clause 8.6.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vector mvL[ 0 ][ 0 ] as output.

- When treeType is equal to SINGLE_TREE, the derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with luma motion vector mvL[ 0 ][ 0 ] as input, and chroma motion vector mvC[ 0 ][ 0 ] as output.

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are both set equal to 1.

    - Otherwise, if treeType is equal to DUAL_TREE_CHROMA, the following applies:

- The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are derived as follows:

numSbX = ( cbWidth >> 2 )		(8-871)

numSbY = ( cbHeight >> 2 )		(8-872)

- The chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] are derived as follows for xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1:

- The luma motion vector mvL[ xSbIdx ][ ySbIdx ] is derived as follows:

- The location ( xCuY, yCuY ) of the collocated luma coding unit is dervied as follows:

xCuY = xCb + xSbIdx*4		 	(8-873)

yCuY = yCb + ySbIdx*4		 	(8-874)

- If CuPredMode[ xCuY ][ yCuY ] is equal to MODE_INTRA, the following applies.

mvL[ xSbIdx ][ ySbIdx ][ 0 ] = 0	(8-875)

mvL[ xSbIdx ][ ySbIdx ][ 1 ] = 0	(8-876)

predFlagL0[ xSbIdx ][ ySbIdx ] = 0	(8-877)

predFlagL1[ xSbIdx ][ ySbIdx ] = 0	(8-878)

- Otherwise ( CuPredMode[ xCuY ][ yCuY ] is equal to MODE_IBC ), the following applies:

mvL[ xSbIdx ][ ySbIdx ][ 0 ]=MvL0[ xCuY ][ yCuY ][ 0 ] 	(8-879)

mvL[ xSbIdx ][ ySbIdx ][ 1 ]=MvL0[ xCuY ][ yCuY ][ 1 ] 	(8-880)

predFlagL0[ xSbIdx ][ ySbIdx ] = 1	(8-881)

predFlagL1[ xSbIdx ][ ySbIdx ] = 0	(8-882)

- The derivation process for chroma motion vectors in clause 8.6.2.5 is invoked with mvL[ xSbIdx ][ ySbIdx ] as inputs, and mvC[ xSbIdx ][ ySbIdx ] as output.

- It is a requirement of bitstream conformance that the chroma motion vector mvC[ xSbIdx ][ ySbIdx ] shall obey the following constraints:

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ), yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) ) as inputs, the output shall be equal to TRUE.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + cbWidth / SubWidthC − 1, yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + cbHeight / SubHeightC − 1 ) as inputs, the output shall be equal to TRUE.

- One or both of the following conditions shall be true:

- ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + xSbIdx * 2 + 2 is less than or equal to 0.

- ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + ySbIdx * 2 + 2 is less than or equal to 0.

- The prediction samples of the current coding unit are derived as follows:

- If  treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the prediction samples of the current coding unit are derived as follows:

      - The decoding process for ibc blocks as specified in clause 8.6.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the luma motion vectors mvL[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the variable cIdx set equal to 0 as inputs, and the ibc prediction samples (predSamples) that are an (cbWidth)x(cbHeight) array predSamplesL of prediction luma samples as outputs.

- Otherwise if treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the prediction samples of the current coding unit are derived as follows:

      - The decoding process ibc blocks as specified in clause 8.6.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1 and the variable cIdx set equal to 1 as inputs, and the ibc prediction samples (predSamples) that are an (cbWidth / 2)x(cbHeight / 2) array predSamplesCb of prediction chroma samples for the chroma components Cb as outputs.

      - The decoding process for ibc blocks as specified in clause 8.6.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1 and the variable cIdx set equal to 2 as inputs, and the ibc prediction samples (predSamples) that are an (cbWidth / 2)x(cbHeight / 2) array predSamplesCr of prediction chroma samples for the chroma components Cr as outputs.

- The variables NumSbX[ xCb ][ yCb ] and NumSbY[ xCb ][ yCb ] are set equal to numSbX and numSbY, respectively.

- The residual samples of the current coding unit are derived as follows:

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the width nTbW set equal to the luma coding block width cbWidth, the height nTbH set equal to the luma coding block height cbHeight and the variable cIdxset equal to 0 as inputs, and the array resSamplesL as output.

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdxset equal to 1 as inputs, and the array resSamplesCb as output.

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdxset equal to 2 as inputs, and the array resSamplesCr as output.

- The reconstructed samples of the current coding unit are derived as follows:

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the picture reconstruction process for a colour component as specified in clause 8.7.5 is invoked with the block location ( xB, yB ) set equal to ( xCb, yCb ), the block width bWidth set equal to cbWidth, the block height bHeight set equal to cbHeight, the variable cIdx set equal to 0, the (cbWidth)x(cbHeight) array predSamples set equal to predSamplesL and the (cbWidth)x(cbHeight) array resSamples set equal to resSamplesL as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA,  the picture reconstruction process for a colour component as specified in clause 8.7.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx set equal to 1, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCb and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCb as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the picture reconstruction process for a colour component as specified in clause 8.7.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx set equal to 2, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCr and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCr as inputs, and the output is a modified reconstructed picture before in-loop filtering.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Mar. 2019

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-N1028, Mar. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).