| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0260 |
| --- | --- |

| Title: | Non-CE3: Rounding offset removal for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Hongbin Liu, Li Zhang, Kai Zhang,Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposed to unify the DCT-IF in VVC. In the VVC draft 5, DCT-IF is used in both intra-prediction interpolation and inter-prediction chroma motion compensation. It is asserted that the current DCT-IF operations in these two places are not aligned. In this proposal, it is proposed to unify the two DCT-IF processes by removing the rounding offset in the intra-prediction.

It is reported that the performance impact is negligible. Under the CTC, the overall impact for {Y, Cb, Cr} is {0.01%, -0.06%, -0.02%} for AI and {0.00%, -0.11%, -0.05%} for RA with similar encoding and decoding time.

# DCT-IF filter in VVC draft 5

In VVC working draft 5 [1], the same DCT-IF filter is used for both the intra-prediction interpolation and the inter-prediction chroma motion compensation. However, it is applied with a rounding offset in the former but without a rounding offset in the latter. This causes a misalignment on DCT-IF.

It is also asserted that in HEVC spec, DCT-IF is applied without a rounding offset.

# Proposed solution

In this proposal, it is proposed to remove the rounding offset in intra-prediction interpolation. Therefore, DCT-IF could also be aligned in VVC.

# Proposed spec changes

The proposed changes on top of VVC draft 5 [1] are described as below. Newly added parts are highlighted in yellow, and the deleted parts from [1] are highlighted in green strikethrough.

##### 8.4.5.2.12 Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If predModeIntra is greater than or equal to 34, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

…

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 + refIdx ) * intraPredAngle )  >>  5 + refIdx	 (8-126)

iFact = ( ( y + 1 + refIdx ) * intraPredAngle ) & 31	(8-127)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(8-128)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1Y( ( ( i=03fT[ i ]*ref[ x+iIdx+i ] ) + 32 )  >>  6 )	(8-129)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

…

- Otherwise (predModeIntra is less than 34), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

…

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 + refIdx ) * intraPredAngle )  >>  5	(8-139)

iFact = ( ( x + 1 + refIdx ) * intraPredAngle ) & 31	(8-140)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(8-141)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1Y( ( ( i=03fT[ i ]*ref[ x+iIdx+i ] ) + 32 )  >>  6 )	(8-142)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

…

# Simulation results

The proposed method was implemented based on the VTM-5.0 reference software [2]. Simulation results based on the common test conditions as specified in [3] are provided in Table 1.

Table 1: Simulation results of the proposed method

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.05% | -0.03% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | -0.06% | 99% | 100% |
| Class B | 0.00% | -0.06% | -0.04% | 100% | 100% |
| Class C | 0.02% | -0.06% | 0.06% | 101% | 96% |
| Class E | 0.03% | -0.12% | -0.02% | 100% | 97% |
| Overall | 0.01% | -0.06% | -0.02% | 100% | 99% |
| Class D | 0.01% | 0.09% | -0.15% | 100% | 97% |
| Class F | 0.01% | 0.05% | -0.02% | 99% | 101% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.21% | 0.13% | 100% | 100% |
| Class A2 | 0.02% | 0.06% | 0.01% | 100% | 100% |
| Class B | 0.02% | -0.21% | -0.23% | 99% | 99% |
| Class C | -0.04% | -0.04% | -0.03% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.11% | -0.05% | 99% | 99% |
| Class D | 0.00% | 0.28% | 0.12% | 99% | 95% |
| Class F | -0.04% | -0.03% | -0.04% | 99% | 96% |

# Conclusions

In this contribution, a unification of DCT-IF in VVC is proposed. It is asserted this modification also aligns with the DCT-IF operation in HEVC. It is recommended adopting the proposed changes into VVC.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March 2019, Geneva, CH.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).