| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0261 |
| --- | --- |

| Title: | Non-CE3: MIP with one-stage boundary sample downsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Hongbin Liu, Kai Zhang, Li Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Matrix-based intra prediction (MIP) in current VVC draft applies a two-stage boundary averaging process to generate reduced boundary samples for matrix vector multiplication and prediction upsampling. The two-stage process may cause potential operation dependency and misaligned processes of left and top boundaries. In this contribution, it is proposed to remove the operation dependency in MIP by using a one-stage boundary sample downsamping method. With the proposed method, the dowsampling process is unified for the left and top boundaries. Besides, eight shift and eight add operations are reduced per block.

It is reported that the BD-rate change is minor. Under the CTC, the overall impact on {Y, Cb, Cr} is {-0.01%, -0.09%, 0.00%} for AI and {-0.01%, -0.07%, -0.16%} for RA with minor decoding time saving.

# MIP boundary sample downsampling in VVC

At the 14th JVET meeting, matrix-based intra prediction (MIP) [1] was adopted into the VVC draft 5 [2]. MIP firstly generates a reduced prediction block and then the reduced prediction block is further upsampled to get the final full-size prediction block. For a block with length larger than 8, a two-stage boundary samples generation process is applied. Figure 1 shows an example for a 16x16 block. In the first stage, the sixteen left neighbouring samples are down-sampled to four averaged left samples, and the sixteen top neighbouring samples are down-sampled to eight immediate top samples. In the second stage, the eight immediate top samples are further down-sampled to four averaged top samples.

The current MIP boundary sample downsampling have the following drawbacks:

- The operations for left and top boundaries generation are inconsistent. The four averaged top samples are generated with two stages while the four averaged left samples are generated with one stage. In the prediction upsampling process, the immediate top samples are utilized for vertical interpolation, while original samples on the left boundary are directly used for horizontal interpolation.

- The two cascaded stages require extra storage and unnecessary operation dependency. Firstly, the immediate samples must be stored for the later upsampling process. Secondly, averaged top samples cannot be generated simultaneously with average left samples because they cannot be generated until immediate top samples are generated.

To sum up, the two-stage downsampling method in MIP causes unnecessary computational complexity and operation dependency. Besides, the operations for left and top boundaries generation are inconsistent.

Figure 1: Illustration of MIP for 16×16 blocks

# Proposed solution

To solve the above problems, this contribution proposes a one-stage boundary sample downsampling method for both left and top boundaries. The derivations for reduced boundary samples and upsampling boundary samples are decoupled. Therefore, the extra storage and operation dependency are removed.

Figure 2 shows an example of the proposed method for a 16x16 block. Each consecutive four samples are used to derive one downsampled value as a reduced boundary sample. In the upsampling process, the original samples on both the left and the top boundaries are used to avoid additional storage and extra computational complexity.

With the proposed method, eight shift and eight add operations can be reduced per block.

(a) Generation of reduced boundaries for matrix multiplication      (b) Original boundaries used for prediction upsampling

Figure 2: Proposed boundary sample downsampling and prediction upsampling (16x16 block for example)

# Proposed spec changes

The proposed changes on top of VVC draft 5 [1] are described as below. Newly added parts are highlighted in yellow, and the deleted parts from [1] are highlighted in green strikethrough.

##### 8.4.5.2.3 MIP boundary sample downsampling process

Inputs to this process are:

- a variable nTbS specifying the transform block size,

- reference samples refS[ x ] with x = 0..nTbS − 1,

- a variable boundarySize specifying the downsampled boundary size,

- a flag needUpsBdry specifying whether intermediate boundary samples are required for upsampling,

- a variable upsBdrySize specifying the boundary size for upsampling.

Outputs of this process are the reduced boundary samples redS[ x ] with x = 0..boundarySize − 1 and upsampling boundary samples upsBdryS[ x ] with x = 0..upsBdrySize − 1.

The upsampling boundary samples upsBdryS[ x ] with x = 0..upsBdrySize − 1 are derived as follows:

- If needUpsBdry is equal to TRUE and upsBdrySize is less than nTbS, the following applies:

uDwn = nTbS / upsBdrySize		(8-75)

upsBdryS[ x ] = ( i = 0uDwn - 1refS x*uDwn+i  + ( 1 << ( Log2( uDwn ) − 1 ) )) >> Log2( uDwn )	(8-76)

upsBdryXx=refS[(x+1)*uDwn-1]

- Otherwise (upsBdrySize is equal to nTbS), upsBdryS[ x ] is set equal to refS[ x ].

The reduced boundary samples redS[ x ] with x = 0..boundarySize − 1 are derived as follows:

- If boundarySize is less than upsBdrySize nTbS, the following applies:

bDwn = upsBdrySize nTbS / boundarySize		(8-77)

redS[ x ] = ( i = 0bDwn - 1upsBdryS refS x*bDwn+i  + ( 1 << ( Log2( bDwn ) − 1 ) )) >> Log2( bDwn )	(8-78)

- Otherwise (boundarySize is equal to upsBdrySize nTbS), redS[ x ] is set equal to upsBdryS refS[ x ].

# Simulation results

The proposed method was implemented based on the VTM-5.0 reference software [3] and simulation results based on the common test conditions as specified in [4] are provided in Table 1.

Table 1: Simulation results of the proposed method

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.11% | -0.07% | 100% | 100% |
| Class A2 | -0.01% | -0.09% | -0.11% | 100% | 100% |
| Class B | -0.02% | -0.08% | 0.03% | 100% | 100% |
| Class C | 0.00% | -0.08% | 0.10% | 99% | 96% |
| Class E | -0.01% | -0.07% | -0.03% | 99% | 97% |
| Overall | -0.01% | -0.09% | 0.00% | 100% | 99% |
| Class D | 0.01% | 0.12% | -0.20% | 100% | 96% |
| Class F | 0.00% | 0.05% | 0.04% | 99% | 98% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.24% | 0.01% | 99% | 100% |
| Class A2 | 0.00% | 0.07% | 0.04% | 100% | 100% |
| Class B | -0.01% | -0.04% | -0.27% | 100% | 100% |
| Class C | 0.00% | -0.08% | -0.30% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.16% | 100% | 99% |
| Class D | -0.01% | 0.09% | 0.27% | 100% | 96% |
| Class F | 0.03% | -0.04% | 0.18% | 99% | 98% |

# Conclusions

In this contribution, a one-stage boundary sample downsampling method is proposed to align the left and top boundary operations. With the method, the dependency of the two cascaded stages in the current MIP design is removed. Meanwhile, eight shift and eight add operations are reduced per block. It is recommended adopting this method to VVC.

# References

J. Pfaff, B. Stallenberger, M. Schäfer, “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, March 2019, Geneva, CH.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March 2019, Geneva, CH.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).