| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0262-v1 |
| --- | --- |

| Title: | Non-CE2: Removal of division operations in LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to replace all division operations in LMCS by a lookup table. The lookup table has 64 entries and each stores an 8-bit integer. Simulation results reportedly show 0.00% and 0.00% BD-rate changing under AI and RA configurations, respectively, with no impact on encoding/decoding time.

# Introduction

In the current design of VVC, division operations are involved to derive the LMCS invert-coefficients. In the working draft, the LMCS invert-coefficients denoted as InvScaleCoeff[ i ] are derived as:

The variable LmcsPivot[ i ] with i = 0..16, the variables ScaleCoeff[ i ] and InvScaleCoeff[ i ] with i = 0..15, are derived as follows:

LmcsPivot[ 0 ] = 0;
for( i = 0; i <= 15; i++ ) {
	LmcsPivot[ i + 1 ] = LmcsPivot[ i ] + lmcsCW[ i ] 
	ScaleCoeff[ i ] = ( lmcsCW[ i ] * (1 << 11) + (1 << (Log2(OrgCW) − 1))) >> (Log2(OrgCW)) 	(7-92)
	if ( lmcsCW[ i ] = = 0 )
		InvScaleCoeff[ i ] = 0
	else
		InvScaleCoeff[ i ] = OrgCW * (1 << 11) / lmcsCW[ i ]
}

# Proposed methods

A table-based method is proposed to replace the division operations in LMCS. The lookup table size is 64 bytes. The specification will be changed as bellow:

The variable LmcsPivot[ i ] with i = 0..16, the variables ScaleCoeff[ i ] and InvScaleCoeff[ i ] with i = 0..15, are derived as follows:

LmcsPivot[ 0 ] = 0;
for( i = 0; i <= 15; i++ ) {
LmcsPivot[ i + 1 ] = LmcsPivot[ i ] + lmcsCW[ i ] 
ScaleCoeff[ i ] = ( lmcsCW[ i ] * (1 << 11) + (1 << (Log2(OrgCW) − 1))) >> (Log2(OrgCW)) 
if ( lmcsCW[ i ] = = 0 )

InvScaleCoeff[ i ] = 0
else

{

x = Floor( Log2( lmcsCW[ i ]) )

normDiff = ( ( diff << 6 ) >> x ) & 63

v = divTable[normDiff ] | 256

r = v * OrgCW

precision = 8 + x + (( normDiff  !=  0 )  ?  1  :  0)

if( precision > 11 )

InvScaleCoeff[ i ] = ( r + (1<< (precision – 11) >>1))>> (precision – 11)

else	
    InvScaleCoeff[ i ] = r << (11-precision)

}
}

divTable[ ] is specified as follows:

divTable[] = { 0, 248, 240, 233, 226, 219, 212, 206, 199, 193, 187, 181, 175, 170, 164, 159, 154, 149, 144, 139, 134, 130, 125, 121, 116, 112, 108, 104, 100, 96, 93, 89, 85, 82, 78, 75, 72, 68, 65, 62, 59, 56, 53, 50, 47, 45, 42, 39, 37, 34, 31, 29, 26, 24, 22, 19, 17, 15, 13, 10, 8, 6, 4, 2}.

# Experimental Results

The proposed method has been implemented on top of VTM-5.0. Results under AI/RA configurations are summarized in Table 1. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results of the proposed method on VTM-5.0

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | -0.02% | 0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | -0.03% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.01% | 0.02% | -0.13% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% | -0.02% | -0.08% | -0.15% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 98% |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | -0.02% | -0.08% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% | 0.05% | 0.00% | 0.04% | 100% | 102% |
| Class F | 0.00% | 0.00% | -0.01% | 101% | 102% | 0.01% | 0.00% | 0.01% | 100% | 99% |

# Conclusions

Since the division operations can be removed without coding performance loss, it is suggested that the proposed method be adopted into VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).