| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0263-v1 |
| --- | --- |

| Title: | Non-CE4: Cleanups on syntax design for sub-block coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Na Zhang, Zhipin Deng, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed that the MaxNumSubblockMergeCand should be inferred to be zero if both sps_affine_enabled_flag and slice_temporal_mvp_enabled_flag are equal to 0. No performance change is reported under the common test conditions.

# Introduction

In the current VVC, slice_temporal_mvp_enabled_flag is signaled in the slice header to control the usage of TMVP and SbTMVP in addition to the sequence level controlling. five_minus_max_num_subblock_merge_cand is signaled to indicate the number of subblock merge candidates. When sps_affine_enabled_flag is equal to 0, five_minus_max_num_subblock_merge_cand is not signaled and inferred to be 5 − sps_sbtmvp_enabled_flag, meaning that there is one subblock merge candidate when sps_sbtmvp_enabled_flag is 1. However, when sps_sbtmvp_enabled_flag is 1 but slice_temporal_mvp_enabled_flag is equal to 0, subblock-based TMVP is not allowed. The number of subblock merge candidate should be 0 in such a case when sps_affine_enabled_flag is equal to 0.

# Proposed methods

To solve the problem mentioned above, it is proposed that MaxNumSubblockMergeCand should be inferred to be zero if both sps_affine_enabled_flag and slice_temporal_mvp_enabled_flag are equal to 0.

The semantics is modified as below:

five_minus_max_num_subblock_merge_cand specifies the maximum number of subblock-based merging motion vector prediction (MVP) candidates supported in the slice subtracted from 5. When five_minus_max_num_subblock_merge_cand is not present, it is inferred to be equal to 5 – (sps_sbtmvp_enabled_flag && slice_temporal_mvp_enabled_flag). The maximum number of subblock-based merging MVP candidates, MaxNumSubblockMergeCand is derived as follows:

MaxNumSubblockMergeCand = 5 − five_minus_max_num_subblock_merge_cand

The value of MaxNumSubblockMergeCand shall be in the range of 0 to 5, inclusive.

# Experimental Results

There is no impact on the coding performance of the proposed changes under the common test conditions.

# Conclusions

Considering the syntax/semantics design in VVC needs to be cleanup, it is suggested that the proposed method be adopted into VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).