| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0264 |
| --- | --- |

| Title: | CE6-related: On DCT-8 in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng, Hongbin Liu, Na Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, two methods are proposed to restrict the usage of DCT-8. In both methods, DCT-8 is removed from MTS. Three context coding bins and three arithmetic coding contexts for MTS can be removed accordingly. In method #1, only 4-point DCT-8 is kept. DCT-8 is replaced by DCT-2 for SBT when the block size is larger than 4. In test #2, only 4-point and 8-point DCT-8 are kept. DCT-8 is replaced by DCT-2 for SBT when the block size is larger than 8. The simulation results reportedly show that the BD-rate changes are -0.01%/0.05%, and -0.01%/0.01% for AI/RA in method #1 and #2, respectively.

# Introduction

DCT-8 is one of the candidate transforms in MTS. Besides, it is used in SBT combined with DST-7.

# Proposed methods

To reduce the storage of different transforms, two tests are conducted to restrict the usage of DCT-8. In test #1, DCT-8 is replaced by DCT-2 when the block size is larger than 4 for SBT. In test #2, DCT-8 is replaced by DCT-2 when the block size is larger than 8 for SBT. In both tests, DCT-8 is removed in MTS. MTS can only switch between DCT2-DCT2 and DST7-DST7. Three context coding bins and three arithmetic coding contexts for MTS can be removed.

# Experimental Results.

The proposed method has been implemented on top of VTM-5.0. Results of Test #1 and Test #2 under AI/RA configurations are summarized in Table 1 and Table 2, respectively. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results of the Test #1

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.05% | 0.00% | 98% | 100% | 0.04% | -0.02% | 0.08% | 101% | 101% |
| Class A2 | -0.01% | -0.08% | -0.02% | 100% | 98% | 0.07% | 0.12% | -0.04% | 100% | 101% |
| Class B | -0.01% | -0.06% | -0.06% | 100% | 99% | 0.05% | -0.04% | -0.16% | 100% | 99% |
| Class C | -0.01% | -0.03% | 0.00% | 100% | 99% | 0.03% | -0.19% | 0.03% | 100% | 99% |
| Class E | -0.02% | -0.01% | -0.01% | 99% | 97% |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.02% | 99% | 99% | 0.05% | -0.04% | -0.04% | 100% | 100% |
| Class D | -0.02% | 0.09% | -0.06% | 100% | 100% | 0.00% | -0.05% | 0.08% | 100% | 98% |
| Class F | -0.02% | -0.02% | 0.07% | 99% | 97% | 0.04% | 0.03% | -0.06% | 100% | 100% |

Table 2: Results of the Test #2

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.05% | 0.00% | 99% | 102% | 0.03% | -0.14% | 0.01% | 99% | 101% |
| Class A2 | -0.01% | -0.08% | -0.02% | 102% | 100% | 0.03% | 0.06% | -0.05% | 100% | 101% |
| Class B | -0.01% | -0.06% | -0.06% | 100% | 99% | 0.01% | -0.09% | -0.12% | 100% | 99% |
| Class C | -0.01% | -0.03% | 0.00% | 100% | 100% | -0.01% | -0.18% | -0.08% | 100% | 100% |
| Class E | -0.02% | -0.01% | -0.01% | 99% | 96% |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.02% | 100% | 99% | 0.01% | -0.09% | -0.07% | 100% | 100% |
| Class D | -0.02% | 0.09% | -0.06% | 100% | 96% | -0.04% | 0.06% | 0.20% | 100% | 100% |
| Class F | -0.02% | -0.02% | 0.07% | 100% | 97% | 0.01% | 0.09% | -0.03% | 100% | 100% |

# Conclusions

Considering DCT-8 can be restricted to a size no larger than 8-point in test #2 with almost no loss, it is suggested that the proposed test #2 be adopted into VVC.

# Changes in the working draft

The proposed changes on top of JVET-N1001_v9 are specified as below, with deleted text highlighted in blue and newly added text highlighted in yellow.

Table 8-17 – Specification of trTypeHor and trTypeVer depending on tu_mts_idx[ x ][ y ]

| tu_mts_idx[ x0 ][ y0 ] | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| trTypeHor | 0 | 1 | 2 | 1 | 2 |
| trTypeVer | 0 | 1 | 1 | 2 | 2 |

| Table 8-18 – Specification of trTypeHor and trTypeVer depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag cu_sbt_horizontal_flag; cu_sbt_pos_flag; trTypeHor; trTypeVer. 0; 0; nTbW<=8 ? 2 : 0; 1. 0; 1; 1; 1. 1; 0; 1; nTbH<=8 ? 2 : 0. 1; 1; 1; 1. Table 9-11 – Syntax elements and associated binarizations |
| --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| … |  |  |  |
| transform_unit( ) | tu_mts_idx[ ][ ] | TR FL | cMax = 1 4, cRiceParam = 0 |
| … |  |  |  |

#### 8.4.7.5	Transformation matrix derivation process

…

- Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-1003)

{

{  88   88   87   85   81   77   73   68   62   55   48   40   33   25   17    8 }

{  88   81   68   48   25    0  −25  −48  −68  −81  −88  −88  −81  −68  −48  −25 }

{  87   68   33   −8  −48  −77  −88  −81  −55  −17   25   62   85   88   73   40 }

{  85   48   −8  −62  −88  −77  −33   25   73   88   68   17  −40  −81  −87  −55 }

{  81   25  −48  −88  −68    0   68   88   48  −25  −81  −81  −25   48   88   68 }

{  77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77  −77 }

{  73  −25  −88  −33   68   77  −17  −88  −40   62   81   −8  −87  −48   55   85 }

{  68  −48  −81   25   88    0  −88  −25   81   48  −68  −68   48   81  −25  −88 }

{  62  −68  −55   73   48  −77  −40   81   33  −85  −25   87   17  −88   −8   88 }

{  55  −81  −17   88  −25  −77   62   48  −85   −8   88  −33  −73   68   40  −87 }

{  48  −88   25   68  −81    0   81  −68  −25   88  −48  −48   88  −25  −68   81 }

{  40  −88   62   17  −81   77   −8  −68   87  −33  −48   88  −55  −25   85  −73 }

{  33  −81   85  −40  −25   77  −87   48   17  −73   88  −55   −8   68  −88   62 }

{  25  −68   88  −81   48    0  −48   81  −88   68  −25  −25   68  −88   81  −48 }

{  17  −48   73  −87   88  −77   55  −25   −8   40  −68   85  −88   81  −62   33 }

{   8  −25   40  −55   68  −77   85  −88   88  −87   81  −73   62  −48   33  −17 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15	(8-1004)

transMatrixCol0to15 =	(8-1005)

{

{  90   90   89   88   87   86   85   84   82   80   78   77   74   72   68   66 }

{  90   87   84   78   72   63   53   42   30   17    4   −9  −21  −34  −46  −56 }

{  89   84   74   60   42   21    0  −21  −42  −60  −74  −84  −89  −89  −84  −74 }

{  88   78   60   34    4  −26  −53  −74  −86  −90  −82  −66  −42  −13   17   46 }

{  87   72   42    4  −34  −66  −85  −89  −77  −50  −13   26   60   82   90   80 }

{  86   63   21  −26  −66  −87  −85  −60  −17   30   68   88   84   56   13  −34 }

{  85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53  −85  −85 }

{  84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84  −84  −42   21 }

{  82   30  −42  −86  −77  −17   53   89   68    4  −63  −90  −60    9   72   88 }

{  80   17  −60  −90  −50   30   85   74    4  −68  −87  −38   42   88   66   −9 }

{  78    4  −74  −82  −13   68   85   21  −63  −87  −30   56   89   38  −50  −90 }

{  77   −9  −84  −66   26   88   53  −42  −90  −38   56   87   21  −68  −82   −4 }

{  74  −21  −89  −42   60   84    0  −84  −60   42   89   21  −74  −74   21   89 }

{  72  −34  −89  −13   82   56  −53  −84    9   88   38  −68  −74   30   90   17 }

{  68  −46  −84   17   90   13  −85  −42   72   66  −50  −82   21   90    9  −86 }

{  66  −56  −74   46   80  −34  −85   21   88   −9  −90   −4   89   17  −86  −30 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..15	(8-1006)

transMatrixCol16to31 =	(8-1007)

{

{  63   60   56   53   50   46   42   38   34   30   26   21   17   13    9    4 }

{ −66  −74  −80  −85  −88  −90  −89  −86  −82  −77  −68  −60  −50  −38  −26  −13 }

{ −60  −42  −21    0   21   42   60   74   84   89   89   84   74   60   42   21 }

{  68   84   90   85   72   50   21   −9  −38  −63  −80  −89  −87  −77  −56  −30 }

{  56   21  −17  −53  −78  −90  −84  −63  −30    9   46   74   88   86   68   38 }

{ −72  −89  −82  −53   −9   38   74   90   80   50    4  −42  −77  −90  −78  −46 }

{ −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53 }

{  74   89   60    0  −60  −89  −74  −21   42   84   84   42  −21  −74  −89  −60 }

{  50  −21  −78  −85  −38   34   84   80   26  −46  −87  −74  −13   56   90   66 }

{ −77  −84  −26   53   90   56  −21  −82  −78  −13   63   89   46  −34  −86  −72 }

{ −46   42   90   53  −34  −88  −60   26   86   66  −17  −84  −72    9   80   77 }

{  78   74  −13  −85  −63   30   89   50  −46  −90  −34   60   86   17  −72  −80 }

{  42  −60  −84    0   84   60  −42  −89  −21   74   74  −21  −89  −42   60   84 }

{ −80  −60   50   85   −4  −87  −42   66   77  −26  −90  −21   78   63  −46  −86 }

{ −38   74   63  −53  −80   26   89    4  −87  −34   77   60  −56  −78   30   88 }

{  82   42  −77  −53   68   63  −60  −72   50   78  −38  −84   26   87  −13  −90 }

},

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).