| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0268-v1 |
| --- | --- |

| Title: | AHG17: Signalling SPS base and patch |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes, for discussion, to specify the base part and patch part for the sequence parameter set (SPS) and signal them based on their base and patch types for the SPS referenced by the coding tools. The SPS base may be shared or reused in terms of several aspects for an application. It is alleged this may help improve the sharing and flexibility in the syntax.

Proposal 1: specify and signal a SPS as the SPS base and the SPS patch, and a new NAL unit type of SPS_BASE_NUT.

Proposal 2: specify the sequence parameter set patch structure with the patch types for update processing.

Proposal 3: specify the relative origin information in the SPS.

# Introduction

As drafted in JVET-N1001-v7 [1], the limited number of sequence parameter sets are referenced during encoding process and signalled accordingly in the bitstream for correct decoding on the receiving side.

In the last meeting, there were some discussions about the parameter set sharing for efficiency, e.g. among independent coding layers or independent coding regions.

It is observed that some source related parameters e.g. color space, bit depth, spatial size and position, and some processing control flags etc. may need adaptation to applications, while majority of coding parameters remains the same and can be shared for regions/subpictures, independent coding layers, point cloud data and attributes during the content generating and encoding process.

Basically, a SPS may be viewed as containing the SPS base and the SPS patch.

# Proposed method

This contribution proposes, for discussion, to specify the base part and patch part for the sequence parameter set (SPS) and signal them based on their base and patch types for the SPS referenced by the coding tools. The SPS base may be shared or reused in terms of several aspects for an application, e.g. geometry and spatial relation, colour space and spatial relation, dynamic range and bit depth, and enabled flag and attribute/layer relation etc. It is alleged this may help improve the sharing and flexibility in the standards syntax.

Generally there is a SPS base for reference by a SPS which may include patches if present, and the SPS with patches (the patched SPS) will be referenced as a regular SPS (currently in the VVC draft) by the coding tools. If no patch is present, the “patched” SPS is the same as the SPS base. Both a SPS base and a patched SPS may be delivered via some external means in applications as needed.

The corresponding syntax and semantics are proposed as follows by renaming the current SPS to the SPS base and modifying the current SPS to reference the SPS base with the patch structure. A new NAL unit type of SPS_BASE_NUT is added as shown in Table 7.1.

Sequence parameter set base RBSP syntax

| seq_parameter_set_base_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gra_enabled_flag | u(1) |
| sps_seq_parameter_set_base_id | ue(v) |
| … |  |
| } |  |

Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_base_id | ue(v) |
| sps_seq_parameter_set_patch_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| if( sps_seq_parameter_set_patch_flag ){ |  |
| num_sps_patch_types_minus1 | ue(v) |
| for( i = 0; i <= num_sps_patch_types_minus1; i++ ) { |  |
| sps_patch_type[ i ] | u(3) |
| sps_seq_parameter_set_patch( sps_patch_type[ i ] ) |  |
| } |  |
| } |  |
| } |  |

Table 7-1 – NAL unit type codes and NAL unit type classes

| NalUnitType | Name of NalUnitType | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 1 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| … |  |  |  |
| 17 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| … |  |  |  |
| 21 | SPS_BASE_NUT | Sequence parameter set base seq_parameter_set_base_rbsp( ) | non-VCL |
| … |  |  |  |
| 27 | GRA_NUT | Coded slice of a gradual random access picture slice_layer_rbsp( ) | VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified | non-VCL |

The patch types are proposed as shown in Table 7-X, and the SPS patch structure is proposed as below. For each patch type, there is a list of corresponding syntax elements in the SPS base, which may be updated via the patch structure. The list may be modified as necessary with the evovling VVC working draft. The list of elements in Table 7-X for each patch type is basically in the current VVC draft. In this contribution, additionally, the elements of pic_origin_present_flag, pic_topleft_luma_sample_x, and pic_topleft_luma_sample_y are added to the SPS base for signalling the relative picture origion together with the picture width and picture height.

Table 7-X - SPS patch type code, types of sps patch and elements in SPS

| sps_patch_type | Name of sps_patch_type | Type of SPS patch elements | List of elements in SPS base |
| --- | --- | --- | --- |
| 0 | SPS_PATCH_GMSR | Geometry and spatial relation | pic_width_in_luma_samples,pic_height_in_luma_samples, pic_origin_present_flag, pic_topleft_luma_sample_x, pic_topleft_luma_sample_y, conformance_window_flag |
| 1 | SPS_PATCH_CSCF | Color space and chroma format | chroma_format_idc, separate_colour_plane_flag |
| 2 | SPS_PATCH_DRBD | Dynamic range and bit depth | bit_depth_luma_minus8, bit_depth_chroma_minus8 |
| 3 | SPS_PATCH_EFAR | Enabled flag and attribute/layer relation | gra_enabled_flag, timing_info_present_flag, vui_parameters_present_flag |
| 4..7 | Reserved | Reserved |  |

Sequence parameter set patch syntax

| sps_seq_parameter_set_patch ( sps_patch_type ) { | Descriptor |
| --- | --- |
| if( sps_patch_type == SPS_PATCH_GMSR){ |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_origin_present_flag | u(1) |
| if( pic_origin_present_flag ) { |  |
| pic_topleft_luma_sample_x | ue(v) |
| pic_topleft_luma_sample_y | ue(v) |
| } |  |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| } else if( sps_patch_type == SPS_PATCH_CSCF){ |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| } else if( sps_patch_type == SPS_PATCH_DRBD){ |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| } else if( sps_patch_type == SPS_PATCH_FLAG){ |  |
| gra_enabled_flag | u(1) |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| hrd_parameters_present_flag | u(1) |
| if( hrd_parameters_present_flag ) |  |
| hrd_parameters( sps_max_sub_layers_minus1 ) |  |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| } else { |  |
| … |  |
| } |  |
| } |  |

sps_seq_parameter_set_base_id provides an identifier for the SPS base for reference by other syntax elements. The value of sps_seq_parameter_set_base_id shall be in the range of 0 to 15, inclusive.

sps_seq_parameter_set_id provides an identifier for the SPS for reference by other syntax elements. The value of sps_seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

sps_seq_parameter_set_patch_flag equal to 1 specifies that one or more sps patches are present in the SPS. sps_seq_parameter_set_patch_flag equal to 0 specifies that no sps patch is present in the SPS.

num_sps_patch_types_minus1 plus 1 specifies the number of sps patches inclueded in the SPS. The value of num_sps_patch_types_minus1 shall be in the range of 0 to 15, inclusive.

sps_patch_type[ i ] specifies the sps patch type of the i-th patch in Table 7-X.

pic_origin_flag equal to 1 indicates that the picture origin parameters follow next in the SPS. pic_origin_flag equal to 0 indicates that the the picture origin parameters are not present.

pic_topleft_luma_sample_x and pic_topleft_luma_sample_y specify the location ( x0, y0 ) of the top-left luma sample of each decoded picture in units of luma samples in the CVS, where x0 = pic_topleft_luma_sample_x and y0 = pic_topleft_luma_sample_y, respectively. When not present, they are inferred to be 0.

In the decoding process, the received SPS base is stored for subsequent references by a SPS. When received a SPS, the decoding process parses the SPS for any patches on the referenced SPS base which is stored previously and store the SPS for reference by the coding tool in the decoding process.

The proposal summary:

Proposal 1: specify and signal a SPS as the SPS base and the SPS patch, and a new NAL unit type of SPS_BASE_NUT.

Proposal 2: specify the sequence parameter set patch structure with the patch types for update processing.

Proposal 3: specify the relative origin information in the SPS.

# References

- “Versatile Video Coding (Draft 5),” JVET-N1001-v7, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).