| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0272-v1 |
| --- | --- |

| Title: | CE2-related: Simplified inverse luma mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents two methods to simplify the inverse luma mapping of the luma mapping with chroma scaling (LMCS). In the inverse luma mapping of the VTM5.0 LMCS, for each luma sample, at most 15 comparisons are required to allocate the interval of the luma sample. Since these 15 comparisons are done for each mapped luma sample, in hardware designs, either the critical path of the sample reconstruction becomes much longer or the gate count is significantly increased for parallel comparators, especially when multiple samples are required to be reconstructed at the same time to meet real-time 4K or 8K encoding or decoding. Two methods are proposed to solve the above issue. In method 1, the value range of mapped luma samples, [0, 1023], is divided into 32 equal-size segments of value range. For each mapped luma sample, its segment index can be easily derived by right shifting the mapped luma sample value by five bits, and the interval index of the mapped luma sample is derived by comparing its value with a threshold pre-calculated for the segment. Only one comparison is needed for each luma sample. In method 2, the processes are based on method 1 with reduced bitdepth requirements. It is reported that method 1 shows no BD-rate changes with negligible runtime changes under the common test conditions (CTC), and method 2 shows ….

# Introduction

In VTM5.0 [1] and VVC Draft 5 [2], when performing inverse luma mapping or deriving the chroma scaling factor, an index idxYInv is used to find an inverse scaling coefficient (i.e., InvScaleCoeff[idxYInv]) and an LMCS pivot (i.e., LmcsPivot[idxYInv]). The variable idxYInv indicates the interval index of the luma sample. Since there are 16 intervals in the current design of LMCS, in the worst case, at most 15 comparisons between the input sample (lumaSample) and the LmcsPivot are required. Since these 15 comparisons are done for each mapped luma sample, in hardware designs, either the critical path of the sample reconstruction becomes much longer or the gate count is significantly increased for parallel comparators, especially when multiple samples are required to be reconstructed at the same time to meet real-time 4K or 8K encoding or decoding.

After obtaining the idxYInv, the inverse mapping result of the mapped luma sample (or shortened as inverse luma sample), invSample, is derived as follows.

invSample = InputPivot[ idxYInv ] + ( InvScaleCoeff[ idxYInv ] * ( lumaSample − LmcsPivot[ idxYInv ] ) + ( 1 << 10 ) ) >> 11

# Proposed methods

## Method 1 – Inverse luma mapping with equal-size segments

In method 1, for 10-bit samples, the value range of mapped luma samples, [0, 1023], is divided into 32 equal-size segments of value range, where all the segment sizes are 32. Next, for each segment, there is one segment index, denoted as idxYSeg, and one base interval index, denoted as idxYBase[idxYSeg]. The idxYBase[idxYSeg] of a segment with segment index equal to idxYSeg is calculated as follows. First, the lowest mapped luma sample value in the segment is found; for example, 0, 32, 64… for the first, second, third… segments, respectively. Then, the interval index of the lowest mapped luma sample value in the segment is derived by comparing the lowest mapped luma sample value in the segment with LmcsPivot values (up to 15 comparisons needed), and the idxYBase[idxYSeg] of a segment with segment index equal to idxYSeg is set to the derived interval index. Note that the above process in method 1 is required only when a new adaptation parameter set (APS) of LMCS type is received, not at luma sample level, so the required computational complexity is very low. In the CTC [3], only one APS of LMCS type is sent roughly every second.

After deriving the idxYBase[idxYSeg] of each equal-size segment, inverse mapping of each mapped luma sample in the picture can be performed as follows. For each mapped luma sample, its value is divided by 32 by right shifting five bits without any comparisons to obtain the idxYSeg and then find the pre-calculated idxYBase[idxYSeg]. If the mapped luma sample is larger than or equal to LmcsPivot[baseIdx+1], idxYInv is set to idxYBase[idxYSeg]+1; otherwise, idxYInv is set to idxYBase[idxYSeg]. After the idxYInv is derived, the same equation for the derivation of the inverse luma sample is applied and shown here again as follows.

invSample = InputPivot[ idxYInv ] + ( InvScaleCoeff[ idxYInv ] * ( lumaSample − LmcsPivot[ idxYInv ] ) + ( 1 << 10 ) ) >> 11

As can be seen, only one comparison is needed for each mapped luma sample. In VTM5.0 LMCS, 15 comparisons are needed for each mapped luma sample. The complexity of inverse luma mapping is significantly reduced.

## Method 2 – Simplification of method 1

A more general equation for inverse luma mapping can be written as follows.

invSample = inverse( LmcsSegmentPivot ) + ( invScale * ( lumaSample – LmcsSegmentPivot ) + ( 1 << 10 ) ) >> 11

In method 1, LmcsSegmentPivot, inverse(LmcsSegmentPivot), and invScale are LmcsPivot[idxYInv], InputPivot[idxYInv], InvScaleCoeff[idxYInv], respectively.

In method 2, similar to method 1, for 10-bit samples, the value range of mapped luma samples, [0, 1023], is divided into 32 equal-size segments of value range, where all the segment sizes are 32. Next, for each segment, there is one segment index, denoted as idxYSeg, one base interval index, denoted as idxYBase[idxYSeg], one LMCS segment pivot, denoted as LmcsSegPivot[idxYSeg], and one input segment pivot, denoted as InputSegPivot[idxYSeg]. The idxYBase of a segment is calculated using the same process as that in method 1. The LmcsSegPivot[idxYSeg] of a segment with segment index equal to idxYSeg is calculated as follows. If there is only one turning point of the piecewise linear mapping function within the current segment, then the LmcsSegPivot[idxYSeg] is set to LmcsPivot[idxYBase+1]; otherwise, LmcsSegPivot[idxYSeg] is set to 32*(idxYSeg+1). The InputSegPivot[idxYSeg] of a segment with segment index equal to idxYSeg is the inverse luma sample of LmcsSegPivot[idxYSeg] using the inverse luma mapping in VTM5.0. Note that the above process in method 2 is required only when a new APS of LMCS type is received.

After deriving the idxYBase[idxYSeg], LmcsSegPivot[idxYSeg], and InputSegPivot[idxYSeg] of each equal-size segment, inverse mapping of each mapped luma sample in the picture can be performed as follows. For each mapped luma sample, its value is divided by 32 by right shifting 5 bits without any comparisons to obtain the idxYSeg and then find the pre-calculated idxYBase[idxYSeg] and LmcsSegPivot[idxYSeg]. If the mapped luma sample is larger than or equal to LmcsSegPivot[idxYSeg], idxYInv is set to idxYBase[idxYSeg]+1; otherwise, idxYInv is set to idxYBase[idxYSeg]. After the idxYInv is derived, the equation for the derivation of the inverse luma sample is applied as follows.

invSample =  InputSegPivot[ idxYSeg ] + ( InvScaleCoeff[ idxYInv ] * ( lumaSample − LmcsSegPivot[ idxYSeg ] ) + ( 1 << 10 ) ) >> 11

As can be seen, only one comparison is needed for each mapped luma sample. In VTM5.0 LMCS, 15 comparisons are needed for each mapped luma sample. The complexity of inverse luma mapping is significantly reduced.

Since the value of the term “( lumaSample − LmcsSegPivot[ idxYSeg ] )” used for multiplication is constrained within [-32, 31] (6 bits), the bitdepths of the inverse luma mapping operations are also reduced as compared with VTM5.0 and method 1, where the corresponding term “( lumaSample − LmcsPivot[ idxYInv ] )” is constrained within [0, 511] (9 bits).

Since for each segment the InputSegPivot[idxYSeg] depends on idxYSeg, unlike method 1 in which InputPivot[idxYInv] depends on idxYInv that may have two different values, InputSegPivot[idxYSeg] has only one value. Therefore, during inverse luma mapping, method 2 does not need any multiplexer circuit to select InputSegPivot[idxYSeg].

## Constraint of up to one turning point per segment

In both method 1 and method 2, an encoder-side normative constraint is applied to make sure there is up to one turning point in each equal-size segment. When constructing the piecewise linear luma mapping function, if two turning points in one segment is detected, the number of the codewords (lmcsCW) in the current interval will be increased by an adjustment value, and the lmcsCW in the next interval will be decreased by the same adjustment value. The adjustment value is set to “the lowest mapped luma sample value of the sample value range of the next segment” minus the current LmcsPivot.

# Experiment results

The proposed methods have been implemented on VTM5.0. The results of method 1 and method 2 under the CTC are summarized in Table 1 and Table 2, respectively.

Table 1.  Results of method 1 (to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class F |  |  |  |  |  |

Table 2.  Results of method 2 (to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusions

This contribution presents two methods to perform inverse luma mapping when enabling LMCS. Both methods do not cause noticeable impact on coding efficiency, while the inverse luma mapping is significantly simplified.

# References

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team, JVET-N1001.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).