| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0272-v2 |
| --- | --- |

| Title: | CE2-related: Simplified inverse luma mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the inverse luma mapping of the VTM5.0 luma mapping chroma scaling (LMCS), for each luma sample, at most 15 comparisons are required to allocate the interval of the luma sample. Since these 15 comparisons are done for each mapped luma sample, in hardware designs, either the critical path of the sample reconstruction becomes much longer or the gate count is significantly increased for parallel comparators, especially when multiple samples are required to be reconstructed at the same time to meet real-time 4K or 8K encoding or decoding. In this contribution, one method is proposed to solve the above issue. The value range of mapped luma samples, [0, 1023], is divided into 32 equal-size segments of value range. For each mapped luma sample, its segment index can be easily derived by right shifting the mapped luma sample value by five bits, and the interval index of the mapped luma sample is derived by comparing its value with a threshold pre-calculated for the segment. Only one comparison is needed for each luma sample. It is reported that the proposed method shows no BD-rate changes with negligible runtime changes under the common test conditions (CTC).

# Introduction

In VTM5.0 [1] and VVC Draft 5 [2], when performing inverse luma mapping or deriving the chroma scaling factor, an index idxYInv is used to find an inverse scaling coefficient (i.e., InvScaleCoeff[idxYInv]) and an LMCS pivot (i.e., LmcsPivot[idxYInv]). The variable idxYInv indicates the interval index of the luma sample. Since there are 16 intervals in the current design of LMCS, in the worst case, at most 15 comparisons between the input sample (lumaSample) and the LmcsPivot are required. Since these 15 comparisons are done for each mapped luma sample, in hardware designs, either the critical path of the sample reconstruction becomes much longer or the gate count is significantly increased for parallel comparators, especially when multiple samples are required to be reconstructed at the same time to meet real-time 4K or 8K encoding or decoding.

After obtaining the idxYInv, the inverse mapping result of the mapped luma sample (or shortened as inverse luma sample), invSample, is derived as follows.

invSample = InputPivot[ idxYInv ] + ( InvScaleCoeff[ idxYInv ] * ( lumaSample − LmcsPivot[ idxYInv ] ) + ( 1 << 10 ) ) >> 11

# Proposed method

## Inverse luma mapping with equal-size segments

In the proposed method, for 10-bit samples, the value range of mapped luma samples, [0, 1023], is divided into 32 equal-size segments of value range, where all the segment sizes are 32. Next, for each segment, there is one segment index, denoted as idxYSeg, and one base interval index, denoted as idxYBase[idxYSeg]. The idxYBase[idxYSeg] of a segment with segment index equal to idxYSeg is calculated as follows. First, the lowest mapped luma sample value in the segment is found; for example, 0, 32, 64… for the first, second, third… segments, respectively. Then, the interval index of the lowest mapped luma sample value in the segment is derived by comparing the lowest mapped luma sample value in the segment with LmcsPivot values (up to 15 comparisons needed), and the idxYBase[idxYSeg] of a segment with segment index equal to idxYSeg is set to the derived interval index. Note that the above process in the proposed method is required only when a new adaptation parameter set (APS) of LMCS type is received, not at luma sample level, so the required computational complexity is very low. In the CTC [3], only one APS of LMCS type is sent roughly every second.

After deriving the idxYBase[idxYSeg] of each equal-size segment, inverse mapping of each mapped luma sample in the picture can be performed as follows. For each mapped luma sample, its value is divided by 32 by right shifting five bits without any comparisons to obtain the idxYSeg and then find the pre-calculated idxYBase[idxYSeg]. If the mapped luma sample is larger than or equal to LmcsPivot[baseIdx+1], idxYInv is set to idxYBase[idxYSeg]+1; otherwise, idxYInv is set to idxYBase[idxYSeg]. After the idxYInv is derived, the same equation for the derivation of the inverse luma sample is applied and shown here again as follows.

invSample = InputPivot[ idxYInv ] + ( InvScaleCoeff[ idxYInv ] * ( lumaSample − LmcsPivot[ idxYInv ] ) + ( 1 << 10 ) ) >> 11

As can be seen, only one comparison is needed for each mapped luma sample. In VTM5.0 LMCS, 15 comparisons are needed for each mapped luma sample. The complexity of inverse luma mapping is significantly reduced.

## Constraint of up to one turning point per segment

An encoder-side normative constraint is applied to make sure there is at most one turning point in each equal-size segment. When constructing the piecewise linear luma mapping function, if two turning points in one segment is detected, the number of the codewords (lmcsCW) in the current interval will be increased by an adjustment value, and the lmcsCW in the next interval will be decreased by the same adjustment value. The adjustment value is set to “the lowest mapped luma sample value of the sample value range of the next segment” minus the current LmcsPivot.

# Experiment results

The proposed methods have been implemented on VTM5.0. The results of method 1 under the CTC are summarized in Table 1.

Table 1.  Results of method 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 97% |

# Conclusions

This contribution presents one method to perform inverse luma mapping when enabling LMCS. The proposed method does not cause noticeable impact on coding efficiency, while the inverse luma mapping is significantly simplified.

# References

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team, JVET-N1001.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).