| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0273-v1 |
| --- | --- |

| Title: | CE2-related: Luma-chroma latency reduction for chroma separate tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Cross-component linear model prediction (CCLM) uses reconstructed samples of luma coding block (CB) to generate prediction for chroma CB. When chroma separate tree (CST) is enabled, partitions of luma and chroma coding tree units (CTUs) can be different, which causes latency for chroma CBs to wait for more luma samples to be reconstructed. In VTM5.0, the worse-case latency is 64x64 luma samples. In this proposal, two methods are proposed to reduce the latency to 32x32 luma samples. Method 1 imposes split constraints on both luma and chroma CBs. Method 2 is based on method 1 and allows more split options with implicit transform block (TB) split and constrained intra prediction boundary samples for chroma CBs. It is asserted that the proposed methods preserve most coding gain of CST. Results are reported as follows.

Method 1 – AI: BD-rates = 0.17% (Y), 2.35% (Cb), 2.30% (Cr); EncT=94%; DecT=101%
RA: BD-rates = 0.04% (Y), 0.77% (Cb), 0.78% (Cr); EncT=100%; DecT=98%

Method 2 – AI: BD-rates = 0.03% (Y), 1.19% (Cb), 1.17% (Cr); EncT=100%; DecT=100%
RA: BD-rates = 0.03% (Y), 0.42% (Cb), 0.36% (Cr); EncT=100%; DecT=98%

# Introduction

In VTM5.0, if CST is enabled, the CB partitions of luma and chroma CTUs can be different, so a luma CB may cross multiple chroma CBs partially or vice versa. This introduces processing latency when generating CCLM prediction for chroma CBs by using luma reconstructed samples. For example, as shown in Figure 1, if the upper 32x16 chroma CB is coded with CCLM mode, to generate the CCLM prediction, it has to wait for the reconstruction of the two 32x64 luma CBs. The worse-case latency is 64x64 luma samples.

Figure 1. An example of luma and chroma CB partitions with potential latency issue.

# Proposed methods

To reduce the luma-chroma latency from 64x64 to 32x32 luma samples for CCLM prediction generation when CST is enabled, the following two methods are proposed. In the rest of this contribution, MxN luma samples and PxQ chroma samples are denoted as MxN-L and PxQ-C, respectively.

## Method 1

As shown in Figure 2, split constraints are applied to luma and chrom CBs as follows.

- If luma CB size is 64x64-L, only no split and quadtree (QT) split are allowed.

- If chroma CB size is 32x32-C, only no split and QT split are allowed.

With these constraints, the worse-case latency is 32x32-L and occurs when the following two cases are satisfied: luma case: QT split is applied to a 64x64-L node, and binary tree (BT) or ternary tree (TT) split is applied to the n-th child node of the QT split; chroma case: QT split is applied to a 32x32-C node collocated with the 64x64-L node, and BT or TT split in the opposite direction of the luma BT or TT is applied to the n-th child node of the QT split.

Figure 2. Split constraints of the proposed method 1.

## Method 2

Method 2 is built on top of method 1. Besides, method 2 includes three more chroma split options, as shown in Figure 3.

Option 1:
A 32x32-C parent node is split into two 32x16-C leaf nodes (i.e., CBs).

Option 2:
A 32x32-C parent node is split into two 32x16-C nodes. In each of the two 32x16-C nodes, if further split into child nodes is applied, implicit vertical TB split in the middle of the 32x16-C node is applied.

Option 3:
A 32x32-C parent node is split into two 16x32-C nodes. In each of the two 16x32-C nodes, regardless of whether further split into child nodes is applied, implicit horizontal TB split in the middle of the 16x32-C node is always applied.

Figure 3. Three additional chroma split options

In addition to allowing the above three options, one additional intra prediction boundary sample constraint is applied to option 2 and option 3. For each chroma TB, if any of the left bottom segment of reconstructed boundary samples with vertical position larger than the vertical position of the bottom sample row of the current 16x16-C node does not belong to the reconstructed boundary samples for intra prediction of the current 32x32-C node, it is set to not available for intra prediction; If any of the top right segment of reconstructed boundary samples with horizontal position larger than the horizontal position of the rightmost sample column of the current 16x16-C node does not belong to the reconstructed boundary samples for intra prediction of the current 32x32-C node, it is set to not available for intra prediction. Figure 4 shows two examples.

| (a) | (b) |
| --- | --- |

Figure 4. Two examples to illustrate the intra prediction boundary sample constraint

The purpose of the intra prediction boundary sample constraint is described as follows. With the proposed constraint, after parsing the 32x32-C node, a hardware decoder can perform reconstruction following the raster scan order of 16x16-C nodes to reduce the luma-chroma latency to 32x32-L. The VVC specification does not need to change the TB order in the bitstream.

# Experimental results

The proposed methods are implemented on top of VTM5.0 [2], and the simulation is performed under common test conditions (CTC) as described in JVET-N1010 [3]. Table 1 and Table 2 summarize the results of method 1 and method 2, respectively. Note that if CST is disabled in VTM5.0, under AI condition the BD-rates are 0.41% (Y), 12.73% (Cb), 13.50% (Cr). The coding efficiency penalty of the proposed luma-chroma latency reduction methods is much lower than that of disabling CST.

Table 1.   Results of the proposed method 1 (LB results to be updated)

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.27% | 3.86% | 3.09% | 90% | 100% |
| Class A2 | 0.18% | 2.39% | 1.41% | 92% | 101% |
| Class B | 0.15% | 2.60% | 3.27% | 94% | 101% |
| Class C | 0.09% | 0.46% | 0.62% | 96% | 100% |
| Class E | 0.18% | 2.91% | 3.03% | 95% | 101% |
| Overall | 0.17% | 2.35% | 2.30% | 94% | 101% |
| Class D | 0.03% | 0.52% | 0.27% | 96% | 100% |
| Class F | 0.15% | 1.22% | 1.27% | 96% | 101% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.84% | 0.77% | 100% | 99% |
| Class A2 | 0.09% | 0.95% | 0.70% | 100% | 100% |
| Class B | 0.05% | 1.20% | 1.32% | 100% | 97% |
| Class C | -0.03% | 0.06% | 0.18% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.77% | 0.78% | 100% | 98% |
| Class D | 0.03% | 0.11% | 0.27% | 99% | 100% |
| Class F | 0.05% | 0.69% | 0.80% | 99% | 97% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | 0.01% | 0.03% | 101% | 101% |
| Class E | 0.15% | 1.18% | 1.26% | 100% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.03% | 0.45% | 0.54% | 98% | 99% |
| Class F |  |  |  |  |  |

Table 2.   Results of the proposed method 2 (LB results to be updated)

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 1.76% | 1.46% | 99% | 100% |
| Class A2 | 0.06% | 1.37% | 0.85% | 99% | 100% |
| Class B | 0.03% | 1.31% | 1.57% | 100% | 100% |
| Class C | 0.01% | 0.24% | 0.35% | 100% | 99% |
| Class E | 0.01% | 1.52% | 1.60% | 100% | 102% |
| Overall | 0.03% | 1.19% | 1.17% | 100% | 100% |
| Class D | 0.01% | 0.39% | 0.05% | 100% | 100% |
| Class F | 0.02% | 0.68% | 0.74% | 101% | 105% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.30% | 0.38% | 100% | 99% |
| Class A2 | 0.05% | 0.76% | 0.36% | 100% | 99% |
| Class B | 0.03% | 0.56% | 0.58% | 100% | 98% |
| Class C | 0.02% | 0.08% | 0.07% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.42% | 0.36% | 100% | 98% |
| Class D | 0.03% | 0.12% | 0.17% | 100% | 100% |
| Class F | 0.00% | 0.34% | 0.42% | 101% | 97% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.05% | 0.13% | -0.06% | 101% | 101% |
| Class E | 0.12% | 1.13% | 1.11% | 100% | 98% |
| Overall |  |  |  |  |  |
| Class D | -0.01% | -0.18% | 0.42% | 98% | 99% |
| Class F |  |  |  |  |  |

# Conclusion

In this contribution, two methods are proposed to reduce the luma-chroma latency from 64x64 to 32x32 luma samples for CCLM prediction generation when CST is enabled. The results shows that the proposed methods can preserve most of the coding gain of CST.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 5)," Document of Joint Video Experts Team, JVET-N1001.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).