| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0275-v1 |
| --- | --- |

| Title: | CE3-related: Chroma DM derivation for IBC and PCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Olena Chubach, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, olena.chubach, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM5.0, chroma direct mode (DM) is set to DC when the corresponding luma mode is intra block copy (IBC) or pulse code modulation (PCM). In VVC Draft 5, chroma DM is even undefined when the corresponding luma mode is IBC or PCM. To solve the issue with a unified method considering that a neighbouring block intra mode, if unavailable (e.g., neighbouring block is non-intra, including IBC and PCM), is treated as planar during the most probable mode (MPM) derivation, this contribution proposes to set chroma DM to planar when the corresponding luma mode is IBC or PCM. In addition, a bug fix is proposed to align the VTM software with the VVC specification when generating most probable modes (MPMs) with a neighbouring block coded in PCM. It is reported that this proposal has almost no impact on BD-rates.

# Introduction

When separate trees are used for luma and chroma, a to-be-clarified case exists when chroma DM is selected. Chroma DM is to use the intra prediction mode of the corresponding luma block. If the coresponding luma block is coded with IBC or PCM, the intra prediciton mode is not defined in VVC Draft 5 (JVET-N1001-v8) [1].

In VTM5.0 [2], the buffer to store luma intra prediction mode is initialized with the value of DC mode at the beginning and will not be updated when the luma block is coded with IBC or PCM. Therefore, chroma DM is treated as DC when the corresponding luma mode is IBC or PCM. However, this behavior is inconsistent with the rule for unavailable intra mode of a neighbouring block in MPM generation. When deriving MPMs, if a neighbouring block intra mode is unavailable, e.g., the neighbouring block is non-intra (including IBC and PCM), it is set to planar, not DC.

In addition, according to VVC Draft 5 (JVET-N1001-v8), when deriving MPMs, if a neighbouring block is coded with PCM, which means intraPredModenei is unavailable, intraPredModenei should be set to planar. However, in VTM5.0, since the buffer to store the luma intra prediction mode is initialized with DC and will not be modified if the luma mode is PCM, intraPredModenei is treated as DC instead, which is inconsistent with VVC Draft 5 (JVET-N1001-v8).

# Proposed methods

It is proposed to set chroma DM to planar when the corresponding luma block is coded with IBC or PCM.

When deriving MPMs, if a neighbouring block is coded with PCM, intraPredModenei is set to planar.

# Experimental results

The proposed methods have been implemented on VTM5.0. The simulations were performed following the JVET common test conditions [2] with IBC enabled in both anchor and test. Table 1 shows the simulation results of the proposed methods. As can be observed, BD-rates are close to zero. Runtime data here are inaccurate, but it is believed that this proposal does not cause noticeable runtime changes if measured accurately using homogenous computing clusters.

Table 1.  Results of this proposal

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.04% | 100% | 103% |
| Class A2 | 0.00% | -0.06% | 0.04% | 101% | 105% |
| Class B | 0.00% | 0.07% | 0.00% | 101% | 101% |
| Class C | 0.00% | -0.01% | 0.08% | 100% | 99% |
| Class E | -0.01% | -0.05% | 0.04% | 100% | 98% |
| Overall | 0.00% | -0.01% | 0.02% | 100% | 101% |
| Class D | 0.00% | 0.01% | 0.01% | 102% | 100% |
| Class F (optional) | 0.01% | 0.03% | 0.12% | 101% | 103% |
| TGM | 0.02% | 0.01% | -0.01% | 100% | 100% |

# Conclusions

This contribution proposes a unification scheme to derive chroma DM when the corresponding luma block is coded with IBC or PCM. Also, a bug fix is proposed to align the VTM software with the VVC Draft 5 when generating MPMs with a neighbouring block coded with PCM. The BD-rate impact caused by the proposed unification and fix is very minor.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team, JVET-N1001.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).