| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0276-v1 |
| --- | --- |

| Title: | CE3-related: Simplifications for MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this proposal, three methods are proposed to simplify the VTM5.0 matrix-based intra prediction (MIP).

The reference boundary sample padding in MIP is different from that in non-MIP intra modes. In method 1, the reference boundary sample padding in MIP is replaced with that in non-MIP intra modes. Smoothing reference boundary samples in MIP is also removed. Test results of method 1 are reported as follows.
VTM5.0-AI: {BD-rates = 0.00% (Y), -0.04% (Cb), -0.02% (Cr); EncT = 100%; DecT = 101%}
VTM5.0-RA: {BD-rates = 0.00% (Y), -0.10% (Cb), -0.03% (Cr); EncT = 100%; DecT = 98%}
VTM5.0-LB: {BD-rates = x.xx% (Y), x.xx% (Cb), x.xx% (Cr); EncT = xxx%; DecT = xxx%}

In MIP, a two-stage downsampling is applied to reference boundary samples. Samples are averaged before downsampling. The first stage outputs downsampled averages of samples for upsampling the output vector of the matrix vector multiplication, i.e., for upsampling intermediate prediction samples. The averages of samples from the first stage are input to the second stage to generate further downsampled averages of samples as the input vector to the matrix vector multiplication for generating intermediate prediction samples. In method 2, the two-stage downsampling is replaced with a one-stage downsampling to obtain the input vector to the matrix vector multiplication for generating intermediate prediction samples. For upsampling intermediate prediction samples, the upsampling inputs are changed from averages of samples to reference boundary samples. Results of method 2 are reported as follows.
VTM5.0-AI: {BD-rates = -0.01% (Y), -0.09% (Cb), 0.00% (Cr); EncT = 100%; DecT = 100%}
VTM5.0-RA: {BD-rates = -0.01% (Y), -0.07% (Cb), -0.16% (Cr); EncT = 100%; DecT = 99%}
VTM5.0-LB: {BD-rates = 0.04% (Y), -0.17% (Cb), 0.19% (Cr); EncT = 100%; DecT = 99%}

In MIP, clipping prediction samples is performed after upsampling intermediate prediction samples. In method 3, clipping is done before upsampling. Since bilinear interpolation is used for upsampling, method 3 guarantees that the output samples are all within the valid range according to sample bitdepth. The number of clipping operations is significantly reduced, and the bitdepth for upsampling is also reduced. Test results of method 3 are reported as follows.
VTM5.0-AI: {BD-rates = 0.00% (Y), 0.01% (Cb), 0.00% (Cr); EncT = 100%; DecT = 101%}
VTM5.0-RA: {BD-rates = 0.00% (Y), -0.05% (Cb), 0.01% (Cr); EncT = 101%; DecT = 98%}
VTM5.0-LB: {BD-rates = -0.03% (Y), -0.10% (Cb), 0.17% (Cr); EncT = 99%; DecT = 99%}

Test results of combining the three proposed methods are reported as follows.
VTM5.0-AI: {BD-rates = -0.01% (Y), -0.08% (Cb), -0.02% (Cr); EncT = 101%; DecT = 100%}
VTM5.0-RA: {BD-rates = 0.00% (Y), -0.09% (Cb),-0.12% (Cr); EncT = 100%; DecT = 99%}
VTM5.0-LB: {BD-rates = x.xx% (Y), x.xx% (Cb), x.xx% (Cr); EncT = xxx%; DecT = xxx%}

# Introduction

In VTM5.0, the method used for checking and padding reference boundary samples in MIP is different from that used in non-MIP intra modes. In MIP, the left reference boundary samples and top reference boundary samples are checked separately, and the top-left reference boundary sample is not considered. However, for non-MIP intra mode, the reference boundary samples are checked together, from bottom-left to top-left, and then to top-right. The different method in MIP unnecessarily complicates the design.

After preparing the reference boundary samples, downsampling is applied to the reference boundary samples for matrix vector multiplication and upsampling intermediate prediction samples. In cases that the number of reference boundary samples required by the matrix vector multiplication is different from that required by the upsampling, a two-stage downsampling is applied with averaging samples before downsampling. That is, the reference boundary samples are first downsampled to the size required by the upsampling, and then downsampled to the size required by the matrix vector multiplication. The purpose of the two-stage downsampling is to reuse averaging operations between the two stages. However, using averaged reference boundary samples for upsampling intermediate prediction samples is not necessary and may be even wrong because an averaged reference boundary sample may not belong to any row or column of intermediate prediction samples.

After upsampling intermediate prediction samples, all the prediction samples are clipped within the 10-bit range. Since bilinear interpolation is used for upsampling, it is unnecessary to put clipping after upsampling.

In this contribution, three methods are proposed to solve the above three issues in MIP.

# Proposed methods

## Method 1 – Unification of reference boundary sample padding

The reference boundary samples padding in MIP is aligned with that in non-MIP intra modes. In addition, smoothing reference boundary samples in MIP is removed.

## Method 2 – One-stage downsampling on the reference boundary samples

The downsampled averages of reference boundary samples are replaced with the original reference boundary samples for upsampling. The downsampling is only applied once to generate the input vector to the matrix vector multiplication.

## Method 3 – Clipping before upsampling

The clipping is performed before upsampling. Therefore, the worst case number of clipping operations in a MIP block is reduced from 4096 (64*64) to 64 (8*8). Moreover, since the bitdepth of the intermediate prediction samples are clipped within 10-bit before upsampling, the bitdepth required for upsampling is also reduced.

# Experiment results

The proposed methods have been implemented on VTM5.0 [1]. The common test conditions [2] are used for evaluation. The results of method 1, method 2, method 3, and the combination of the three methods are summarized in Table 1, Table 2, Table 3, and Table 4, respectively.

Table 1.  Results of method 1 (LB to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.02% | 100% | 100% |
| Class A2 | 0.00% | -0.03% | -0.04% | 101% | 103% |
| Class B | 0.00% | -0.06% | -0.01% | 100% | 100% |
| Class C | 0.01% | -0.02% | 0.01% | 100% | 99% |
| Class E | 0.01% | -0.06% | -0.03% | 100% | 102% |
| Overall | 0.00% | -0.04% | -0.02% | 100% | 101% |
| Class D | 0.01% | 0.31% | -0.01% | 102% | 103% |
| Class F | -0.02% | -0.01% | -0.01% | 101% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.21% | 0.15% | 100% | 98% |
| Class A2 | 0.03% | -0.02% | 0.03% | 100% | 99% |
| Class B | 0.01% | 0.01% | -0.03% | 100% | 97% |
| Class C | -0.05% | -0.20% | -0.20% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.10% | -0.03% | 100% | 98% |
| Class D | 0.04% | 0.08% | 0.26% | 99% | 99% |
| Class F | 0.02% | -0.04% | 0.02% | 100% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.05% | 0.04% | -0.07% | 100% | 103% |
| Class E | -0.04% | -0.26% | 0.58% | 100% | 98% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | -0.47% | 0.05% | 98% | 99% |
| Class F | 0.00% | 0.43% | 0.12% | 98% | 98% |

Table 2.  Results of method 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.11% | -0.07% | 100% | 99% |
| Class A2 | -0.01% | -0.09% | -0.11% | 100% | 100% |
| Class B | -0.02% | -0.08% | 0.03% | 100% | 101% |
| Class C | 0.00% | -0.08% | 0.10% | 100% | 99% |
| Class E | -0.01% | -0.07% | -0.03% | 100% | 100% |
| Overall | -0.01% | -0.09% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.12% | -0.20% | 100% | 100% |
| Class F | 0.00% | 0.05% | 0.04% | 101% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.24% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.07% | 0.04% | 100% | 102% |
| Class B | -0.01% | -0.04% | -0.27% | 99% | 97% |
| Class C | 0.00% | -0.08% | -0.30% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.16% | 100% | 99% |
| Class D | -0.01% | 0.09% | 0.27% | 99% | 100% |
| Class F | 0.03% | -0.04% | 0.18% | 99% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.22% | -0.07% | 100% | 99% |
| Class C | 0.07% | 0.11% | 0.18% | 100% | 100% |
| Class E | 0.06% | -1.18% | 0.66% | 100% | 98% |
| Overall | 0.04% | -0.17% | 0.19% | 100% | 99% |
| Class D | -0.01% | -0.96% | 0.22% | 97% | 99% |
| Class F | -0.01% | 0.00% | -0.67% | 97% | 97% |

Table 3.  Results of method 3

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.02% | 100% | 102% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.02% | 0.03% | 100% | 101% |
| Class C | 0.00% | 0.02% | -0.04% | 100% | 99% |
| Class E | 0.00% | 0.01% | 0.04% | 101% | 101% |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.02% | -0.01% | 101% | 103% |
| Class F | 0.01% | -0.05% | 0.05% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | -0.02% | 101% | 99% |
| Class A2 | 0.00% | 0.02% | 0.02% | 100% | 99% |
| Class B | 0.00% | -0.04% | 0.05% | 101% | 97% |
| Class C | -0.02% | -0.09% | 0.00% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.05% | 0.01% | 101% | 98% |
| Class D | 0.01% | 0.05% | 0.05% | 101% | 100% |
| Class F | 0.00% | -0.11% | -0.04% | 101% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.05% | 0.59% | 100% | 99% |
| Class C | -0.02% | 0.04% | -0.11% | 100% | 100% |
| Class E | -0.08% | -0.56% | -0.15% | 100% | 99% |
| Overall | -0.03% | -0.10% | 0.17% | 100% | 99% |
| Class D | 0.05% | 0.05% | 0.27% | 98% | 99% |
| Class F | -0.02% | -0.21% | 0.70% | 98% | 97% |

Table 4.  Results of combining the three methods (LB to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.09% | -0.01% | 101% | 100% |
| Class A2 | -0.02% | -0.06% | -0.08% | 101% | 102% |
| Class B | -0.02% | -0.13% | 0.00% | 101% | 99% |
| Class C | 0.03% | -0.07% | 0.03% | 101% | 100% |
| Class E | -0.01% | -0.02% | -0.07% | 101% | 101% |
| Overall | -0.01% | -0.08% | -0.02% | 101% | 100% |
| Class D | 0.02% | 0.23% | 0.01% | 100% | 100% |
| Class F | 0.01% | 0.04% | 0.05% | 101% | 103% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.19% | 0.01% | 100% | 97% |
| Class A2 | 0.04% | -0.03% | 0.04% | 101% | 102% |
| Class B | 0.00% | -0.09% | -0.29% | 100% | 100% |
| Class C | -0.01% | -0.06% | -0.14% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.09% | -0.12% | 100% | 99% |
| Class D | -0.01% | 0.25% | 0.70% | 100% | 99% |
| Class F | 0.00% | -0.07% | -0.02% | 101% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.03% | 0.04% | -0.05% | 100% | 100% |
| Class E | 0.12% | 0.17% | 0.58% | 100% | 98% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -0.09% | 0.60% | 98% | 99% |
| Class F | 0.00% | -0.06% | -0.56% | 98% | 98% |

# Conclusions

In this contribution, three methods are proposed to simplify the MIP effectively. The coding efficiency impact is reported to be very minor.

# References

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).