| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0277-v1 |
| --- | --- |

| Title: | CE3-related: Simplification and unification for intra reference sample filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 5, intra reference sample filtering including one reference sample smoothing filter and two interpolation filters (Gaussian and DCT-IF) is selected with multiple checks for angular intra prediction modes. This contribution proposes to modify the size-dependent threshold in interpolation filter selection to simplify and unify the intra reference sample filter selections. Two tests are conducted. Test 1 is to test the modified size-dependent threshold. Test 2 is based on test 1 and uses unfiltered reconstructed boundary samples in position dependent intra prediction combination (PDPC) for angular intra prediction modes. Results compared with VTM5.0 is reported as follows.

Test 1 – AI BD-rates = 0.00% (Y), -0.02% (U), -0.03% (V); EncT=100%; DecT=100%
RA BD-rates = 0.00% (Y), -0.08% (U), 0.02% (V); EncT=100%; DecT=97%

Test 2 – AI BD-rates = -0.01% (Y), -0.02% (U), 0.03% (V); EncT=100%; DecT=101%
RA BD-rates = 0.00% (Y), -0.04% (U), -0.05% (V); EncT=100%; DecT=98%

# Introduction

In VVC Draft 5 [1], for a luma intra coding block (CB), if intra sub-partitions (ISP) and intra prediction with non-first reference line are not applied, reference sample smoothing filter is applied to the reconstructed boundary samples of the current luma CB when any of the following conditions is true:

- The luma CB size is larger than 32 luma samples and the current intra mode is planar mode.

- The luma CB size is larger than 32 luma samples and the current intra mode is one of {-14, -12, -10, -6, 2, 34, 66, 72, 76, 78, 80} (the intra prediction mode refers to integer reconstructed boundary samples).

Moreover, Gaussian interpolation filter is applied to generate the intra angular prediction when both of the following conditions are true:

- The current intra mode is not anyone of {-14, -12, -10, -6, 2, 34, 66, 72, 76, 78, 80} (the intra prediction mode refers to fractional reconstructed boundary samples).

- The minimum of “absolute mode number difference between the current intra mode and horizontal mode” and “absolute mode number difference between the current intra mode and vertical mode” is larger than a size-dependent threshold as defined in Table 1, where the variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) ) >> 1 and nTbW and nTbH are transform block width and height, respectively.

Otherwise, DCT-IF interpolation filter is applied.

Table 1. Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 16 | 14 | 2 | 0 | 0 | 0 |

It is shown that when the current intra mode is angular prediction mode, different checks are required to select among one reference sample smoothing filter and two interpolation filters. This contribution proposes to modify the size-dependent threshold to unify the filter selection procedure.

# Proposed Method

This contribution proposes to change the size-dependent threshold from 16 to 24 for CB size of 4x4, 4x8 and 8x4 (i.e., nTbS = 2). Table 2 shows the modification, which corresponds to Table 8-9 in VVC Draft 5. So, for the case of “CB size is 8x4 and the intra mode is 72”, intra reference sample smoothing filtering is not enabled since the absolute mode number difference from vertical mode (50) is not larger than the size dependent threshold, which is 24. Similarly, for the case of “CB size is 4x8 and the intra mode is -6”, intra reference sample smoothing filtering is not enabled since the absolute mode number difference from horizontal mode (18) is not larger than the size dependent threshold, which is 24. With the proposed modification, the size constraint of prohibiting the CB size from being smaller than or equal to 32 for intra reference sample smoothing filtering is automatically fulfilled by the modified table.

Since the table is also used for interpolation filter selection, DCT-IF interpolation filter is always used for 4x4, 4x8, and 8x4 luma CB.

Table 2. Proposed intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 16 24 | 14 | 2 | 0 | 0 | 0 |

With the proposed method, the reference sample filtering for angular intra prediction mode is unified as follows.

For a luma intra coding block (CB), if intra sub-partitions (ISP) and intra prediction with non-first reference line are not applied,

- The luma CB size is larger than 32 luma samples and the current intra mode is planar mode.

- Reference sample smoothing filter is applied to the reconstructed boundary samples of the current luma CB. (no change for planar mode)

- If the minimum of “absolute mode number difference between the current intra mode and horizontal mode” and “absolute mode number difference between the current intra mode and vertical mode” is larger than a size-dependent threshold as defined in Table 2.

- If current intra mode is one of {-14, -12, -10, -6, 2, 34, 66, 72, 76, 78, 80}, reference sample smoothing filter is applied to the reconstructed boundary samples of the current luma CB.

- Otherwise, Gaussian interpolation filter is applied to generate the intra prediction.

Otherwise, DCT-IF interpolation filter is applied.

Besides, for angular intra prediction modes, if unfiltered reconstructed boundary samples are also used in PDPC, then the check can be further removed as follows.

For a luma intra coding block (CB), if intra sub-partitions (ISP) and intra prediction with non-first reference line are not applied,

- The luma CB size is larger than 32 luma samples and the current intra mode is planar mode.

- Reference sample smoothing filter is applied to the reconstructed boundary samples of the current luma CB. (no change for planar mode)

- If the minimum of “absolute mode number difference between the current intra mode and horizontal mode” and “absolute mode number difference between the current intra mode and vertical mode” is larger than a size-dependent threshold as defined in Table 2.

- Gaussian interpolation filter is applied to generate the intra prediction.

Otherwise, DCT-IF interpolation filter is applied.

# Experimental results

The proposed method is implemented on top of VTM5.0 [2], and the simulation is performed under common test conditions (CTC) as described in JVET-N1010 [3]. There are two tests. Test 1 is to test the modified size-dependent threshold. Test 2 is based on test 1 and uses unfiltered reconstructed boundary samples in PDPC for angular intra prediction modes. The results of test 1 and test 2 are summarized in Table 3 and Table 4, respectively.

Table 3. Results of test 1

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.01% | 101% | 101% |
| Class A2 | 0.01% | -0.01% | -0.03% | 100% | 100% |
| Class B | 0.01% | -0.02% | -0.02% | 100% | 100% |
| Class C | -0.01% | -0.06% | -0.03% | 101% | 100% |
| Class E | 0.00% | -0.01% | -0.08% | 100% | 100% |
| Overall | 0.00% | -0.02% | -0.03% | 100% | 100% |
| Class D | -0.05% | 0.17% | -0.09% | 101% | 101% |
| Class F | -0.03% | 0.02% | -0.04% | 100% | 101% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.25% | 0.17% | 100% | 99% |
| Class A2 | 0.02% | -0.05% | 0.09% | 100% | 98% |
| Class B | 0.02% | 0.09% | -0.02% | 100% | 97% |
| Class C | -0.04% | -0.17% | -0.08% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.08% | 0.02% | 100% | 97% |
| Class D | 0.01% | 0.03% | 0.27% | 99% | 99% |
| Class F | -0.01% | 0.02% | -0.04% | 100% | 98% |

Table 4. Results of test 2

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | 0.05% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | -0.02% | 100% | 100% |
| Class B | 0.00% | -0.04% | 0.04% | 100% | 100% |
| Class C | -0.04% | -0.02% | 0.05% | 101% | 106% |
| Class E | 0.01% | -0.07% | 0.04% | 100% | 100% |
| Overall | -0.01% | -0.02% | 0.03% | 100% | 101% |
| Class D | -0.06% | 0.19% | -0.11% | 100% | 101% |
| Class F | -0.10% | -0.05% | -0.06% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.09% | 100% | 99% |
| Class A2 | 0.02% | 0.05% | -0.08% | 100% | 99% |
| Class B | 0.03% | 0.02% | -0.10% | 100% | 97% |
| Class C | -0.06% | -0.22% | -0.05% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.05% | 100% | 98% |
| Class D | -0.05% | 0.02% | 0.04% | 100% | 99% |
| Class F | -0.04% | 0.11% | 0.01% | 100% | 97% |

# Conclusion

This contribution proposes to modify the size-dependent threshold in interpolation filter selection. It is shown that with the modification a unified procedure can be used for intra reference sample filtering. Compared with VTM5.0, the BD-rate change is minor.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, Document of Joint Video Experts Team, JVET-N1001

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

- G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, and M. Karczewicz, “Non-CE3: Intra simplifications,” Document of Joint Video Experts Team, JVET-M0095.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).