| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0278-v3 |
| --- | --- |

| Title: | CE4-related: On signalling of merge flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, chunchia.chen , cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The current merge syntax design may suffer decoder crash and redundant merge flag signalling outside common test conditions (CTCs). This contribution proposes to check the availability of all the following types of merge modes before signalling current type of merge mode flag. With this method, one default type of merge mode is guaranteed and redundant flag signalling can be removed when all the following types of merge modes are not available. The proposed fix has no impact to CTCs, and results of a non-CTC setting are summarized as follows.

Proposed method over VTM5.0 when Triangle Prediction Mode (TPM) is disabled:
VTM5.0-RA: {Y-BD-rate = -0.20%, EncT = 99%, DecT = 100%}
VTM5.0-LB: {Y-BD-rate = -0.22%, EncT = 99%, DecT = 100%}

# Introduction

In JVET-N0324 [1], regular merge flag was adopted for both skip and merge modes with the shortest code word, since regular merge mode was reported as the most frequently selected one among all types of merge modes. By the proposed method, the context coded bins can be decreased accordingly.

However, regular merge mode was used as default mode in the original design. The syntax reorder proposed by JVET-N0324 introduces two problems. First, without the default mode, it may occur that no merge mode are valid. For example, if regular_merge_flag, mmvd_merge_flag, merge_subblock_flag, ciip_flag are all equal to 0 by signalling or inferring, and sps_triangle_enabled_flag is set equal to 0, then none of the merge modes can be used and the decoder may not be able to handle this situation and crash. Second, there is redundant flag signalling when all the following types of merge modes are not available. For example, when sps_triangle_enabled_flag is equal to 0, the signalling of ciip_flag is not necessary and should be inferred to be 1 when regular_merge_flag and mmvd_merge_flag and merge_subblock_flag are all 0.

To deal with these two problems, it is proposed to check whether all the following types of merge modes are available or not before signalling current type of merge mode flag to remove the redundant signalling. Details are described in the following sections.

# Proposed method

It is proposed to check whether all the following types of merge modes are available or not before signalling current type of merge mode flag. If all the following types of merge modes are not available, current type of merge mode flag is not needed and should be inferred, where “one type of merge mode is not available” means that this mode is turned off by SPS or some pre-defined constraints for the current CU.

For example, the ciip_flag should be inferred if triangle mode is not available. For another example, the merge_subblock_flag should be inferred if both triangle mode and CIIP (Combined Inter and Intra Prediction) mode are not available.

Table 1 shows the proposed merge data syntax, and the changes are highlighted in yellow colour. The variables allowMMVD, allowSubblock, allowCIIP, and allowTriangle are defined and derived in merge data semantics as shown in Table 2. All the types of merge flags, including regular_merge_flag, mmvd_merge_flag, merge_subblock_flag, ciip_flag, and MergeTriangleFlag, use proposed merge data semantics variables and the previous merge flag to infer the value of the current merge flag, and the rules of inferring are shown in Table 3.

Table 1. Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( allowMMVD ¦ ¦ allowSubblock ¦ ¦ allowCIIP ¦ ¦ allowTriangle ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( allowMMVD && ( allowSubblock ¦ ¦ allowCIIP ¦ ¦ allowTriangle ) ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( allowSubblock && ( allowCIIP ¦ ¦ allowTriangle ) ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( allowCIIP && allowTriangle ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Table 2. Merge data semantics to derive variables used in merge data syntax

allowMMVD = sps_mmvd_enabled_flag

allowSubblock = MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8

allowCIIP = sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth*cbHeight >= 64 && cbWidth < 128 && cbHeight < 128

allowTriangle = MaxNumTriangleMergeCand >= 2 && slice_type = = B && cbWidth*cbHeight >= 64

Table 3. Merge data semantics to infer merge flags.

When regular_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows: - If all the following conditions are true, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1: - general_merge_flag[ x0 ][ y0 ] is equal to 1. - ( allowMMVD ¦¦ allowSubblock ¦¦ allowCIIP ¦¦ allowTriangle ) is equal to 0. - Otherwise, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When mmvd_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows: - If all the following conditions are true, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1: - general_merge_flag[ x0 ][ y0 ] is equal to 1. - regular_merge_flag[ x0 ][ y0 ] is equal to 0. - allowMMVD is equal to 1. - ( allowSubblock ¦¦ allowCIIP ¦¦ allowTriangle ) is equal to 0. - Otherwise, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When merge_subblock_flag[ x0 ][ y0 ] is not present, it is inferred as follows: - If all the following conditions are true, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 1: - general_merge_flag[ x0 ][ y0 ] is equal to 1. - regular_merge_flag[ x0 ][ y0 ] is equal to 0. - mmvd_merge_flag[ x0 ][ y0 ] is equal to 0. - allowSubblock is equal to 1. - ( allowCIIP ¦¦ allowTriangle ) is equal to 0. - Otherwise, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When ciip_flag [ x0 ][ y0 ] is not present, it is inferred as follows: - If all the following conditions are true, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 1: - general_merge_flag[ x0 ][ y0 ] is equal to 1. - regular_merge_flag[ x0 ][ y0 ] is equal to 0. - mmvd_merge_flag[ x0 ][ y0 ] is equal to 0. - merge_subblock_flag[ x0 ][ y0 ] is equal to 0. - allowCIIP is equal to 1. - allowTriangle is equal to 0. - Otherwise, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 0.

The variable MergeTriangleFlag[ x0 ][ y0 ], which specifies whether triangular shape based motion compensation is used to generate the prediction samples of the current coding unit, when decoding a B slice, is derived as follows: - If all the following conditions are true, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 1: - general_merge_flag[ x0 ][ y0 ] is equal to 1. - regular_merge_flag[ x0 ][ y0 ] is equal to 0. - mmvd_merge_flag[ x0 ][ y0 ] is equal to 0. - merge_subblock_flag[ x0 ][ y0 ] is equal to 0. - ciip_flag[ x0 ][ y0 ] is equal to 0. - allowTriangle is equal to 1. - Otherwise, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 0.

# Experiment results

The proposed method has been implemented on VTM5.0. Since the proposed method does not change results in CTCs, the CTCs [2] with TPM disabled are used for evaluation. Table 4 summarizes the results of the proposed method, the anchor of Table 4 is VTM5.0 with TPM disabled, and the test of Table 4 is VTM5.0 with the proposed method and TPM disabled.

Table 4.  Results of the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with TPM disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.28% | -0.28% | 99% | 99% |
| Class A2 | -0.19% | -0.29% | -0.23% | 99% | 99% |
| Class B | -0.18% | -0.16% | -0.12% | 99% | 102% |
| Class C | -0.22% | -0.18% | -0.20% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.22% | -0.20% | 99% | 100% |
| Class D | -0.19% | -0.39% | -0.25% | 98% | 99% |
| Class F | -0.14% | -0.12% | -0.19% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 with TPM disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.35% | -0.60% | 99% | 101% |
| Class C | -0.20% | -0.43% | -0.30% | 100% | 99% |
| Class E | -0.35% | -0.14% | -0.04% | 99% | 100% |
| Overall | -0.22% | -0.32% | -0.36% | 99% | 100% |
| Class D | -0.28% | -0.59% | -0.85% | 99% | 101% |
| Class F | -0.03% | -0.90% | -1.29% | 99% | 99% |

# Conclusion

In this contribution, it is proposed to solve the issues of decoder crash and redundant merge flag signalling introduced by JVET-N0324. Compared with VTM5.0 when TPM is disabled, minor coding gain can be achieved.

# References

[1] Y.-W. Chen and X. Wang, “CE4: Regular merge flag coding (CE4-1.2.a and CE4-1.2.b),” Document of Joint Video Experts Team, JVET-N0324.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).