| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0281-v1 |
| --- | --- |

| Title: | CE4-related: Alignment of displacement bitdepths for BDOF and PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM5.0, the displacements derived by bi-directional optical flow (BDOF) are constrained in the range of [-32, 32] and have to be represented by 7-bit signed variables. In order to use 6-bit signed variables to represent the displacements, this contribution proposes to clip the displacement within [-32, 31]. Moreover, it is proposed to apply the same constraint to prediction refinement with optical flow (PROF) tested in CE4-2.1 in JVET-O0070 to align the displacement bitdepths for BDOF and PROF. Specifically, the range of PROF displacement is also clipped within [-32, 31] instead of [-64, 63] in CE4-2.1, and the clipping operations applied to the products of gradients and displacements in PROF are removed. With the proposed changes for PROF, BDOF hardware can be reused for performing PROF. Simulation results reportedly show that the BD-rate impact is very minor.

# Introduction

BDOF refines the luma sample predictors for a coding unit (CU) when the CU is coded with bi-prediction, one reference picture is before the current picture and the other reference picture is after the current picture in display order, and the bi-prediction with CU weights (BCW) selects equal weights for list 0 (L0) and list 1 (L1) prediction samples. The eight-tap interpolation is used for generating the initial L0 and L1 prediction samples according to the motion vectors (MVs). Next, a two-level early termination process is performed. If the early termination does not occur, gradients and displacements are derived in order to refine the prediction samples. The gradients are calculated by applying one three-tap filter with coefficients equal to [-1, 0, 1] to eight-tap interpolated results and followed by a right shift operation. The displacements are derived in 1/64-sample precision and clipped in the range of [-32, 32]. The luma prediction samples are refined by products of gradients and displacements.

PROF, proposed as CE4-2.1 in JVET-O0070 [1], applies an optical flow based approach to refine the affine prediction samples. The refinement process is similar to BDOF. In PROF, an affine motion model is utilized to derive the displacements instead of applying the displacement derivation process in BDOF. The derived displacement in PROF is also 1/64-sample precision, which is the same as BDOF, but it is clipped within [-64, 63]. The gradients in PROF are calculated by the same process used in BDOF. The products of gradients and displacements are further clipped within [-8192, 8191] before refining prediction samples.

# Proposed method

In VTM5.0 [2], the BDOF displacements are represented by 7-bit signed variables because the valid range of the displacements is from -32 to 32. In order to use 6-bit signed variables to represent the BDOF displacements, the allowed range of the displacement is changed to [-32, 31]. Moreover, in order to allow reusing the BDOF circuits to perform PROF, the displacement bitdepths of BDOF and PROF should be aligned. Therefore, the range of PROF displacement is also clipped within [-32, 31] instead of [-64, 63] in CE4-2.1, and the clipping operations applied to the products of gradients and displacements in PROF are removed.

# Experimental results

The proposed method has been implemented on VTM5.0. The simulations were performed following the JVET common test conditions (CTCs) [3]. Table 1 shows the simulation results of limiting the BDOF displacement within [-32, 31] instead of [-32, 32]. It is shown that there is almost no impact on coding efficiency. Table 2 shows the simulation results of limiting the BDOF and PROF displacements within [-32, 31] and removing the clipping operations on the products of gradients and displacements in PROF. Compared with the BD-rates reported by CE4-2.1 in JVET-O0070, the proposed changes to PROF do not have any significant impact to coding efficiency of PROF. In both Table 1 and Table 2, the run time data are not accurate. We think all the proposed changes will not cause noticeable runtime changes if measured accurately using homogenous computing clusters.

Table 1. Simulation results of limiting the BDOF displacement within [-32, 31]

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.16% | 0.01% | 100% | 101% |
| Class A2 | 0.00% | 0.02% | -0.01% | 101% | 105% |
| Class B | 0.01% | 0.09% | 0.00% | 100% | 99% |
| Class C | -0.01% | 0.00% | -0.06% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.02% | 100% | 101% |
| Class D | -0.02% | -0.12% | 0.04% | 100% | 104% |
| Class F | 0.01% | 0.02% | 0.02% | 101% | 98% |

Table 2. Simulation results of limiting the BDOF and PROF displacements within [-32, 31] and removing the clipping operations on the products of gradients and displacements in PROF

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.13% | 0.16% | 102% | 100% |
| Class A2 | -0.80% | 0.30% | 0.14% | 102% | 105% |
| Class B | -0.58% | -0.29% | -0.33% | 101% | 100% |
| Class C | -0.32% | -0.19% | -0.26% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.46% | -0.11% | -0.12% | 102% | 101% |
| Class D | -0.56% | -0.20% | -0.03% | 101% | 101% |
| Class F | -0.84% | -0.43% | -0.40% | 101% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | -0.39% | -0.12% | 103% | 105% |
| Class C | -0.21% | -0.08% | -0.18% | 103% | 103% |
| Class E | -0.27% | 0.39% | -0.28% | 105% | 103% |
| Overall | -0.30% | -0.09% | -0.18% | 103% | 104% |
| Class D | -0.38% | -0.41% | 0.43% | 99% | 101% |
| Class F | -1.18% | -0.84% | -1.17% | 100% | 99% |

# Conclusions

In order to allow reusing BDOF circuits to perform PROF, this contribution proposes to change the range of BDOF displacement from [-32, 32] to [-32, 31], and the same constraint is also applied to PROF to align the displacement bitdepths for BDOF and PROF. The clipping operations applied to the products of gradients and displacements in PROF are further removed. The BD-rate changes are very minor.

# References

- J. Luo, Y. He, “CE4-2.1: Prediction refinement with optical flow for affine mode,” Document of Joint Video Experts Team, JVET-O0070.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).