| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0283-v1 |
| --- | --- |

| Title: | CE4-related: Simplification for SbTMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Cheng Lin, Chun-Chia Chen, Chih-Wei Hsu, Tzu-Der Chuang Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yucheng.lin, chunchia.chen, cw.hsu, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to remove all position clipping operations for subblock-based temporal motion vector prediction (SbTMVP) and use a method of two-subblock check to conditionally adjust the temporal motion vector (MV) to zero. It is asserted that the SbTMVP derivation is significantly simplified. It is reported that proposed method results in 0.00% and 0.01% luma BD-rates with 100% and 100% encoding time and 98% and 99% decoding time over VTM5.0-RA and VTM5.0-LB, respectively.

# Introduction

In SbTMVP of VTM5.0 [1] and VVC Draft 5 [2], a luma coding block (CB) is divided into 8x8 luma subblocks, and MV data are derived for each 8x8 luma subblock separately. A temporal MV pointing from the current picture to the collocated picture is derived from a left neighbour of the current coding unit (CU). A position clipping area is defined as “corresponding luma samples of ‘the current luma coding tree block (CTB) and four luma sample columns of luma CTB height to the right of the current luma CTB’ in the collocated picture” excluding samples outside the collocated picture. The center position of the current luma CB is shifted by the temporal MV pointing to the collocated picture, and then the shifted position is clipped within the position clipping area. If the shifted and clipped position from the center of the current luma CB is coded with intra or intra block copy (IBC), SbTMVP is unavailable; otherwise, the MV data of the shifted and clipped position from the center of the current luma CB are used to derive luma CB MV data. For each luma subblock, its center position is also shifted by the temporal MV pointing to the collocated picture, and then the shifted position is clipped within the position clipping area. If the shifted and clipped position from the center of the luma subblock is coded with intra or IBC, luma subblock MV data are set equal to the luma CB MV data; otherwise, the MV data of the shifted and clipped position from the center of the luma subblock are used to derive luma subblock MV data.

Let the position clipping area be represented by xmin, xmax, ymin, and ymax. The position clipping operation is described in Equation (1), where the horizontal position and the vertical position represent the shifted center position of the current luma CB or a luma subblock by the temporal MV.

xmin≤horizontal position→ If false set horizontal position =xminhorizontal position ≤xmax→If false, set horizontal position=xmaxymin≤vertical position →If false, set vertical position=yminvertical position ≤ymax→If false, set vertical position=ymax             (1)

One position clipping operation needs four comparisons. For one 16x16 luma CB, there are five position clipping operations (one for luma CB and four for luma subblocks). For one 128x128 luma CB, there are 257 position clipping operations (one for luma CB and 256 for luma subblocks).

# Proposed method

To simplify SbTMVP, a method of two-subblock check is developed as shown in Figure 1.

Figure 1: Proposed two-subblock check for SbTMVP derivation

Two subblocks at top-left and bottom-right corners of the current luma CB are used to perform the check. The subblock check operation is described in Equation (2), where the horizontal position and the vertical position represent the shifted center position of the top-left or bottom-right subblock by the temporal MV:

xmin≤horizontal position→ If false, set the temporal MV to zerohorizontal position ≤xmax→If false, set the temporal MV to zeroymin≤vertical position →If false, set the temporal MV to zerovertical position ≤ymax→If false, set the temporal MV to zero     (2)

In case of 8x8 luma CB, only one subblock check operation is needed. With the proposed two-subblock check to conditionally adjust the temporal MV to zero, all position clipping operations in the current SbTMVP of VTM5.0 are unnecessary and thus removed. For example, for 128x128 luma CB, the current design needs 257 position clipping operations, but the proposed method only needs two subblock check operations. Table 1 shows more examples.

Table 1: Complexity comparison per CTU

| Luma CTB | SbTMVP in VTM5.0 # of position clipping operations* | Proposed Method # of subblock check operations** |
| --- | --- | --- |
| 256 8x8 luma CBs | (1+1)*256 = 512 | 1*256 = 256 |
| 64 16x16 luma CBs | (1+4)*64 = 320 | 2*64 = 128 |
| 16 32x32 luma CBs | (1+16)*16 = 272 | 2*16 = 32 |
| Four 64x64 luma CBs | (1+64)*4 = 260 | 2*4 = 8 |
| One 128x128 luma CB | (1+256)*1 = 257 | 2*1 = 2 |

*For each position clipping operation, there are 4 comparisons and 2 assignments (horizontal and vertical)

** For each subblock check operation, there are 4 comparisons and 1 assignment

# Experimental results

The proposed method is implemented on top of VTM5.0. All the tests are conducted according to common test conditions (CTCs) [3] for evaluation. Test results of proposed method are summarized in Table 2.

Table 2: Results of the proposed method; anchor: VTM5.0; test: anchor + the proposed method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.08% | 0.01% | 100% | 99% |
| Class A2 | -0.01% | 0.01% | 0.00% | 100% | 101% |
| Class B | 0.01% | 0.03% | -0.07% | 100% | 97% |
| Class C | -0.01% | 0.03% | 0.02% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.02% | 100% | 98% |
| Class D | 0.01% | 0.01% | 0.05% | 99% | 100% |
| Class F (optional) | -0.01% | -0.01% | -0.03% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.40% | 0.16% | 100% | 98% |
| Class C | 0.04% | -0.04% | -0.22% | 100% | 100% |
| Class E | -0.02% | 0.29% | -0.17% | 101% | 99% |
| Overall | 0.01% | 0.23% | -0.05% | 100% | 99% |
| Class D | 0.00% | -0.07% | 0.18% | 98% | 99% |
| Class F (optional) | 0.01% | 0.03% | -0.21% | 97% | 100% |

# Conclusions

In this contribution, it is proposed to remove position clipping operations and to use a method of two-subblock check to conditionally adjust the temporal MV for SbTMVP. It is shown that the proposed method can simplify SbTMVP without coding efficiency loss.

# References

[1] VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-5.0

[2] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team, JVET-N1001

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).