| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0285-v1 |
| --- | --- |

| Title: | CE4-related: On merging candidate list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Cheng Lin, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yucheng.lin, chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to replace the MaxNumMergeCand variable with a fixed value equal to six only during merging candidate list construction. In addition, it is proposed to change the history-based motion vector predictor (HMVP) table size from the variable (MaxNumMergeCand − 1) to five. It is claimed that the hardware decoder can be simplified by removing the support of the variable derivation process. No BD-rate change is reported for the common test conditions (CTC).

Introduction

In VTM5.0 [1], when inserting HMVP candidates to merging candidate list, the last candidate of the list is reserved for pairwise average merging candidate or default candidate, even if the HMVP candidates are not fully used. The last candidate has a merge index equal to (MaxNumMergeCand − 1), in which MaxNumMergeCand is equal to (6 − six_minus_max_num_merge_cand) and represents the maximum number of merging candidates when constructing merging candidate list and signalling merge index. The six_minus_max_num_merge_cand is signalled in slice header. Moreover, the HMVP table size also depends on MaxNumMergeCand. Because of considering the MaxNumMergeCand variable in merging candidate list construction and HMVP table size, decoder has to support different values of MaxNumMergeCand.

In JVET-N0851 [2], it was proposed to fix the MaxNumMergeCand variable and the HMVP table size to six and five, respectively, which is beneficial for hardware design since the decoder can generate merging candidate list by a constant MaxNumMergeCand and select the one indicated by a merge index. There was no action taken on this during the last meeting; however, a ticket [3] after the 14th JVET meeting trying to fix the HMVP table size to five was issued, and the fix has been integrated into VVC Draft 5 [4].

Proposed method

In view of the fixed HMVP table size equal to five in VVC Draft 5, it is proposed to also replace the MaxNumMergeCand with a fixed value equal to six. By both of the modifications, the hardware decoder is capable of constructing the merging candidate list in advance. The hardware decoder does not need to consider different MaxNumMergeCand values which require different circuits in hardware implementation. The implementation effort and cost for hardware are reduced.

Proposed changes to VVC Draft 5

The proposed specification changes are added on top of JVET-N1001-v8 [4]. The changes as summarized below are marked in yellow.

8.5.2.2	Derivation process for luma motion vectors for merge mode

…

6.	The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

7.	When numCurrMergeCand is less than 5(MaxNumMergeCand − 1) and NumHmvpCand is greater than 0, the following applies:

–	The derivation process of history-based merging candidates as specified in 8.5.2.6 is invoked with mergeCandList and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

–	numOrigMergeCand is set equal to numCurrMergeCand.

8.	When numCurrMergeCand is less than 6MaxNumMergeCand and greater than 1, the following applies:

–	The derivation process for pairwise average merging candidate specified in clause 8.5.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0avgCand and refIdxL1avgCand, the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand and the motion vectors mvL0avgCand and mvL1avgCand of candidate avgCand being added into mergeCandList. The bi-prediction weight index bcwIdx of candidate avgCand being added into mergeCandList is set equal to 0.

–	numOrigMergeCand is set equal to numCurrMergeCand.

…

8.5.2.5	Derivation process for zero motion vector merging candidates

…

When numCurrMergeCand is less than 6MaxNumMergeCand, the variable numInputMergeCand is set equal to numCurrMergeCand, the variable zeroIdx is set equal to 0 and the following ordered steps are repeated until numCurrMergeCand is equal to 6MaxNumMergeCand:

…

8.5.2.6	Derivation process for history-based merging candidates

Inputs to this process are:

–	a merge candidate list mergeCandList,

–	the number of available merging candidates in the list numCurrMergeCand.

Outputs to this process are:

–	the modified merging candidate list mergeCandList,

–	the modified number of merging candidates in the list numCurrMergeCand.

The variables isPrunedA1 and isPrunedB1 are both set equal to FALSE.

For each candidate in HmvpCandList[ hMvpIdx ] with index hMvpIdx = 1..NumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to 5(MaxNumMergeCand − 1):

…

Experimental results

The proposed method is implemented on top of VTM5.0. All the tests were conducted under the CTC [5] for evaluation. Test results of proposed method are summarized in Table 1.

Table 1: Results of the proposed method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 98% | 99% |

Conclusions

In this contribution, it is proposed to replace a variable with a constant for merging candidate list construction. It is also proposed to fix the HMVP table size to five. It is claimed that hardware design is simplified. No BD-rate change under the CTC is reported.

References

[1] VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-5.0

[2] C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, Y.-J. Chang, C.-C. Chen, W.-J. Chien, and M. Karczewicz, “On merge list construction,” Document of Joint Video Experts Team, JVET-N0851

[3] JVET VVC Trac Ticket #273, available at https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/273

[4] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team, JVET-N1001

[5] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).