| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0286-v1 |
| --- | --- |

| Title: | CE4-related: Extension of phase-variant affine subblock motion compensation in CE4-2.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Chen-Yen Lai, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, jenny.lai, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an extension of phase-variant affine subblock motion compensation (MC) in CE4-2.5 is proposed. The conventional two-stage horizontal-vertical interpolation is applied. Unlike the phase-variant MC proposed in CE4-2.5 that uses the horizontal filter with the same phase for all samples of a row and the vertical filter with the same phase for all samples of a column, the phase-variant affine subblock MC is extended to allow using filters with different phases within a horizontal row or a vertical column for better approximating the sample-based affine MC in an affine subblock. The used interpolation filter sets in the proposed method are the same as those in VTM5.0. The number of filtering operations for each affine subblock in the proposed method is also the same as that in VTM5.0. Four methods are proposed in this contribution. Method-A is the extended phase-variant MC which uses filters with different phases within a horizontal row or a vertical column. Method-B is the extended phase-variant MC with phase clipping within a row or a column. For both method-A and method-B, at most 11×9 reference samples are needed for a 4×4 luma subblock MC. Method-C is based on method-A with horizontal reference sample padding. Method-D is based on method-B with horizontal reference sample padding. For both method-C and method-D, at most 9×9 reference samples are needed for a 4×4 luma subblock MC. Simulation results reportedly show that, for method-A, the average luma BD-rates are -0.61% and -0.28% in RA and LB, respectively. For method-B, the average luma BD-rates are -0.57% and -0.xx% in RA and LB, respectively. For method-C, the average luma BD-rates are -0.60% and -0.xx% in RA and LB, respectively. For method-D, the average luma BD-rates are -0.57% and -0.29% in RA and LB, respectively.

# Introduction

In VTM5.0, when affine mode is applied, the coding unit (CU) is divided into 4×4 subblocks. The motion vector (MV) of each subblock is derived by an affine motion model. For affine mode MC, the 4×4 block-based translational motion model MC is applied. The 4×4 block-based translational motion model MC can be divided into two stages. The first stage is to filter an (4+L–1)×(4+L–1) reference block window with (4+L–1) rows of horizontal filtering, where the L is the filter tap length of the interpolation filer. For each row, four times of horizontal filtering are performed to generate 4×1 horizontally filtered samples. In the first stage, in total 4×(4+L–1) times of horizontal filtering are performed, and 4×(4+L–1) horizontally filtered samples are generated. The horizontal filter phases of all the sample rows are the same. In the second stage, the 4×(4+L–1) filtered samples are further vertically filtered for generating the four sample columns. For each column, four times of vertical filtering are performed to generate 1×4 vertically filtered samples. In the second stage, in total 4×4 times of vertical filtering are performed, and 4×4 vertically filtered samples are generated as the MC results. The vertical filter phases of all the sample columns are also the same.

In CE4-2.5 [1], phase-variant affine subblock MC is proposed to approximate the affine motion model in an affine subblock. The conventional two-stage horizontal-vertical interpolation is applied. Unlike the phase-invariant block-based MC that uses the same horizontal filter for all samples of a row and the same vertical filter for all samples of a column, different phases of filters can be applied to different sample rows in the affine subblock for horizontal filtering, and different phases of filters can be applied to different sample columns in the affine subblock for vertical filtering. Within the same horizontal row, the same phase of horizontal filter is applied. Also, within the same vertical column, the same phase of vertical filter is applied. In this contribution, the phase-variant affine subblock MC is extended to allow using different phases of filters within a horizontal row or a vertical column for better approximating the sample-based affine MC in an affine subblock.

# Proposed methods

## Method-A: Extended phase-variant MC

To better approximate the affine motion model in an affine subblock, phase-variant MC is extended to allow using different phases of filters within a horizontal row or a vertical column. In the proposed method, the affine coded block is also divided into 4×4 subblocks, and a subblock MV is derived for each subblock as done in VTM5.0. The MC of each subblock is divided into two stages. The first stage is to perform (4+L–1) rows of horizontal filtering to generate a 4×(4+L–1) horizontally filtered samples. However, different from the translational MC, the filter phase of each sample can be variant. For each sample row, four horizontal filtering with variant phases are applied. The MVx is derived as follows.

MVx[rowIdx] [colIdx] = (subblockMVxAdj + mvdRow[rowIdx][colIdx]) >>7       (Equation 1)

mvdRow[rowIdx][colIdx] = dMvVerX×(rowIdx – L/2 – 1) + dMvHorX×(colIdx–1) – (dMvHorX>>1)             																(Equation 2)

The filter phase of each sample of a row is derived from the MVx. Let subblockMVx be the x component of the subblock MV as derived in VTM5.0. The subblockMVxAdj is equal to subblockMVx plus an adjustment term of “isLuma ? ((8 – subblockMVy&15)×dMvVerX)>>4 : ((16 – subblockMVy&31)×dMvVerX)>>5”, where the subblockMVy is the y component of the subblock MV and FractionalPart(subblockMVy) is the fractional part of subblockMVy. This adjustment term only needs to be calculated once per 4x4 subblock or can be pre-calculated with different values of fractional sample part of subblockMVy (0, 1, 2, … to 15 for luma; 0, 1, 2, … to 31 for chroma) and stored in a buffer once per CU. The rowIdx is the sample row index and the colIdx is the sample column index. The dMvVerX is (cuBottomLeftCPMVx – cuTopLeftCPMVx) << (7 – log2LumaCbHeight), where cuBottomLeftCPMVx is the x component of the CU bottom left control point motion vector (CPMV), cuTopLeftCPMVx is the x component of the CU top left CPMV, and LumaCbHeight is the log2 of the height of the luma coding block (CB). The dMvHorX is (cuTopRightCPMVx – cuTopLeftCPMVx) << (7 – log2LumaCbWidth), where cuTopRightCPMVx is the x component of the CU top right CPMV, cuTopLeftCPMVx is the x component of the CU top left CPMV, and log2LumaCbWidth is the log2 of the width of the luma CB. The values stored in mvdRow buffer are pre-calculated once per CU. The values are clipped within one inter sample, e.g. ± (1<<(7+4)) for luma and ± (1<<(7+5)) for chroma. Therefore, the maximum horizontal position difference of the reference samples of the top-row and the bottom-row is two samples. At most a (4+L–1+2)×(4+L–1) reference sample window is required (e.g., 11×9 reference sample window for luma). Compared with the phase-variant affine MC in CE4-2.5, the “dMvHorX×(colIdx–1) – (dMvHorX>>1)” term is added to allow each sample of a row can use different filter phases.

After the horizontal filtering, a 4×(4+L–1) horizontally filtered samples are generated. Figure 1 shows the concept of the proposed horizontal filtering. The gray points are the samples of the reference samples, and the orange points denote the horizontally filtered samples. A blue tube of 6×1 samples denotes applying one time of 6-tap horizontal filtering for luma, as shown in Figure 1 and Figure 2. Each sample row needs four times of horizontal filtering. The filter phases on a sample row can be different.

Figure 1.  Phase-variant horizontal filtering.

Figure 2.  Illustration of applying one time of 6-tap horizontal filtering.

In the second stage, the 4×(4+L–1) horizontally filtered samples (orange samples in Figure 1) are further vertically padded and filtered. For each sample of a column, MVy is derived as follows.

MVy[rowIdx][colIdx] = (subblockMVy + mvdColumn[rowIdx][colIdx]) >>7        (Equation 3)

mvdColumn[rowIdx][colIdx] = dMvHorY×(colIdx–1) – (dMvHorY>>1) + 
dMvVerY×(rowIdx–1) – (dMvVerY >>1)		(Equation 4)

The filter phase of each sample of a column is derived from the MVy. The dMvHorY is (cuTopRightCPMVy – cuTopLeftCPMVy) << (7 – log2LumaCbWidth), where cuTopRightCPMVy is the y component of the CU top right CPMV, and cuTopLeftCPMVy is the y component of the CU top left CPMV. The dMvVerY is (cuBottomLeftCPMVy – cuTopLeftCPMVy) << (7 – log2LumaCbHeight), where cuBottomLeftCPMVy is the y component of the CU bottom left CPMV, and cuTopLeftCPMVy is the y component of the CU top left CPMV. The values stored in mvdColumn buffer are pre-calculated once per CU. The values are clipped within one inter sample, e.g. ± (1<<(7+4)) for luma and ± (1<<(7+5)) for chroma. Compared with the phase-variant affine MC in CE4-2.5, the “dMvVerY×(rowIdx–1) – (dMvVerY >>1)” term is added to allow each sample of a column can use different filter phases.

After the vertical filtering, 4×4 affine subblock prediction samples are generated. Figure 3 shows the concept of the proposed vertical filtering. The light orange points are the horizontally filtered samples from first stage. The green points are the vertically padded samples from the top row and the bottom row. The red points are the vertically filtered samples as the final prediction samples.

Figure 3. Non-uniform phase vertical filtering.

In this proposal, the used interpolation filter sets are the same as those in VTM5.0. The only difference is that the horizontal filter phases can be variant for different sample rows and the vertical filter phases can be variant for different sample columns. As for the number of filtering operations for each affine subblock in the proposed method, it is the same as that in VTM5.0.

In the proposed method, a fallback mode is conditionally applied. When |dMvVerX| < 64 and |dMvHorX| < 64, the phase-invariant horizontal filtering in VTM5.0 is applied. When |dMvHorY| < 64 and |dMvVerY| < 64, the phase-invariant vertical filtering in VTM5.0 is applied.

## Method-B: Extended phase-variant MC with phase clipping within a row or a column

In method-A, the MVx of each sample of a row can be different. The integer part of the MVx of each sample of a row can also be different, which means, compared with CE4-2.5 horizontal filtering, not only the filter phase of the horizontal filtering of each sample can be different, but also the input reference samples of the horizontal filtering of each sample can be different. An additional multiplexer is required to select the input reference samples for each horizontal filter from an 11×1 reference sample line. In this method, the MVx of each sample of a row is clipped to have the same integer part of the MVx. Therefore, compared with CE4-2.5 horizontal filtering, only the filter phase of the horizontal filtering of each sample is variant. The MVx of each sample of a row is clipped within a range of [intRowCenterMVx[rowIdx] to intRowCenterMVx[rowIdx] + (1<<shift)], where the shift is equal to 4 for luma and equal to 5 for chroma. The intRowCenterMVx[rowIdx] is derived as follow.

intRowCenterMVx[rowIdx] = (rowCenterMVx[rowIdx] >> (7+shift)) << shift       	(Equation 5)

rowCenterMVx[rowIdx] = (subblockMVxAdj + mvdRowCenter[rowIdx]) >>7      	(Equation 6)

mvdRowCenter[rowIdx] = dMvVerX×(rowIdx – L/2 – 1)				(Equation 7)

Also, the MVy of each sample of a column is clipped within a range of [intColCenterMVy[colIdx] to intColCenterMVy[colIdx] + (1<<shift)]. The intColCenterMVy[rowIdx] is derived as follow.

intColCenterMVy[colIdx] = (colCenterMVy[colIdx] >> (7+shift)) << shift       	(Equation 8)

colCenterMVy[colIdx] = (subblockMVy + mvdRowCenter[colIdx]) >>7      	(Equation 9)

mvdRowCenter[colIdx] = dMvHorY×(colIdx–1) – (dMvHorY>>1)			(Equation 10)

The values stored in rowCenterMVx and colCenterMVy buffer are pre-calculated once per CU.

## Method-C: Extended phase-variant MC with horizontal reference sample padding

This method is similar to the method-A proposed in Section 2.1, but it only requires a 9×9 luma reference sample window and a 7×7 chroma reference sample window for the phase-variant horizontal filtering. The 9×9 and 7×7 reference sample windows are enlarged to 11×9 and 9×7 reference sample windows by padding, and the rest filtering processes are not changed.

## Method-D: Extended phase-variant MC with phase clipping within a row or a column and with horizontal reference sample padding

This method is similar to the method-B proposed in Section 2.2, but it only requires a 9×9 luma reference sample window and a 7×7 chroma reference sample window for the phase-variant horizontal filtering. The 9×9 and 7×7 reference sample windows are enlarged to 11×9 and 9×7 reference sample windows by padding, and the rest filtering processes are not changed.

# Simulation results

Four tests are evaluated in this contribution.

- Method-A: Extended phase-variant MC, described in Section 2.1

- Method-B: Extended phase-variant MC with phase clipping within a row or a column, described in Section 2.2

- Method-C: Extended phase-variant MC with horizontal reference sample padding, described in Section 2.3

- Method-D: Extended phase-variant MC with phase clipping within a row or a column and with horizontal reference sample padding, described in Section 2.4

The common test conditions (CTC) [2] are used for evaluation. The results of method-A, method-B, method-C, and method-D are shown in Table 1, Table 2, Table 3, and Table 4, respectively. Simulation results reportedly show that, for method-A, the average luma BD-rates are -0.61% and -0.28% in RA and LB, respectively. For method-B, the average luma BD-rates are -0.57% and -0.xx% in RA and LB, respectively. For method-C, the average luma BD-rates are -0.60% and -0.xx% in RA and LB, respectively. For method-D, the average luma BD-rates are -0.57% and -0.29% in RA and LB, respectively.

Table 1.  Results of method-A

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.32% | -0.12% | 104% | 100% |
| Class A2 | -1.07% | -0.88% | -1.03% | 105% | 102% |
| Class B | -0.75% | -0.84% | -0.99% | 103% | 99% |
| Class C | -0.42% | -0.52% | -0.72% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.61% | -0.66% | -0.75% | 104% | 100% |
| Class D | -0.69% | -0.38% | -0.45% | 102% | 100% |
| Class F | -1.64% | -1.88% | -2.02% | 103% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.43% | -0.59% | -0.51% | 104% | 101% |
| Class C | -0.25% | -0.33% | -0.40% | 103% | 102% |
| Class E | -0.09% | -0.08% | -0.07% | 103% | 101% |
| Overall | -0.28% | -0.38% | -0.36% | 103% | 101% |
| Class D | -0.26% | -0.33% | -0.06% | 99% | 100% |
| Class F | -1.95% | -2.56% | -3.06% | 100% | 99% |

Table 2.  Results of method-B (to be updated)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.33% | -0.23% | 105% | 99% |
| Class A2 | -1.00% | -0.92% | -1.00% | 106% | 101% |
| Class B | -0.71% | -0.82% | -0.96% | 104% | 99% |
| Class C | -0.39% | -0.52% | -0.61% | 103% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.57% | -0.66% | -0.73% | 104% | 99% |
| Class D | -0.64% | -0.58% | -0.67% | 102% | 100% |
| Class F | -1.26% | -1.47% | -1.53% | 103% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.25% | -0.39% | -0.40% | 103% | 102% |
| Class E | -0.26% | -0.22% | 0.08% | 104% | 99% |
| Overall |  |  |  |  |  |
| Class D | -0.28% | -0.45% | 0.10% | 99% | 100% |
| Class F | -1.76% | -2.19% | -1.98% | 100% | 99% |

Table 3.  Results of method-C (to be updated)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.34% | -0.09% | 106% | 100% |
| Class A2 | -1.07% | -0.98% | -0.99% | 108% | 103% |
| Class B | -0.74% | -0.89% | -0.93% | 106% | 98% |
| Class C | -0.42% | -0.52% | -0.56% | 105% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.60% | -0.70% | -0.68% | 106% | 100% |
| Class D | -0.68% | -0.56% | -0.63% | 104% | 100% |
| Class F | -1.63% | -1.87% | -1.95% | 105% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.22% | -0.29% | -0.47% | 104% | 102% |
| Class E | -0.21% | -0.34% | -1.23% | 105% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.24% | -0.14% | -0.01% | 100% | 101% |
| Class F | -2.04% | -2.30% | -2.39% | 101% | 99% |

Table 4.  Results of method-D

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.38% | -0.15% | 108% | 101% |
| Class A2 | -1.00% | -0.94% | -0.94% | 110% | 103% |
| Class B | -0.72% | -0.90% | -0.89% | 107% | 101% |
| Class C | -0.39% | -0.44% | -0.60% | 106% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.57% | -0.68% | -0.68% | 108% | 101% |
| Class D | -0.63% | -0.63% | -0.67% | 104% | 102% |
| Class F | -1.27% | -1.44% | -1.50% | 107% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.42% | -0.59% | -0.29% | 109% | 104% |
| Class C | -0.23% | -0.23% | -0.55% | 105% | 103% |
| Class E | -0.14% | 0.18% | -0.65% | 111% | 101% |
| Overall | -0.29% | -0.28% | -0.47% | 108% | 103% |
| Class D | -0.22% | -1.27% | -0.01% | 101% | 105% |
| Class F | -1.68% | -2.17% | -2.31% | 104% | 100% |

# Conclusions

In this contribution, the extension of the phase-variant affine subblock MC is proposed. Different phases of filters are applied to different samples of a row for horizontal filtering and different samples of a column for vertical filtering. The BD-rate is roughly -0.6%, compared with the current phase-invariant affine subblock MC.

# References

- T.-D. Chuang, C.-Y. Lai, Z.-Y. Lin, C.-Y. Chen, Y.-W. Huang, and S.-M. Lei, “CE4-2.5: Phase-variant affine MC,” Document of Joint Video Experts Team, JVET-O0051.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).