| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0287-v2 |
| --- | --- |

| Title: | CE5-related: Simplification of subblock deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
|  | Anand Meher Kotra, Semih Esenlik, Biao Wang, Han Gao, Elena Alshina |  | Anand.meher.kotra@huawei.com |
| Source: | MediaTek Inc., Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the subblock deblocking conditions by using the method proposed in JVET-M0187. In JVET-M0187, to better support parallel deblocking at multiple parallel subblock boundaries on 8x8 grids, if a subblock coding mode is chosen on one side of the subblock boundary, then the maximum allowed filtered samples for the side is set to 3. It is reported that the proposed deblocking results in negligible changes in BD-rate and encoding/decoding time for VTM5.0 common test conditions (CTC) and for VTM5.0 CTC with adaptive loop filter (ALF) off.

# Introduction

In VTM5.0, maxFilterLengthP is used to control the maximum allowed filtered samples for p-side (i.e., the upper side of a horizontal block edge when performing vertical filtering, or the left side of a vertical block edge when performing horizontal filtering), and maxFilterLengthQ is used to control the maximum allowed filtered samples for q-side (i.e., the lower side of a horizontal block edge when performing vertical filtering, or the right side of a vertical block edge when performing horizontal filtering). When performing horizontal filtering across vertical subblock boundaries, as shown in Fig. 1, the configurations of maxFilterLengthP and maxFilterLengthQ for a coding unit (CU) coded with subblock mode (e.g., SbTMVP or affine) are as follows:

- maxFilterLengthP is set to 5 if p-side is coded with subblock coding mode and the vertical subblock boundaries that are the associated transform block boundaries.

- maxFilterLengthQ is set to 5 if q-side is coded with subblock coding mode and the vertical subblock boundaries that are the associated transform block boundaries.

- maxFilterLengthP and maxFilterLengthQ are both set to 2 for the internal vertical subblock boundaries next to the transform block boundary, i.e., only weak filtering can be applied.

- Otherwise, maxFilterLengthP and maxFilterLengthQ are both set to 3 for the rest internal vertical subblock boundaries, i.e., only strong filtering or weak filtering can be applied.

The configuration conditions of maxFilterLengthP and maxFilterLengthQ in performing vertical filtering across horizontal subblock boundaries is similar to that in performing horizontal filtering, but the p-side and q-side are the upper side and lower side of a horizontal block edge.

Figure 1. A horizontal filtering example for the configurations of maxFilterLengthP and maxFilterLengthQ when a subblock coding mode is chosen on one side of a block edge.

# Proposed method

This contribution proposes to set maxFilterLengthP to 3 if p-side is coded with subblock coding mode, and also set maxFilterLengthQ to 3 if q-side is coded with subblock coding mode, as the horizontal filtering example given in Fig. 2.

Figure 2. A horizontal filtering example of the proposed method for configuring of maxFilterLengthP and maxFilterLengthQ when a subblock coding mode is chosen on one side of a block edge.

# Complexity evaluation

Complexity of the proposal is evaluated based on the following criteria described in the CE5 document, JVET-N1025 [1].

Luma deblocking complexity:

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

  - It is the same as in VTM5.0. Except when the edge is shared with a subblock block, then only maximum 3 samples are modified.

- Number of samples from the block boundary that are needed for deblocking decisions:

  - It is the same as in VTM5.0. Except when the edge is shared with a subblock block, then only maximum 4 samples are needed

- Number of filtering operations per sample including decisions and filtering (such as additions, multiplications, comparisons)

  - For all 8x8 blocks case: 9.4 operations (5.6563 for additions, 0.2500 for multiplications, 2.0625 for comparisons, and 1.4375 for shifts), only strong filtering or weak filtering can be applied.

- Number of line buffers:

  - 4 lines (same as VTM5.0)

- Parallelization capabilities, such as size of a unit of a picture that can be processed in parallel to other units

  - All edges overlapping with 8 x 8 grids (same as VTM5.0)

- Whether proposals increase the worst case complexity and if so, by how much: No

Chroma deblocking complexity:

- Chroma filtering is not changed

# Experimental results

The proposed method is implemented on top of VTM5.0 [2], and the simulation is performed under CTC as described in JVET-N1010 [3]. Table 1 and Table 2 show the results for ALF ON and ALF OFF, respectively.

Table 1. Results on top of VTM5.0 for CTC configurations

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.02% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.02% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.04% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.02% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.16% | 0.09% | 100% | 100% |
| Class A2 | 0.01% | 0.08% | -0.01% | 101% | 101% |
| Class B | -0.02% | 0.06% | 0.00% | 101% | 101% |
| Class C | -0.03% | -0.01% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.02% | 101% | 101% |
| Class D | 0.00% | -0.08% | 0.09% | 100% | 100% |
| Class F | -0.01% | 0.08% | 0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | 0.12% | -0.40% | 101% | 103% |
| Class C | -0.01% | 0.25% | 0.16% | 103% | 107% |
| Class E | -0.17% | -0.36% | -0.32% | 100% | 101% |
| Overall | -0.09% | 0.04% | -0.19% | 101% | 104% |
| Class D | -0.02% | -0.17% | 0.23% | 100% | 101% |
| Class F | -0.03% | 0.46% | -0.22% | 102% | 102% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.18% | 0.07% | 101% | 102% |
| Class C | 0.02% | -0.21% | 0.18% | 100% | 100% |
| Class E | -0.04% | 0.69% | -0.19% | 101% | 101% |
| Overall | -0.03% | 0.03% | 0.04% | 101% | 101% |
| Class D | -0.05% | 0.08% | 0.11% | 100% | 100% |
| Class F | -0.06% | 0.17% | 0.34% | 101% | 101% |

Table 2. Results on top of VTM5.0 for CTC with ALF switched off configurations

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.01% | 0.01% | 100% | 98% |
| Class A2 | 0.02% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.02% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.04% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.02% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.01% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.05% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.07% | 0.03% | 101% | 100% |
| Class B | 0.01% | 0.00% | 0.02% | 101% | 100% |
| Class C | -0.01% | 0.14% | 0.11% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | 0.04% | 101% | 100% |
| Class D | 0.00% | 0.05% | -0.04% | 100% | 101% |
| Class F | 0.00% | 0.01% | 0.07% | 101% | 101% |
|  |  |  | Low Delay B |  |  |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.08% | 0.07% | 101% | 100% |
| Class C | -0.06% | -0.08% | -0.17% | 100% | 100% |
| Class E | 0.01% | -0.68% | -0.23% | 100% | 101% |
| Overall | -0.04% | -0.16% | -0.09% | 100% | 100% |
| Class D | 0.01% | 0.11% | -0.46% | 101% | 102% |
| Class F | -0.14% | 0.09% | -0.06% | 100% | 100% |
|  |  |  | Low Delay P |  |  |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.15% | -0.47% | 100% | 100% |
| Class C | 0.01% | -0.20% | -0.01% | 101% | 101% |
| Class E | 0.16% | 0.00% | 1.00% | 101% | 100% |
| Overall | 0.03% | -0.13% | 0.05% | 101% | 100% |
| Class D | 0.02% | 0.32% | 0.22% | 100% | 100% |
| Class F | -0.09% | -0.15% | -0.58% | 101% | 100% |

# Conclusion

In this contribution, the subblock deblocking conditions of maxFilterLengthP and maxFilterLengthQ are simplified. It is suggested to adopt this technique to the VVC draft specification and the VTM reference software.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team, JVET-N1001.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

- C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, “CE11-related: Long deblocking filters with reduced line buffer requirement and enhanced parallel processing accessibility,” Document of Joint Video Experts Team, JVET-M0187.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).