| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0289-v1 |
| --- | --- |

| Title: | CE5-related: Redundancy removal in ALF coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {olena.chubach, chingyeh.chen, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution is related to redundancy removal in adaptive loop filter (ALF) signalling. It proposes two cleanups for coding filter coefficients in ALF. The first cleanup is to signal alf_luma_coeff_delta_flag when there are two or more filters in one filter set. The second cleanup is to avoid signalling alf_luma_eg_order_increase_flag[ i ] and alf_chroma_eg_order_increase_flag[ i ] for i=0. It is reported that the two cleanups result in luma BD-rates as 0.00%, 0.00%, and -0.02% for AI, RA, and LB, respectively, without noticeable runtime changes.

# Introduction

In the current VVC specification working draft [1], when the ALF data syntax elements are signalled, alf_luma_coeff_delta_flag is coded. If this flag is equal to zero, filter coefficient prediction between two successive signalled filters is allowed. One additional flag alf_luma_coeff_delta_prediction_flag is signalled to indicate whether the signalled luma filter coefficients are predicted from the previous luma coefficients.

According to the current signalling design, alf_luma_coeff_delta_prediction_flag is signalled when the number of filter classes is two or more (alf_luma_num_filters_signalled_minus1>0). However, there is no such check on the number of filter classes for alf_luma_coeff_delta_flag.

Considering the above mentined conditions, following issues may appear if there is only one filter class for which luma coefficients can be signalled (alf_luma_num_filters_signalled_minus1=0):

- If a fixed filter set is not used to predict ALF coefficients (alf_luma_use_fixed_filter_flag=0), the filter with all signalled zero coefficients is an identical filter, i.e. the filtered samples will be the same as the input samples, which is identical to ALF OFF case.

- If a fixed filter set is used to predict ALF coefficients (alf_luma_use_fixed_filter_flag=1), the filter with all signalled zero coefficients is the same as the filter in the selected fixed filter set, i.e. the reconstructed filters will be the filters in the selected fixed filter set. This case is identical to directly using the selected fixed filter set.

When the j-th luma filter coefficients alf_luma_coeff_delta_abs[ sfIdx ][ j ] are signalled, they are classified into three coding groups, as it is shown in Figure 1. Then an exp-Golomb code of the order expGoOrderY[ i ] is used to decode the values, where the value of expGoOrderY[ i ] is derived as follows:

expGoOrderY[ i ] = ( i == 0 ? alf_luma_min_eg_order_minus1 + 1 : expGoOrderY[ i − 1 ] )
+ alf_luma_eg_order_increase_flag[ i ]                                    (1)

for i=0, 1, 2.

golombIdx = {

0,

0, 1, 0,

0, 1, 2, 1, 0,

0, 1, 2, x     };

Figure 1. Derivation of the coefficient group order for luma ALF binarization.

Similarly, when the j-th chroma filter coefficients alf_chroma_coeff_abs[ j ] are signalled, they are classified into two coding groups, as it is shown in Figure 2. Then, an exp-Golomb code of the order expGoOrderC[ i ] is used to decode the values, where the value of expGoOrderC[ i ] is derived as follows:

expGoOrderC[i] = ( i == 0 ? alf_chroma_min_eg_order_minus1 + 1 : expGoOrderC[i − 1] ) 
+ alf_chroma_eg_order_increase_flag[ i ]                      	          (2)

for i=0, 1, 2.

golombIdx = {

0,

0, 1, 0,

0, 1, x     };

Figure 2. Derivation of the coefficient group order for chroma ALF binarization.

At the same time, by signalling a value of alf_luma_min_eg_order_minus1 and three increase flags alf_luma_eg_order_increase_flag[ i ] for i=0, 1, and 2, it is possible to obtain four different values of expGoOrderY[ i ]. Since there are only three exponential-Golomb orders needed for coding coefficients of luma ALF (see Figure 1), one alf_luma_eg_order_increase_flag is considered to be redundant.

Correspondingly, by signalling a value of alf_chroma_min_eg_order_minus1 and two increase flags alf_chroma_eg_order_increase_flag[ i ] for i=0, and 1, it is possible to obtain three different values of expGoOrderC[ i ]. Since there are only two exponential-Golomb orders needed for coding coefficients of luma ALF (see Figure 2), one alf_chroma_eg_order_increase_flag is considered to be redundant.

# Proposed methods

## Cleanup 1: Conditional signalling of alf_luma_coeff_delta_flag

As it can be seen, when the number of filters is equal to one (alf_luma_num_filters_signalled_minus1=0) the flag alf_luma_coeff_delta_flag becomes redundant. Thus, it is proposed to signal this flag only when the number of signalled filters is greater than one (alf_luma_num_filters_signalled_minus1>0).  Otherwise, alf_luma_coeff_delta_flag is not signalled and inferred to be equal to zero.

## Cleanup 2: Removal signalling of alf_luma_eg_order_increase_flag[0] and alf_chroma_eg_order_increase_flag[0]

It is proposed not to signal alf_luma_eg_order_increase_flag[ 0 ] and infer its value to be equal to 0. Therefore, the expGoOrderY[ i ] is defined as follows:

expGoOrderY[ i ] = ( i == 0 ? alf_luma_min_eg_order_minus1 + 1 : expGoOrderY[ i − 1 ] ) +
	( i == 0 ? 0 : alf_luma_eg_order_increase_flag[ i ] )                                 (3)

The same modification is proposed for chroma ALF coding: it is proposed not to signal alf_chroma_eg_order_increase_flag[ 0 ] and infer its value to be equal to 0. Therefore, the expGoOrderC[ i ] is derived as follows:

expGoOrderC[ i ] = ( i == 0 ? alf_chroma_min_eg_order_minus1 + 1 : expGoOrderC[ i − 1 ] ) +
                ( i == 0 ? 0 : alf_chroma_eg_order_increase_flag[ i ] )                               (4)

# Proposed modifications to adaptive loop filter data syntax

Table 1. ALF data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … |  |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_flag | u(1) |
| if( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| alf_luma_min_eg_order_minus1 | ue(v) |
| for( i = 01; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if( alf_luma_coeff_delta_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| … |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_clip_flag |  |
| alf_chroma_min_eg_order_minus1 |  |
| for( i = 01; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] |  |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] |  |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] |  |
| } |  |
| … |  |
| } |  |
| } |  |

alf_luma_coeff_delta_flag equal to 1 indicates that alf_luma_coeff_delta_prediction_flag is not signalled. alf_luma_coeff_delta_flag equal to 0 indicates that alf_luma_coeff_delta_prediction_flag may be signalled. When alf_luma_num_filters_signalled_minus1 is equal to 0, alf_luma_coeff_delta_flag is inferred to be equal to 0.

alf_luma_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma filter coefficient signalling is incremented by 1. alf_luma_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma filter coefficient signalling is not incremented by 1. The alf_luma_eg_order_increase_flag[ 0 ] is not signalled and inferred to be equal to 0.

alf_chroma_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma filter coefficient signalling is incremented by 1. alf_chroma_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma filter coefficient signalling is not incremented by 1. The alf_chroma_eg_order_increase_flag[ 0 ] is not signalled and inferred to be equal to 0.

# Experimental results

The proposed methods have been implemented on top of VTM5.0. The common test conditions [2] are used for evaluation. Table 1summarizes the results of applying the two cleanups.

Table 3. Results of applying the two cleanups.

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 68% | 80% |
| Class A2 | 0.00% | 0.00% | 0.00% | 68% | 81% |
| Class B | 0.00% | 0.00% | 0.00% | 67% | 83% |
| Class C | 0.00% | 0.00% | 0.00% | 68% | 81% |
| Class E | 0.00% | 0.00% | 0.00% | 68% | 81% |
| Overall | 0.00% | 0.00% | 0.00% | 68% | 81% |
| Class D | 0.00% | 0.00% | -0.01% | 65% | 73% |
| Class F | 0.00% | 0.00% | 0.00% | 71% | 82% |
|  |  |  | RA Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.01% | 108% | 104% |
| Class A2 | 0.00% | 0.00% | 0.00% | 106% | 106% |
| Class B | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class C | -0.01% | -0.01% | -0.04% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.01% | 104% | 102% |
| Class D | 0.00% | -0.01% | -0.01% | 96% | 99% |
| Class F | 0.00% | 0.01% | 0.00% | 97% | 99% |
|  |  |  | LB Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.16% | 0.53% | 97% | 94% |
| Class C | -0.03% | 0.15% | -0.09% | 99% | 103% |
| Class E | -0.05% | 0.34% | -0.42% | 102% | 103% |
| Overall | -0.02% | 0.20% | 0.09% | 99% | 99% |
| Class D | 0.06% | 0.18% | 0.13% | 97% | 99% |
| Class F | -0.03% | 0.04% | -0.22% | 97% | 95% |

# Conclusion

In this contribution, two ALF coefficient coding cleanups are proposed to remove redundancy. The two cleanups reportedly result in 0.00%, 0.00%, and -0.02% luma BD-rate for AI, RA, and LB, respectively, without noticeable runtime changes.

# References

[1] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team, JVET-N1001.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).