| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0292-v1 |
| --- | --- |

| Title: | CE6-related: Simplifications for LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two methods to simplify transform for intra coding unit (CU) as follows.

- Method 1 is to disable low frequency non-separable transform (LFNST) when multiple transform set (MTS) is used (tu_mtx_idx > 0).

- Method 2 is to remove the option of LFNST index equal to 2 and select the LFNST kernel using the intra prediction mode when LFNST index is equal to 1.

Results of the proposed methods compared with VTM5.0 are reported as follows.

Method 1:
AI: BD-rates = 0.07% (Y), 0.06% (Cb), 0.10% (Cr); EncT=94%; DecT=101%
RA: BD-rates = -0.04% (Y), -0.07% (Cb), -0.02% (Cr); EncT=98%; DecT=97%

Method 2:
AI: BD-rates = 0.12% (Y), 0.82% (Cb), 0.88% (Cr); EncT=82%; DecT=100%
RA: BD-rates = xxx% (Y), xxx% (Cb), xxx% (Cr); EncT=xxx%; DecT=xxx%

# Introduction

LFNST was adopted into VTM5.0 [1]. LFNST can be applied to intra CUs for both luma and chroma. An index, denoted as lfnst_idx, specifies whether LFNST is enabled and which one of the two LFNST kernels in a selected transform set is used. lfnst_idx equal to 0 means LFNST is not used. lfnst_idx equal to 1 or 2 indicates to use the first or second kernel in the selected transform set. The transform set is selected according to the intra predcition mode of the current CU. The specification of the transform set is shown in Table 1.

Table 1. Specification of the traansform set for LFNST

| Intra prediction mode (predModeIntra) | Transform set |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 12 | 1 |
| 13 <= predModeIntra <= 23 | 2 |
| 24 <= predModeIntra <= 44 | 3 |
| 45 <= predModeIntra <= 55 | 2 |
| 56 <= predModeIntra <= 80 | 1 |
| 81 <= predModeIntra <= 83 | 0 |

In VTM5.0, for an intra luma CU, LFNST is performed after primary transform, which can be DCT-II or  non-DCT-II from MTS. In this proposal, it is proposed to decouple the transform processing of MTS and LFNST and reduce the syntax of LFNST as simplification.

# Proposed methods

Two methods are provided to simplify LFNST in terms of transform processing and syntax signalling.

In method 1, LFNST is disabled when MTS is applied (tu_mts_idx > 0).

In method 2, the signalling of lfnst_idx is reduced by removing the option of lfnst_idx equal to 2. lfnst_idx equal to 0 means LFNST is not used. lfnst_idx equal to 1 means LFNST is used, and the LFNST kernel in a transform set is specified according to the intra prediciton mode of the current CU. For chroma, linear model (LM) mode is treated as planar mode. The specification of LFNST kernel for lfnst_idx equal to 1 is shown in Table 2.

Table 2. Proposed selection of LFNST kernel

| Intra prediction mode | LFNST kernel |
| --- | --- |
| INTRA_PLANAR, INTRA_ANGULAR31, INTRA_ANGULAR32, INTRA_ANGULAR34, INTRA_ANGULAR36, INTRA_ANGULAR37 | 1st |
| INTRA_DC, INTRA_ANGULAR33, INTRA_ANGULAR35 | 2nd |
| INTRA_ANGULAR2, INTRA_ANGULAR4,…,INTRA_ANGULAR28, INTRA_ANGULAR30, INTRA_ANGULAR39, INTRA_ANGULAR41,…,INTRA_ANGULAR63, INTRA_ANGULAR65 | 2nd |
| INTRA_ANGULAR3, INTRA_ANGULAR5,…, INTRA_ANGULAR27, INTRA_ANGULAR29, INTRA_ANGULAR38, INTRA_ANGULAR40,…,INTRA_ANGULAR64, INTRA_ANGULAR66 | 1st |

# Experimental results

The proposed methods are implemented on VTM5.0. The simulations were performed following the common test conditions [2]. Table 3 and Table 4 show the simulation results of the method 1 and method 2, respectively.

Table 3.  Results of method 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.15% | 0.20% | 94% | 100% |
| Class A2 | 0.03% | 0.11% | 0.05% | 95% | 100% |
| Class B | 0.07% | 0.08% | 0.16% | 94% | 100% |
| Class C | 0.08% | -0.04% | 0.09% | 94% | 103% |
| Class E | 0.08% | 0.04% | -0.01% | 94% | 100% |
| Overall | 0.07% | 0.06% | 0.10% | 94% | 101% |
| Class D | 0.08% | 0.20% | -0.10% | 94% | 99% |
| Class F | 0.04% | 0.07% | 0.12% | 96% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.14% | 0.09% | 98% | 98% |
| Class A2 | -0.01% | 0.02% | -0.03% | 98% | 100% |
| Class B | -0.04% | -0.06% | -0.11% | 98% | 96% |
| Class C | -0.04% | -0.11% | 0.01% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.07% | -0.02% | 98% | 97% |
| Class D | 0.06% | -0.04% | 0.38% | 96% | 100% |
| Class F | 0.03% | -0.10% | -0.03% | 96% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0% | 2% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0% | 11% |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0% | 10% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0% | 5% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 0% | 40% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 11% |

Table 4.  Results of method 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.89% | 0.96% | 82% | 99% |
| Class A2 | 0.04% | 0.67% | 0.63% | 81% | 100% |
| Class B | 0.10% | 0.85% | 1.02% | 81% | 100% |
| Class C | 0.11% | 0.62% | 0.87% | 82% | 99% |
| Class E | 0.20% | 1.10% | 0.79% | 86% | 106% |
| Overall | 0.12% | 0.82% | 0.88% | 82% | 100% |
| Class D | 0.07% | 0.79% | 0.69% | 82% | 99% |
| Class F | 0.09% | 0.58% | 0.46% | 89% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.22% | 0.65% | 94% | #NUM! |
| Class A2 | 0.92% | 1.91% | 1.60% | 31% | #NUM! |
| Class B | 0.09% | 0.65% | 0.43% | 94% | 97% |
| Class C | 0.03% | 0.22% | 0.50% | 94% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.26% | 0.70% | 0.73% | 75% | #NUM! |
| Class D | 0.07% | 0.32% | 1.03% | 94% | 99% |
| Class F | 0.05% | 0.49% | 0.40% | 94% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0% | 2% |
| Class C | #VALUE! | #VALUE! | #VALUE! | 0% | 11% |
| Class E | #VALUE! | #VALUE! | #VALUE! | 0% | 10% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0% | 5% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 0% | 40% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0% | 11% |

# Conclusions

Two methods are proposed to simplify transform for intra luma CU in terms of transform processing and LFNST signalling. With the proposed methods, the encoder complexity can also be reduced. Compared to VTM5.0, method 1 results in 0.07% and -0.04% luma BD-rates in AI and RA condition with 6% and 2% encoding time reduction, respectively, and method 2 results in 0.12% and xxx% luma BD-rates in AI and RA condition with 18% and xx% encoding time reduction, respectively.

# References

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).