| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0295-v2 |
| --- | --- |

| Title: | CE7-related: Modified subblock coding passes for coding TS residual |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 5, each subblock may need to go through six subblock coding passes for entropy coding the associated coefficient levels. This contribution proposes modified subblock coding passes for coding transform skip (TS) residual. The proposed method integrates multiple subblock coding passes for signalling syntax element abs_level_gtx_flag[n][j], j = 0..4, into one single subblock coding pass, for entropy coding each coefficient at scanning position n in a current subblock. The proposed method further moves coding par_level_flag[n] from the 1st subblock coding pass to a separate subblock coding pass after signaling abs_level_gtx_flag[n][j]. The number of the subblock coding passes is thus reduced to four from six in the proposed method.

The proposed method reportedly provides -0.05%, -0.05%, and 0.01% luma BD-rates for the AI, RA, and LB settings, respectively, for Class F and provides -0.01%, -0.12%, and -0.07% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 SCC common test conditions (CTC).

In version 2, the further experimental results are provided for employing additional context variables for entropy coding abs_level_gtx_flag[n][j], j = 0..4, and par_level_flag[n]. The proposed method reportedly provides -0.23%, -0.16%, and -0.10% luma BD-rates for the AI, RA, and LB settings, respectively, for Class F and provides -0.45%, -0.31%, and -0.20% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 SCC common test conditions (CTC) while increasing the number of context variables by 7.

# Introduction

In VVC Draft 5[1], a separate method is adopted for entropy coding the luma transform blocks (TBs) associated with the TS mode. In this method, each subblock in a current TB is coded by multiple subblock coding passes. The syntax elements sig_coeff_flag[n], coeff_sign_flag[n], abs_level_gtx_flag[n][0], and par_level_flag[n] are signaled in the 1st subblock pass for each coefficient at scanning position n in a current subblock. The syntax elements abs_level_gtx_flag[n][1], abs_level_gtx_flag[n][2], abs_level_gtx_flag[n][3], and abs_level_gtx_flag[n][4] are entropy coded in the subblock coding passes 2, 3, 4, and 5, respectively. The remaining coefficient value abs_remainder[ n ] is coded in the final subblock coding pass. As a result, each subblock may need to go through six subblock coding passes for entropy coding the associated coefficient levels.

This contribution proposes a modified order for signalling syntax elements for improving performance in compressing and complexity.

# Proposed method

## 2.1 Subblock coding passes

In the proposed method, a single subblock coding pass is employed for entropy coding syntax elements abs_level_gtx_flag[n][j], j = 0..4. In this way, the multiple refinement subblock passes for signalling syntax element abs_level_gtx_flag[n][j], j = 0..4, can be integrated into one pass and the number of subblock coding passes can be reduced by 3.

The proposed method further removes coding par_level_flag[n] in the 1st subblock coding pass and adds a separate subblock coding pass dedicated to coding par_level_flag[n], right after the subblock coding pass for coding abs_level_gtx_flag[n][j], as suggested in CE7-3.5 and CE7-3.9 [2], respectively.

In an alternative method, abs_level_gtx_flag[n][0] remains coded in the 1st subblock coding pass and abs_level_gtx_flag[n][j], j = 1..4, are coded in the 2nd subblock coding pass and par_level_flag[n] is coded in the 3rd subblock coding pass.

## Context modeling

In the proposed method, the number of context variables is increased by one for coding each of the syntax elements abs_level_gtx_flag[n][j], j = 0..4. The context increment, ctxInc, for entropy coding abs_level_gtx_flag[n][j] is derived by

ctxInc = ( (absLevelPartA >= ( (j+1 )<< 1 )) || (absLevelPartL >= ( (j+1 )<< 1 )))? 1 : 0.,

where the variables absLevelPartA and absLevelPartL are the absolute values of the partially reconstructed levels of the above and left neighboring coefficients, respectively.

The number of context variables is increased by two for coding par_level_flag. In the proposed method, par_level_flag is coded after signalling abs_level_gtx_flag[n][j], j = 0..4. Context modeling for entropy coding par_level_flag is determined by the absolute values of the partially reconstructed level of the current coefficient after coding abs_level_gtx_flag[n][j], j = 0..4. The context increment, ctxInc, for entropy coding par_level_flag is derived by

ctxInc = ((AbsLevelPassX >> 1) == 1)? 0 : (((AbsLevelPassX >> 2) == 1)? 1 : 2),

where AbsLevelPassX is the partially reconstructed level of the current coefficient and is given by
    AbsLevelPassX =	sig_coeff_flag[n] + abs_level_gtX[n][0] +

2 * (abs_level_gtX[n][1] + abs_level_gtX[n][2]+ abs_level_gtX[n][3]+ abs_level_gtX[n][4]).

# Experimental results

The proposed method was integrated into the VTM5.0 reference software [3]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM5.0 reference software under the SCC CTCs defined for CE8 in [4][5]. Table 1 first summarizes the experimental results generated by the proposed method for coding par_level_flag[n] in a separate subblock coding pass after coding abs_level_gtx_flag[n][j]. Table 2 summarizes the experimental results generated by the proposed method further integrating coding abs_level_gtx_flag[n][j] ], j = 0..4, into one subblock coding pass.

Please note the run time results are not reliable. The detailed simulation results can be found in the attached spread sheets.

Table 1: Summary of the average BD bitrate results using the proposed method by coding par_level_flag[n] in a separate subblock coding pass after coding abs_level_gtx_flag[n][j].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.04% | 0.01% | -0.01% | 100% | 98% |
| TGM | -0.06% | -0.07% | -0.07% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.07% | 0.00% | -0.09% | 100% | 96% |
| TGM | -0.14% | -0.09% | -0.17% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.12% | 0.27% | 0.15% | 99% | 101% |
| TGM | 0.00% | 0.04% | 0.05% | 101% | 99% |

Table 2: Summary of the average BD bitrate results using the proposed method by coding par_level_flag[n] in a separate subblock coding pass after coding abs_level_gtx_flag[n][j], j=0..4, in a single pass.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.05% | -0.04% | -0.05% | 100% | 100% |
| TGM | -0.01% | 0.00% | -0.02% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.05% | -0.11% | -0.05% | 100% | 102% |
| TGM | -0.12% | -0.05% | -0.08% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.01% | -0.67% | -0.43% | 99% | 101% |
| TGM | -0.07% | -0.09% | -0.15% | 97% | 97% |

Table 3: Summary of the average BD bitrate results using the proposed method by coding abs_level_gtx_flag[n][0] in the 1st subblock coding pass and coding par_level_flag[n] in a separate subblock coding pass after coding abs_level_gtx_flag[n][j], j=1..4, in a single pass.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.05% | -0.03% | -0.02% | 103% | 100% |
| TGM | -0.05% | -0.06% | -0.05% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.08% | -0.10% | 0.03% | 100% | 101% |
| TGM | -0.11% | -0.09% | -0.13% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.01% | 0.04% | 0.07% | 102% | 101% |
| TGM | -0.08% | -0.20% | -0.11% | 102% | 104% |

Table 4: Summary of the average BD bitrate results using the proposed method by coding abs_level_gtx_flag[n][0] in the 1st subblock coding pass and coding par_level_flag[n] in a separate subblock coding pass after coding abs_level_gtx_flag[n][j], j=1..4, in a single pass further using 2 additional context variables for coding par_level_flag[n] and 5 additional context variables for coding abs_level_gtx_flag[n][j], j=0..4.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.23% | -0.15% | 0.00% | 104% | 101% |
| TGM | -0.45% | -0.37% | -0.36% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.16% | -0.02% | 0.15% | 102% | 104% |
| TGM | -0.31% | -0.16% | -0.23% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.10% | -0.05% | -0.46% | 99% | 101% |
| TGM | -0.20% | -0.19% | -0.18% | 100% | 99% |

# Conclusions

This contribution proposes modified subblock coding passes for coding TS residual. The experimental results show that the proposed modification for coding par_level_flag slightly improves BD bitrate performance under the SCC CTCs. The proposed modification for integrating the multiple subblock passes for abs_level_gtx_flag[n][j]. j = 0..4, into one single subblock coding pass leads to negligible impacts on BD bitrate performance under the SCC CTCs while reducing the number of subblock coding passes by three. This contribution further proposes the modified method for entropy coding abs_level_gtx_flag[n][j], j = 0..4, and par_level_flag[n]. The proposed method reportedly provides -0.23%, -0.16%, and -0.10% luma BD-rates for the AI, RA, and LB settings, respectively, for Class F and provides -0.45%, -0.31%, and -0.20% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 SCC common test conditions (CTC).

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1001, 14th Meeting: Geneva, CH, 19–27 March 2019.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1028, 14th Meeting: Geneva, CH, 19–27 March 2019.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

- X. Xu, Y.-H. Chao, Y.-C. Sun, and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1028, 14th Meeting: Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).