| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0296-v1 |
| --- | --- |

| Title: | CE8-related: Intra prediction mode for BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Olena Chubach, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, olena.chubach, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM5.0, the intra prediction mode for block-based quantized residual domain differential pulse code modulation (BDPCM) is assigned with planar for reference of the following intra coding blocks (CBs). Since for BDPCM, horizontal or vertical prediction mode is used for intra prediction, the referenced intra prediction mode for BDPCM is inconsistent with the real intra prediction mode for BDPCM. The inconsistency happens in two cases. One is to derive most probable modes (MPMs) with the referenced neighbouring CB coded with BDPCM, and the other is to use chroma direct mode (DM) with the corresponding luma CB coded with BDPCM. This contribution proposes to assign the real intra prediction mode for BDPCM, which can be horizontal or vertical direction depending on intra_bdpcm_dir_flag, for reference. It is reported that, compared to VTM5.0 IBC, the proposed method results in

- Class F: -0.12% and -0.09% luma BD-rates for AI and RA, respectively.

- TGM: -0.18% and -0.18% luma BD-rates for AI and RA, respectively.

In addition, a change for the VVC draft specification is proposed to use transform_not_skip_flag instead of transform_skip_flag to achieve truncated unary binarization for (transform_not_skip_flag, tu_mts_idx).

# Introduction

BDPCM [1] was adopted into VTM5.0 [2]. When BDPCM is applied, an index, denoted as intra_bdpcm_dir_flag, is signalled to indicate whether vertical or horizontal direction is used as the intra prediction mode and the BDPCM direction. intra_bdpcm_dir_flag equal to 0 (or 1) indicates horizontal (or vertical) direction. For a BDPCM CB, after generating intra prediction samples with the selected direction, DPCM is applied to the quantized residual samples with the BDPCM direction and then transform skip (TS) is used. The intra prediction mode for BDPCM is assigned with planar mode for reference of the following CBs. Therefore, two issues are observed as follows. One is that when deriving MPMs, if the referenced neighbouring CB is coded with BDPCM, the intra prediction mode for BDPCM is planar mode rather than the real intra prediciton mode for BDPCM. The other is that when chroma DM is used, if the corresponding luma is a BDPCM CB, the intra prediction mode for BDPCM is planar mode instead of the real intra prediciton mode for BDPCM. This proposal is to solve this inconsistent issue by using the real intra prediciton mode for reference.

In addition, JVET-M0464 [3] was adopted into VTM to combine the syntax of TS and multiple transform set (MTS) with an accociated syntax element, denoted as tu_mts_idx. In JVET-M0464, the binarization of tu_mts_idx is defined in Table 1. However, the text was not changed accordingly. In VVC Draft 5 (JVET-N1001-v8) [4], transform_skip_flag and tu_mts_idx are used to indicate the transform type as follows. transform_skip_flag equal to 1 means no transform is applied and when transform_skip_flag is equal to 0, tu_mts_idx can be signalled to select the transform type from DCT-II and MTS. The binarization of transform_skip_flag and tu_mts_idx is defined in Table 2. When both TS and MTS are enabled, the first bin is for transform_skip_flag.

Table 1. Binarization of tu_mts_idx in JVET-M0464

| tu_mts_idx | transform type |  | Binarization |  |  |
| --- | --- | --- | --- | --- | --- |
|  | horizontal | vertical | MTS & TS enabled | MTS enabled | TS enabled |
| 0 | SKIP |  | 0 | - | 0 |
| 1 | DCT-II | DCT-II | 10 | 0 | 1 |
| 2 | DST-VII | DST-VII | 110 | 10 | - |
| 3 | DCT-VIII | DST-VII | 1110 | 110 | - |
| 4 | DST-VII | DCT-VIII | 11110 | 1110 | - |
| 5 | DCT-VIII | DCT-VIII | 11111 | 1111 | - |

Table 2. Binarization of transform_skip_flag and tu_mts_idx in VVC Draft 5 (JVET-N1001-v8)

| (transform_skip_flag, tu_mts_idx) | transform type |  | binarization |  |  |
| --- | --- | --- | --- | --- | --- |
|  | horizontal | vertical | MTS & TS enabled | MTS enabled | TS enabled |
| (1, -) | SKIP |  | 1 | - | 1 |
| (0, 0) | DCT-II | DCT-II | 00 | 0 | 0 |
| (0, 1) | DST-VII | DST-VII | 010 | 10 | - |
| (0, 2) | DCT-VIII | DST-VII | 0110 | 110 | - |
| (0, 3) | DST-VII | DCT-VIII | 01110 | 1110 | - |
| (0, 4) | DCT-VIII | DCT-VIII | 01111 | 1111 | - |

# Proposed methods

It is proposed that the intra prediction mode for BDPCM is assigned with horizontal or vertical direction (depending on intra_bdpcm_dir_flag) for reference of the following CBs. intra_bdpcm_dir_flag equal to 0 (or 1) means the intra prediction mode for BDPCM is set as horizontal (or vertical) direction. With the proposed method, in the following two cases, the referenced intra prediction mode for BDPCM is the real intra prediction mode for BDPCM.

- Deriving MPMs with the referenced neighbouring CB coded with BDPCM

- Using chroma DM with the corresponding luma CB coded with BDPCM

A change is proposed to use “transform_not_skip_flag” instead of “transform_skip_flag” to indicate whether to use TS. transform_not_skip_flag equal to 0 means no transform is applied and transform_not_skip_flag equal to 1 means DCT-II or MTS can be applied. The binarization of transform_not_skip_flag and tu_mts_idx is defined in Table 3. In this way, truncated unary binarization is achieved.

Table 3. Binarization of proposed transform_not_skip_flag and tu_mts_idx

| (transform_not_skip_flag, tu_mts_idx) | transform type |  | binarization |  |  |
| --- | --- | --- | --- | --- | --- |
|  | horizontal | vertical | MTS & TS enabled | MTS enabled | TS enabled |
| (0, -) | SKIP |  | 0 | - | 0 |
| (1, 0) | DCT-II | DCT-II | 10 | 0 | 1 |
| (1, 1) | DST-VII | DST-VII | 110 | 10 | - |
| (1, 2) | DCT-VIII | DST-VII | 1110 | 110 | - |
| (1, 3) | DST-VII | DCT-VIII | 11110 | 1110 | - |
| (1, 4) | DCT-VIII | DCT-VIII | 11111 | 1111 | - |

# Experimental results

The proposed method has been implemented on VTM5.0. The simulations were performed following the JVET common test conditions (CTC) [5] with IBC enabled in both anchor and test. Table 4 shows the simulation results of the proposed method. The runtime data are not accurate. It is claimed that the runtime change will not be noticeable if measured accurately using homogeneous computing clusters.

Table 4.  Results of this proposal (LB to be updated)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 146% | 125% |
| Class A2 | 0.00% | 0.00% | 0.00% | 148% | 125% |
| Class B | 0.00% | 0.00% | 0.00% | 150% | 125% |
| Class C | 0.00% | 0.00% | 0.00% | 151% | 132% |
| Class E | 0.00% | 0.00% | 0.00% | 146% | 122% |
| Overall | 0.00% | 0.00% | 0.00% | 149% | 126% |
| Class D | 0.00% | 0.00% | 0.00% | 148% | 129% |
| Class F (optional) | -0.12% | 0.04% | -0.08% | 147% | 125% |
| TGM | -0.18% | -0.06% | -0.07% | 146% | 121% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 (enable IBC) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 140% | 128% |
| Class A2 | 0.00% | 0.00% | 0.00% | 141% | 117% |
| Class B | 0.00% | 0.00% | 0.00% | 141% | 128% |
| Class C | 0.00% | 0.00% | 0.00% | 142% | 131% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 141% | 126% |
| Class D | 0.00% | 0.00% | 0.00% | 142% | 135% |
| Class F (optional) | -0.09% | 0.08% | 0.10% | 142% | 129% |
| TGM | -0.18% | -0.02% | -0.13% | 142% | 127% |

# Conclusions

This contribution proposes that the intra prediction mode for BDPCM is assigned with either horizontal or vertical mode (depending on intra_bdpcm_dir_flag) for reference of the following CBs. Therefore, for generating MPMs with the neighbouring CB coded with BDPCM or using chroma DM with the corresponding luma CB coded with BDPCM, the intra prediction mode for BDPCM is set to the real intra prediction mode for BDPCM. It is reported that, compared to VTM5.0 IBC, the proposed method results in -0.18% and -0.18% luma BD-rates in AI and RA condition for TGM sequences, respectively. In addition, a change for the VVC draft specification is proposed to use transform_not_skip_flag instead of transform_skip_flag as to achieve truncated unary binarization for (transform_not_skip_flag, tu_mts_idx).

# References

- M. Karczewicz and M. Coban, “CE8-related: Quantized residual BDPCM,” Document of Joint Video Experts Team, JVET-N0413.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, and T. Wiegand, “Non-CE8: Unified transform type signalling and residual coding for transform skip,” Document of Joint Video Experts Team, JVET-M0464.

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team, JVET-N1001.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).