| Joint Video Experts Team (JVET)of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0298-r1 |
| --- | --- |

| Title: | AHG15: Chroma Quantization QpC Parameter Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jie Zhao Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | seethal.paluri@lge.com jie.zhao@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

This document proposes the inclusion of user defined Chroma Quantization (QpC) in the VVC Specification Text. A flag in the Sequence Parameter Set (SPS) is introduced to indicate if the default table in the specification text is to be used or if the contents of the table should be derived from the information signaled in the SPS. In the current working draft of VVC, the QpC values are derived using a default table given in the specification text. It is asserted that this design enables functionality to accommodate user defined chroma quantization to benefit different content. Additionally, two options are presented in the proposal. Option 1 asserts that only 1 user defined table is sufficient while Option 2 provides added flexibility to have separate user defined tables for Cb and Cr components

# Introduction

In HEVC and current VVC draft, chroma QP is derived from luma QP, and updated by additionally signalled chroma QP offsets [1]. The chroma quantization parameter QpC table exists in the VVC specification text. The table corresponding to the Specification of QpC as a function of qPi for ChromaArrayType equal to 1 is shown below in Table 1.

Table 1 – Specification of QpC as a function of qPi for ChromaArrayType equal to 1

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

Here, the qPi is an index.

# Proposal

Option 1: Cb, and Cr share the same user defined chroma quantization table.

It is proposed to add functionality for signaling the Chroma Quantization, QpC as a function of qPi index using the syntax and semantics described below in Table 2.

Table 2 - shows parameters from QpC_data().

| QpC_data( ) { | Descriptor |
| --- | --- |
| qPi_min_idx_minus1 | ue(v) |
| qPi_delta_max_idx | ue(v) |
| for ( i = qPi_min_idx; i <= qPiMaxIdx; i++ ) |  |
| QpC_qPi_flag[ i ] | u(1) |
| } |  |

7.x.x Chroma Quantization data semantics

qPi_min_idx_minus1 plus 1 specifies the minimum qPi index used in the chroma quantization.

The variable qPi_min_idx is set equal to qPi_min_idx_minus1 + 1. The value of qPi_min_idx shall be in the range of 1 to 69.

qPi_delta_max_idx specifies the delta value between the qPi_min_idx and the maximum qPi index used for the chroma QpC derivation. The value of qPiMaxIdx shall be greater than or equal to qPi_min_idx. The value of qPi_delta_max_idx shall be in the range of 0 to 69, inclusive. The maximum index qPiMaxIdx used in QpC derivation is derived as follows:

qPiMaxIdx = qPi_min_idx  +  qPi_delta_max_idx

The value of qPiMaxIdx shall be greater than or equal to qPi_min_idx.

QpC_qPi_flag[i] equal to 1 specifies that QpC value is to be incremented by 1. QpC_qPi_flag[i] equal to 0 specifies that the QpC value is not incremented.

The variable QpCIdx[ qPi ] for qPi with qPi = 0 … 69, is derived as follows:

- For qPi < qPi_min_idx, QpCIdx[qPi ] is set equal to qPi.

- For qPi = qPi_min_idx … qPiMaxIdx, the following applies:

QpCIdx[ qPi ] = QpC_qPi_flag[ qPi ] + QpCIdx[ qPi -1]

- For qPi > qPiMaxIdx

QpCIdx[ qPi ] = qPi – (qPiMaxIdx - QpCIdx[ qPiMaxIdx ])

The value of QpC is derived as QpCIdx[qPi].

Table 3 - shows an excerpt from the PPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| Qpc_data_default_flag | u(1) |
| if(!QpC_data_default_flag) |  |
| QpC_data( ) |  |
| … |  |

Where the semantics may be described as:

Qpc_data_default_flag equal to 0 specifies that the user defined mode for the derivation of the quantization parameters is to be used. Qpc_data_default_flag equal to 1 specifies that the default Table 8-15 as specified in clause 8.7.1 is used to derive the chroma quantization parameter. If QpC_data_default_flag is not present, it is inferred to be equal to 1.

### 8.7.1 Derivation process for quantization parameters

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left luma sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed.

In this process, the luma quantization parameter Qp′Y and the chroma quantization parameters Qp′Cb and Qp′Cr are derived.

The luma location ( xQg, yQg ), specifies the top-left luma sample of the current quantization group relative to the top left luma sample of the current picture. The horizontal and vertical positions xQg and yQg are set equal to CuQgTopLeftX and CuQgTopLeftY, respectively.

NOTE – : The current quantization group is a rectangluar region inside a coding tree block that shares the same qPY_PRED. Its width and height are equal to the width and height of the coding tree node of which the top-left luma sample position is assigned to the variables CuQgTopLeftX and CuQgTopLeftY.

When treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the predicted luma quantization parameter qPY_PRED is derived by the following ordered steps:

- The variable qPY_PREV is derived as follows:

–	If one or more of the following conditions are true, qPY_PREV is set equal to SliceQpY:

–	The current quantization group is the first quantization group in a slice.

–	The current quantization group is the first quantization group in a brick.

–	Otherwise, qPY_PREV is set

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg − 1, yQg ) as inputs, and the output is assigned to availableA. The variable qPY_A is derived as follows:

–	If one or more of the following conditions are true, qPY_A is set equal to qPY_PREV:

–	availableA is equal to FALSE.

–	the CTB address ctbAddrA of the CTB containing the luma coding block covering the luma location ( xQg − 1, yQg ) is not equal to CtbAddrInBs, where ctbAddrA is derived as follows:

xTmp = ( xQg − 1 )  >>  MinTbLog2SizeY
yTmp = yQg  >>  MinTbLog2SizeY
minTbAddrA = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrA = minTbAddrA  >>  ( 2  * ( CtbLog2SizeY − MinTbLog2SizeY ) )

–	Otherwise, qPY_A is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg − 1, yQg ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg, yQg − 1 ) as inputs, and the output is assigned to availableB. The variable qPY_B is derived as follows:

–	If one or more of the following conditions are true, qPY_B is set equal to qPY_PREV:

–	availableB is equal to FALSE.

–	the CTB address ctbAddrB of the CTB containing the luma coding block covering the luma location ( xQg,  yQg − 1 ) is not equal to CtbAddrInBs, where ctbAddrB is derived as follows:

xTmp = xQg  >>  MinTbLog2SizeY
yTmp = ( yQg − 1 )  >>  MinTbLog2SizeY
minTbAddrB = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrB = minTbAddrB  >>  ( 2 * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-922)

–	Otherwise, qPY_B is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ).

- The predicted luma quantization parameter qPY_PRED is derived as follows:

- If all the following conditions are true, then qPY_PRED is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ):

- availableB is equal to TRUE.

- the current quantization group is the first quantization group in a CTB row within a brick

- Otherwise, qPY_PRED is derived as follows:

qPY_PRED = ( qPY_A + qPY_B + 1 )  >>  1

The variable QpY is derived as follows:

QpY = ( ( qPY_PRED + CuQpDeltaVal + 64 + 2 * QpBdOffsetY )%( 64 + QpBdOffsetY ) ) − QpBdOffsetY

The luma quantization parameter Qp′Y is derived as follows:

Qp′Y = QpY + QpBdOffsetY

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma coding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2 ).

–	The variables qPCb, qPCr and qPCbCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 69, QpY + pps_cb_qp_offset + slice_cb_qp_offset )

qPiCr = Clip3( −QpBdOffsetC, 69, QpY + pps_cr_qp_offset + slice_cr_qp_offset )

qPiCbCr = Clip3( −QpBdOffsetC, 69, QpY + pps_joint_cbcr_qp_offset + slice_joint_cbcr_qp_offset )

–		If ChromaArrayType is equal to 1, and Qpc_data_default_flag is equal to FALSE, the variables qPCb, qPCr and qPCbCr are set equal to the value of QpC as specified in clause 7.x.x  Table 8-15 based on the index qPi equal to qPiCb, qPiCr and qPiCbCr, respectively.

–		If ChromaArrayType is equal to 1, and Qpc_data_default_flag is equal to TRUE, the variables qPCb, qPCr and qPCbCr are specified in Table 8-15 based on the index qPi equal to qPiCb, qPiCr and qPiCbCr, respectively.

–	Otherwise, the variables qPCb, qPCr and qPCbCr are set equal to Min( qPi, 63 ), based on the index qPi equal to qPiCb, qPiCr and qPiCbCr, respectively.

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, and joint Cb-Cr coding Qp′CbCr are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC

Qp′Cr = qPCr + QpBdOffsetC

Qp′CbCr = qPCbCr + QpBdOffsetC

Table 8-15 – Specification of QpC as a function of qPi for ChromaArrayType equal to 1

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

# Option 2: Have separate tables for Cb and Cr

This option presents the syntax where different chroma Qp tables can be signaled.

Table 4 - shows parameters from QpC_data().

| QpC_data( ) { | Descriptor |
| --- | --- |
| is_separate_chroma_table | u(1) |
| for ( i = 0; i <is_separate_chroma_table ? 2 : 1; i++){ |  |
| qPi_min_idx_minus1[ i ] | ue(v) |
| qPi_delta_max_idx [ i ] | ue(v) |
| for ( j = qPi_min_idx[i]; j <= qPiMaxIdx[i]; j++ ) |  |
| QpC_qPi_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |

7.x.x Chroma Quantization data semantics

is_separate_chroma_table equal to 1 specifies that a separate chroma quantization tables are signaled for cb, and cr separately. is_separate_chroma_table equal to 0 specifies that a single table is specified for cb, cr and joint cbcr chroma components.

qPi_min_idx_minus1[i] plus 1 specifies the minimum qPi index of the ith chorma component used in the chroma quantization. The value of qPi_min_idx_minus1[i] shall be in the range of 1 to 69.

The variable qPi_min_idx[i] is set equal to qPi_min_idx_minus1[i] + 1

qPi_delta_max_idx[i] specifies the delta value between the qPi_min_idx[i] and the maximum qPi index used for the chroma QpC derivation. The value of qPiMaxIdx[i] shall be greater than or equal to qPi_min_idx[i]. The value of qPi_delta_max_idx[i] shall be in the range of 0 to 69, inclusive. The maximum index qPiMaxIdx[i] used in QpC derivation is derived as follows:

qPiMaxIdx[i] = qPi_min_idx[i]  + qPi_delta_max_idx_minus1[i] + 1

The value of qPiMaxIdx[i] shall be greater than or equal to qPi_min_idx_minus1[i].

QpC_qPi_flag[i][j] equal to 1 specifies that the jth QpC value of the ith chroma component is to be incremented by 1. QpC_qPi_flag[i][j] equal to 0 specifies that the jth QpC value of the ith chroma component is not incremented.

For i =0; i< is_separate_chroma_table ? 2 : 1, i++

The variable QpCIdx[i][ qPi ] for qPi with qPi = 0 … 69, is derived as follows:

- For qPi < qPi_min_idx[i], QpCIdx[i][qPi ] is set equal to qPi.

- For qPi = qPi_min_idx[i] … qPiMaxIdx[i], the following applies:

QpCIdx[i][ qPi ] = QpC_qPi_flag[i][ qPi ] + QpCIdx[i][ qPi -1]

- For qPi > qPiMaxIdx

QpCIdx[i][ qPi ] = qPi – (qPiMaxIdx[i] - QpCIdx[i][ qPiMaxIdx ])

The value of QpC of the ith chroma component is derived as QpCIdx[i][qPi].

Table 5 - shows an excerpt from the PPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| Qpc_data_default_flag | u(1) |
| if(!QpC_data_default_flag) |  |
| QpC_data( ) |  |
| … |  |

Where the semantics may be described as:

Qpc_data_default_flag equal to 0 specifies that the user defined mode for the derivation of the quantization parameters is to be used. Qpc_data_default_flag equal to 1 specifies that the default Table 8-15 as specified in clause 8.7.1 is used to derive the chroma quantization parameter. If QpC_data_default_flag is not present, it is inferred to be equal to 1.

### 8.7.1 Derivation process for quantization parameters

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left luma sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed.

In this process, the luma quantization parameter Qp′Y and the chroma quantization parameters Qp′Cb and Qp′Cr are derived.

The luma location ( xQg, yQg ), specifies the top-left luma sample of the current quantization group relative to the top left luma sample of the current picture. The horizontal and vertical positions xQg and yQg are set equal to CuQgTopLeftX and CuQgTopLeftY, respectively.

NOTE – : The current quantization group is a rectangluar region inside a coding tree block that shares the same qPY_PRED. Its width and height are equal to the width and height of the coding tree node of which the top-left luma sample position is assigned to the variables CuQgTopLeftX and CuQgTopLeftY.

When treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the predicted luma quantization parameter qPY_PRED is derived by the following ordered steps:

- The variable qPY_PREV is derived as follows:

–	If one or more of the following conditions are true, qPY_PREV is set equal to SliceQpY:

–	The current quantization group is the first quantization group in a slice.

–	The current quantization group is the first quantization group in a brick.

–	Otherwise, qPY_PREV is set

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg − 1, yQg ) as inputs, and the output is assigned to availableA. The variable qPY_A is derived as follows:

–	If one or more of the following conditions are true, qPY_A is set equal to qPY_PREV:

–	availableA is equal to FALSE.

–	the CTB address ctbAddrA of the CTB containing the luma coding block covering the luma location ( xQg − 1, yQg ) is not equal to CtbAddrInBs, where ctbAddrA is derived as follows:

xTmp = ( xQg − 1 )  >>  MinTbLog2SizeY
yTmp = yQg  >>  MinTbLog2SizeY
minTbAddrA = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrA = minTbAddrA  >>  ( 2  * ( CtbLog2SizeY − MinTbLog2SizeY ) )

–	Otherwise, qPY_A is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg − 1, yQg ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg, yQg − 1 ) as inputs, and the output is assigned to availableB. The variable qPY_B is derived as follows:

–	If one or more of the following conditions are true, qPY_B is set equal to qPY_PREV:

–	availableB is equal to FALSE.

–	the CTB address ctbAddrB of the CTB containing the luma coding block covering the luma location ( xQg,  yQg − 1 ) is not equal to CtbAddrInBs, where ctbAddrB is derived as follows:

xTmp = xQg  >>  MinTbLog2SizeY
yTmp = ( yQg − 1 )  >>  MinTbLog2SizeY
minTbAddrB = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrB = minTbAddrB  >>  ( 2 * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-922)

–	Otherwise, qPY_B is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ).

- The predicted luma quantization parameter qPY_PRED is derived as follows:

- If all the following conditions are true, then qPY_PRED is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ):

- availableB is equal to TRUE.

- the current quantization group is the first quantization group in a CTB row within a brick

- Otherwise, qPY_PRED is derived as follows:

qPY_PRED = ( qPY_A + qPY_B + 1 )  >>  1

The variable QpY is derived as follows:

QpY = ( ( qPY_PRED + CuQpDeltaVal + 64 + 2 * QpBdOffsetY )%( 64 + QpBdOffsetY ) ) − QpBdOffsetY

The luma quantization parameter Qp′Y is derived as follows:

Qp′Y = QpY + QpBdOffsetY

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma coding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2 ).

–	The variables qPCb, qPCr and qPCbCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 69, QpY + pps_cb_qp_offset + slice_cb_qp_offset )

qPiCr = Clip3( −QpBdOffsetC, 69, QpY + pps_cr_qp_offset + slice_cr_qp_offset )

qPiCbCr = Clip3( −QpBdOffsetC, 69, QpY + pps_joint_cbcr_qp_offset + slice_joint_cbcr_qp_offset )

–		If ChromaArrayType is equal to 1, and Qpc_data_default_flag is equal to FALSE, the variables qPCb, qPCr and qPCbCr are set equal to the value of QpC as specified in clause 7.x.x  Table 8-15 based on the index qPi[i] equal to qPiCb, qPiCr and qPiCbCr, respectively.

–		If ChromaArrayType is equal to 1, and Qpc_data_default_flag is equal to TRUE, the variables qPCb, qPCr and qPCbCr are specified in Table 8-15 based on the index qPi equal to qPiCb, qPiCr and qPiCbCr, respectively.

–	Otherwise, the variables qPCb, qPCr and qPCbCr are set equal to Min( qPi, 63 ), based on the index qPi equal to qPiCb, qPiCr and qPiCbCr, respectively.

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, and joint Cb-Cr coding Qp′CbCr are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC

Qp′Cr = qPCr + QpBdOffsetC

Qp′CbCr = qPCbCr + QpBdOffsetC

Table 8-15 – Specification of QpC as a function of qPi for ChromaArrayType equal to 1

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” document JVET-N1001, 14th Meeting: Geneva, CH, 19-27 Mar. 2019

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).