| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0300_r1 |
| --- | --- |

| Title: | Non-CE5: Clean-up of ALF Syntax Parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | seethal.paluri@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

This document proposes clean-ups of two Adaptive Loop Filter (ALF) Parameters, namely, alf_luma_fixed_filter_idx and alf_luma_fixed_filter_set_idx. Currently, both parameters use truncated binary (tb) binarizations. However, these two syntax elements have a maximum value of 15. Hence, there is no coding benefit obtained from TB binarization compared to fixed length coding (FLC). Using Truncated Binary binarization introduces more complexity than fixed length coding. Additionally FLC is especially useful for representing numbers when they are a power of two.

# Introduction

Truncated Binary Coding is a form of binary coding. It is parameterized by an alphabet with the total size of number n. It is very useful when n is not a power of two. First, k =floor(log2(n)) is determined such that 2k < n< 2k+1 and let u = 2k+1 –n. The first u symbol codewords are assigned a length k bits and the remaining n-u symbol are assigned the length k+1 bits. Fixed length coding is a coding technique where a fixed number source symbols are encoded into a fixed number of output symbols. It has the advantage of being simpler and very useful especially when the total size n is a power of two, as there is no difference between fixed length code and truncated binary code.

# Proposal

We propose to simplify the signaling of the syntax elements alf_luma_fixed_filter_set_idx  and alf_luma_fixed_filter_idx by using Fixed length coding. Since the maximum value is 15 for both the parameters, it can be achieved using Fixed Length coding i.e., u(4). The Tables below demonstrate the proposed changes.

Table 1 Shows the modified alf_data( ) syntax elements.

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … | u(1) |
| alf_luma_use_fixed_filter_flag | u(1) |
| if( alf_luma_use_fixed_filter_flag ) { |  |
| alf_luma_fixed_filter_set_idx | tb(v) u(4) |
| alf_luma_fixed_filter_pred_present_flag | u(1) |
| ….. |  |

alf_luma_fixed_filter_set_idx specifies a fixed filter set index. The value of alf_luma_fixed_filter_set_idx shall be in a range of 0 to 15, inclusive. The maximum value maxVal of the truncated binary binarization tb(v) is set equal to 15.

Table 2 Shows the modified coding_tree_unit( ) syntax elements.

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| … |  |
| if( alf_use_aps_flag ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| else |  |
| alf_luma_fixed_filter_idx | ae(v) u(4) |
| … |  |

Table 3 Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| slice_data( ) | end_of_brick_one_bit | FL | cMax = 1 |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | FL | cMax = 1 |
|  | alf_ctb_use_first_aps_flag | FL | cMax = 1 |
|  | alf_use_aps_flag | FL | cMax = 1 |
|  | alf_luma_fixed_filter_idx | TB FL | cMax = 15 |
|  | alf_luma_prev_filter_idx_minus1 | TB | cMax = num_alf_aps_ids − 2 |

Table 4 Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_brick_one_bit | terminate | na | na | na | na | na |
| alf_ctb_flag[ ][ ][ ] | 0..8 (clause 9.5.4.2.2) | na | na | na | na | na |
| alf_ctb_use_first_aps_flag | 0 | na | na | na | na | na |
| alf_use_aps_flag | 0 | na | na | na | na | na |
| alf_luma_fixed_filter_idx | bypass | bypass | bypass | bypass | bypass na | Bypass na |
| alf_luma_prev_filter_idx_minus1 | bypass | bypass | bypass | bypass | bypass | bypass |

# References

[1] G. J. Sullivan, J.-R. Ohm, “Meeting Report of the 14th JVET Meeting,” document JVET-N1000, 14th JVET meeting: Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).