| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0301_r1 |
| --- | --- |

| Title: | Non-CE5: Simplification of ALF Clipping Coefficients in the APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | seethal.paluri@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

In the current working draft of the VVC standard, the ALF syntax elements alf_luma_clip_idx and alf_chroma_clip_idx are coded using unsigned integer with the kth order Exponential Golob (EG) code. The kth order for each clipping index corresponding to each filter coefficient is determined using a multi-step process. It is further noted that the range of clip indices are from 0 to 3, inclusive. It is asserted that the complexity introduced by using the unsigned exponential golomb kth – ordering for the small range of clip indices used is unnecessary. Therefore, this proposal suggests to use fixed length coding with two bits, thereby simplifying the existing structure by removing the associated signaling of the minimum EG order and its corresponding increase flags.

# Introduction

ALF parameters, alf_luma_clip_idx, and alf_chroma_clip_idx are coded using kth order EG codes where the kth order is derived based on the signaled information i.e., alf_luma_clip_min_eg_order_minus1, alf_luma_clip_eg_order_increase_flag; and alf_chroma_clip_min_eg_order_minus1 and alf_chroma_clip_eg_order_increase_flag.  Table 1 below shows an excerpt the syntax elements of the alf_data().

Table 1 Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The order kClipY[ i ] of the exp-Golomb code used to decode the values of alf_luma_clip_idx[ sfIdx ][ j ] is derived as follows:

kClipY[ i ] = ( i = = 0 ? alf_luma_clip_min_eg_order_minus1 + 1 : kClipY[ i − 1 ] ) +
alf_luma_clip_eg_order_increase_flag[ i ]

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipY[ golombOrderIdxY[ j ] ]

A similar process is used outlined for the derivation of the chroma clip EG order.

# Proposal

The complexity of the clipping index values for both luma and chroma are within the range from 0 to 3 inclusive. As the clipping indices are also coded using EG-kth order, this can also be simplified by using fixed length coding with 2 bits i.e., u(2). Since the range of the clipping indices do not vary between luma and chroma components, the later can also be coded using fixed length codes. This is beneficial because, the number of bits used to signal the clipping parameters can be reduced. The suggested syntax changes are shown below in Table 2.

Table 2 The proposed modifications of alf_data ( )

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v) u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v) u(2) |
| } |  |
| } |  |
| } |  |

alf_luma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma clipping index signalling. The value of alf_luma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is incremented by 1. alf_luma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is not incremented by 1.

The order kClipY[ i ] of the exp-Golomb code used to decode the values of alf_luma_clip_idx[ sfIdx ][ j ] is derived as follows:

kClipY[ i ] = ( i = = 0  ?  alf_luma_clip_min_eg_order_minus1 + 1  :  kClipY[ i − 1 ] ) +	
	alf_luma_clip_eg_order_increase_flag[ i ]

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_clip_idx[ sfIdx ][ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipY[ golombOrderIdxY[ j ] ]

The variable filterClips[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1, j = 0..11 is initialized as follows:

filterClips[ sfIdx ][ j ] = Round( 2( BitDepthY * ( 4 − alf_luma_clip_idx[ sfIdx ][ j ] ) / 4 ) )

The luma filter clipping values AlfClipL[ adaptation_parameter_set_id ] with elements AlfClipL[ adaptation_parameter_set_id ][ filtIdx ][ j ], with filtIdx = 0..NumAlfFilters − 1 and j = 0..11 are derived as follows:

AlfClipL[ adaptation_parameter_set_id ][ filtIdx ][ j ] = filterClips[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ]

alf_chroma_clip_flag equal to 0 specifies that linear adaptive loop filtering is applied on chroma components; alf_chroma_clip_flag equal to 1 specifies that non-linear adaptive loop filtering is applied on chroma component. When not present, alf_chroma_clip_flag is inferred to be equal to 0.

alf_chroma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma clipping index signalling. The value of alf_chroma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is incremented by 1. alf_chroma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is not incremented by 1.

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_clip_idx[ j ] is derived as follows:

kClipC[ i ] = ( i = = 0  ?  alf_chroma_clip_min_eg_order_minus1 + 1  :  kClipC[ i − 1 ] ) +	
	alf_chroma_clip_eg_order_increase_flag[ i ]

alf_chroma_clip_idx[ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the chroma filter. When alf_chroma_clip_idx[ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ j ] with j = 0..5 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k =  kClipC[ golombOrderIdxC[ j ] ]

The chroma filter clipping values AlfClipC[ adaptation_parameter_set_id ] with elements AlfClipC[ adaptation_parameter_set_id ][ j ], with j = 0..5 are derived as follows:

AlfClipC[ adaptation_parameter_set_id ][ j ] = Round( 2( BitDepthC − 8 ) * 2( 8 * ( 3 − alf_chroma_clip_idx[ j ] ) / 3 ) )

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, Document of Joint Video Experts Team (JVET), JVET-N1001, 14th Meeting: Geneva, CH, 19-27 Mar. 2019

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).