| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0302_r1 |
| --- | --- |

| Title: | Non-CE5: Simplification of ALF Coefficients in the APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | seethal.paluri@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

The current working draft of the VVC standard uses, the unsigned integer with the kth order Exponential Golob (EG) code to signal the ALF syntax elements (i) alf_luma_coeff_delta_abs and, (ii) alf_chroma_coeff_abs. To determine the kth EG order a multi-step process is carried out. Therefore, this proposal suggests simplifying the existing ALF structure by removing the associated signaling of the minimum EG order and its corresponding increase flags for the derivation of both the luma and chroma filter coefficients and instead use a pre-fixed EG order. The results obtained show the Luma BD-rate  performance as follows: (0.00%, 0.00%, 0.02% and -0.03%)for AI, RA, LDB and LDP respectively.

# Introduction

ALF parameters, alf_luma_coeff_delta_abs and alf_chroma_coeff_abs are coded using kth order EG codes where the kth order is derived based on the signaled information i.e., alf_luma_min_eg_order_minus1, alf_luma_eg_order_increase_flag; and alf_chroma_min_eg_order_minus1, alf_chroma_eg_order_increase_flag.  Table 1 below shows excerpts of the syntax elements of the alf_data().

Table 1 Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … |  |
| alf_luma_coeff_delta_flag | u(1) |
| if( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| alf_luma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if( alf_luma_coeff_delta_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_delta_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_delta_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_delta_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_clip_flag | u(1) |
| … |  |
| alf_chroma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] | u(1) |
| } |  |
| … |  |

The kth order is derived from the parsing of the elements listed above. First a minimum EG order is parsed, then a flag indicating if it should be increased or not is parsed. The variable expGoOrderY[i] is first determined as shown below.

expGoOrderY[ i ] = ( i = = 0 ? alf_luma_min_eg_order_minus1 + 1 : expGoOrderY[ i − 1 ] ) +
	alf_luma_eg_order_increase_flag[ i ]

The order k of the exp-Golomb binarization uek(v) is derived as follows:

golombOrderIdxY[ ] = { 0, 0, 1, 0, 0, 1, 2, 1, 0, 0, 1, 2 }

k = expGoOrderY[ golombOrderIdxY[ j ] ]

# Proposal

In this proposal, the complexity introduced in using the kth order EG coding is simplified. After analysing the distribution of the kth order for the Luma and Chroma coefficients and the clipping coefficients, it is observeed from statistics that the each coefficient has a frequently occurring order as shown in Table 2 below.

Table 2 The kth order for the coefficient.

| Coefficient Index | kth order |
| --- | --- |
| C0 | 2 |
| C1 | 2 |
| C2 | 2 |
| C3 | 2 |
| C4 | 2 |
| C5 | 3 |
| C6 | 3 |
| C7 | 3 |
| C8 | 2 |
| C9 | 2 |
| C10 | 2 |
| C11 | 3 |

Figure 1 The coefficient index for the 7x7.

It is suggested to pre-define the kth order and thereby reduce the complexity of the signaling. Similarly, for the Chroma case the predominant kth order is 3. Therefore, it is suggested to fix the order to 3, i.e., k = 3. The proposed changes to the syntax and semantics are shown in the Table 3 below.

Table 3 Shows the proposed modifications of alf_data ( )

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … |  |
| if( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| alf_luma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if( alf_luma_coeff_delta_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) {0 to 11 |  |
| alf_luma_coeff_delta_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_delta_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_delta_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_clip_flag | u(1) |
| alf_chroma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] | u(1) |
| } |  |
| … |  |

alf_luma_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma filter coefficient signalling. The value of alf_luma_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma filter coefficient signalling is incremented by 1. alf_luma_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma filter coefficient signalling is not incremented by 1.

The order expGoOrderY[ i ] of the exp-Golomb code used to decode the values of alf_luma_coeff_delta_abs[ sfIdx ][ j ] is derived as follows:

expGoOrderY[ i ] = ( i = = 0  ?  alf_luma_min_eg_order_minus1 + 1  :  expGoOrderY[ i − 1 ] ) +	
	alf_luma_eg_order_increase_flag[ i ]

alf_luma_coeff_flag[ sfIdx ] equal 1 specifies that the coefficients of the luma filter indicated by sfIdx are signalled. alf_luma_coeff_flag[ sfIdx ] equal to 0 specifies that all filter coefficients of the luma filter indicated by sfIdx are set equal to 0. When not present, alf_luma_coeff_flag[ sfIdx ] is set equal to 1.

alf_luma_coeff_delta_abs[ sfIdx ][ j ] specifies the absolute value of the j-th coefficient delta of the signalled luma filter indicated by sfIdx. When alf_luma_coeff_delta_abs[ sfIdx ][ j ] is not present, it is inferred to be equal 0.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

golombOrderIdxY[ ] = { 0, 0, 1, 0, 0, 1, 2, 1, 0, 0, 1, 2 }

expGoOrderY = { 2, 2, 2, 2, 2, 3, 3, 3, 2, 2, 2, 3 }

k = expGoOrderY[ golombOrderIdxY[ j ] ]

….

alf_chroma_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma filter coefficient signalling. The value of alf_chroma_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma filter coefficient signalling is incremented by 1. alf_chroma_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma filter coefficient signalling is not incremented by 1

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_coeff_abs[ j ] is derived as follows:

expGoOrderC[ i ] = alf_chroma_min_eg_order_minus1 + 1+ alf_chroma_eg_order_increase_flag[ i ]

alf_chroma_coeff_abs[ j ] specifies the absolute value of the j-th chroma filter coefficient. When alf_chroma_coeff_abs[ j ] is not present, it is inferred to be equal 0. It is a requirement of bitstream conformance that the values of alf_chroma_coeff_abs[ j ] shall be in the range of 0 to 27 − 1, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

golombOrderIdxC[ ] = { 0, 0, 1, 0, 0, 1 }

k = 3 expGoOrderC[ golombOrderIdxC[ j ] ]

alf_chroma_coeff_sign[ j ] specifies the sign of the j-th chroma filter coefficient as follows:

- If alf_chroma_coeff_sign[ j ] is equal to 0, the corresponding chroma filter coefficient has a positive value.

- Otherwise (alf_chroma_coeff_sign[ j ] is equal to 1), the corresponding chroma filter coefficient has a negative value.

When alf_chroma_coeff_sign[ j ] is not present, it is inferred to be equal to 0.

The chroma filter coefficients AlfCoeffC[ adaptation_parameter_set_id ] with elements AlfCoeffC[ adaptation_parameter_set_id ][ j ], with j = 0..5 are derived as follows:

AlfCoeffC[ adaptation_parameter_set_id ][ j ] = alf_chroma_coeff_abs[ j ] * 	
	( 1 − 2 * alf_chroma_coeff_sign[ j ] )

It is a requirement of bitstream conformance that the values of AlfCoeffC[ adaptation_parameter_set_id ][ j ] with j = 0..5 shall be in the range of −27 − 1 to 27 − 1, inclusive.

# Results

This section discusses the results obtained over VTM 5.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 105% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class E | 0.00% | -0.01% | -0.02% | 99% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class D | 0.00% | 0.00% | -0.04% | 100% | 89% |
| Class F | 0.00% | -0.01% | 0.00% | 102% | 107% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.00% | 102% | 103% |
| Class A2 | 0.01% | 0.01% | 0.02% | 101% | 102% |
| Class B | 0.00% | 0.01% | -0.04% | 101% | 104% |
| Class C | -0.03% | 0.03% | -0.03% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.02% | 101% | 103% |
| Class D | 0.01% | 0.14% | 0.17% | 100% | 102% |
| Class F | -0.01% | 0.02% | -0.01% | 101% | 103% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.36% | -0.21% | 102% | 105% |
| Class C | 0.02% | -0.14% | -0.20% | 101% | 110% |
| Class E | 0.08% | 0.11% | -0.18% | 102% | 108% |
| Overall | 0.02% | 0.13% | -0.20% | 102% | 107% |
| Class D | 0.05% | 0.16% | 0.11% | 100% | 105% |
| Class F | 0.12% | -0.23% | -0.66% | 100% | 105% |

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.14% | 0.51% | 99% | 99% |
| Class C | -0.01% | -0.21% | -0.07% | 100% | 100% |
| Class E | -0.09% | 0.66% | -0.35% | 100% | 101% |
| Overall | -0.03% | 0.15% | 0.10% | 100% | 100% |
| Class D | -0.02% | -0.24% | -0.63% | 100% | 99% |
| Class F | 0.11% | 0.58% | -0.07% | 99% | 100% |

# Conclusion

This proposal introduces simplification for the decoding of alf_luma_coeff_delta_abs and alf_chroma_coeff_abs by removing the signaling of the minimum EG order and its corresponding increase flags and instead using a fixed EG order.

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, Document of Joint Video Experts Team (JVET), JVET-N1001, 14th Meeting: Geneva, CH, 19-27 Mar. 2019

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).