| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0303-v2 |
| --- | --- |

| Title: | AHG 8: Adaptive Resolution Change |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peisong Chen, Tim Hellman, Brian Heng, Wade Wan, Minhua Zhou 1320 Ridder Park Drive San Jose, California 95131 United States | Email: | peisong.chen@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a method to support adaptive resolution change within a coded video sequence. It includes the signalling of adaptive resolution change in parameter sets and modifications to the current motion compensated prediction process in the VTM when there is a resolution change between a current picture and its reference pictures. These modifications are limited to motion vector scaling and subpel location derivations. There are no changes in the existing motion compensation interpolators with the asserted benefits of the proposed solution that these interpolators can be reused and does not introduce new processing blocks. The contribution also provides recommendations on constraining the interaction of certain coding tools with adaptive resolution changes as they are asserted to add additional complexity and memory bandwidth.

# Introduction

According to “Requirements for a Future Video Coding Standard” [1], “the standard shall support fast representation switching in the case of adaptive streaming services that offer multiple representations of the same content, each having different properties (e.g. spatial resolution or sample bit depth).” In real-time video communication, allowing resolution change within a coded video sequence without inserting an I picture can not only adapt the video data to dynamic channel conditions or user preference seamlessly, but also remove the beating effect caused by I pictures. A hypothetical example of adaptive resolution change is shown in Figure 1 where the current picture is predicted from reference pictures of different sizes.

This contribution proposes high level syntax to signal adaptive resolution change as well as modifications to the current motion compensated prediction process in the VTM. These modifications are limited to motion vector scaling and subpel location derivations with no changes in the existing motion compensation interpolators. This would allow the existing motion compensation interpolators to be reused and not require new processing blocks to support adaptive resolution change which would introduce additional cost.

Ref 1

Ref 0

Current

down scale

up scale

Figure 1 An example of adaptive resolution change.

# Adaptive Resolution Change Signalling

## SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| max_pic_width_in_luma_samples | ue(v) |
| max_pic_height_in_luma_samples | ue(v) |
| ... |  |
| } |  |

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

max_pic_width_in_luma_samples specifies the maximum width of decoded pictures referring to the SPS in units of luma samples. max_pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

max_pic_height_in_luma_samples specifies the maximum height of decoded pictures referring to the SPS in units of luma samples. max_pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

## PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_size_different_from_max_flag | u(1) |
| if (pic_size_different_from_max_flag) { |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| } |  |
| … |  |
| } |  |

pic_size_different_from_max_flag equal to 1 specifies that the PPS signals different picture width or picture height from the max_pic_width_in_luma_samples and max_pic_height_in_luma_sample in the referred SPS. pic_size_different_from_max_flag equal to 0 specifies that pic_width_in_luma_samples and pic_height_in_luma_sample are the same as max_pic_width_in_luma_samples and max_pic_height_in_luma_sample in the referred SPS.

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. When pic_width_in_luma_samples is not present, it is inferred to be equal to max_pic_width_in_luma_samples

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY. When pic_height_in_luma_samples is not present, it is inferred to be equal to max_pic_height_in_luma_samples.

It is a requirement of bitstream conformance that all of the following conditions are satisfied for every active reference picture whose width and height are represented by reference_pic_width_in_luma_samples and reference_pic_height_in_luma_samples:

- 2 * pic_width_in_luma_samples  >= reference_pic_width_in_luma_samples

- 2 * pic_height_in_luma_samples  >= reference_pic_height_in_luma_samples

- pic_width_in_luma_samples  <= 8 * reference_pic_width_in_luma_samples

- pic_height_in_luma_samples  <= 8 * reference_pic_height_in_luma_samples

# Reference Picture Scaling Process

When there is a resolution change within a CVS, a picture may have a different size from one or more of its reference pictures. This proposal normalizes all motion vectors to the current picture grid instead of their corresponding reference picture grids. This is asserted to be beneficial to keep the design consistent and make resolution changes transparent to the motion vector prediction process. Otherwise, neighboring motion vectors pointing to reference pictures with different sizes cannot be used directly for spatial motion vector prediction due to the different scale.

When a resolution change happens, both the motion vectors and reference blocks have to be scaled while doing motion compensated prediction. The scaling range is limited to [1/8, 2], i.e. the upscaling is limited to 1:8 and downscaling is limited to 2:1. Note that upscaling refers to the case where the reference picture is smaller than the current picture, while downscaling refers to the case where the reference picture is larger than the current picture. In the following sections, the scaling process is described in more detail.

## Luma Block

The scaling factors and their fixed-point representations are defined as

hori_scale_fp= widthref≪14+(widthcur2)widthcur,	( 1 )

vert_scale_fp= heightref≪14+(heightcur2)heightcur.	( 2 )

The scaling process includes two parts:

- Map the upper left corner pixel of the current block to the reference picture;

- Use the horizontal and vertical step sizes to address the reference locations of the current block’s other pixels.

If the coordinate of the upper left corner pixel of the current block is (x, y), the subpel location (x', y') in the reference picture pointed to by a motion vector (mvX, mvY) in units of 1/16th pel is specified as follows:

- The horizontal location in the reference picture is

x'=(x≪4+mvX)∙hori_scale_fp,	( 3 )

and x' is further scaled down to only keep 10 fractional bits

x'= Signx'∙Abs(x')+ 1≪7≫8.	 ( 4 )

- Similarly, the vertical location in the reference picture is

y'=( y≪4+mvY)∙vert_scale_fp,                                                     ( 5 )

and y' is further scaled down to

y'= Signy'∙Abs(y')+ 1≪7≫8.                                            ( 6 )

At this point, the reference location of the upper left corner pixel of the current block is at (x', y'). The other reference subpel/pel locations are calculated relative to (x', y') with horizontal and vertical step sizes. Those step sizes are derived with 1/1024-pel accuracy from the above horizontal and vertical scaling factors as follows:

x_step = ( hori_scale_fp + 8) >> 4,                                                        ( 7 )

y_step = ( vert_scale_fp + 8) >> 4.                                                        ( 8 )

As an example, if a pixel in the current block is i-column and j-row away from the upper left corner pixel, its corresponding reference pixel’s horizontal and vertical coordinates are derived by

x'i=x'+ i * x_step,                                                                ( 9 )

y'j=y'+ j * y_step.                                                              ( 10 )

In subpel interpolation, x'i and y'j have to be broken up into full-pel parts and fractional-pel parts:

- The full-pel parts for addressing reference block are equal to

x'i+32≫10,                                                                   ( 11 )

y'j+32≫10.                                                                   ( 12 )

- The fractional-pel parts used to select interpolation filters are equal to

∆x=x'i+32≫6&15,                                                    ( 13 )

∆y=y'j+32≫6&15.                                                   ( 14 )

Once the full-pel and fractional-pel locations within a reference picture are determined, the existing motion compensation interpolators can be used without any additional changes. The full-pel location will be used to fetch the reference block patch from the reference picture and the fractional-pel location will be used to select the proper interpolation filter.

## Chroma Block

When the chroma format is 4:2:0, chroma motion vectors have 1/32-pel accuracy. The scaling process of chroma motion vectors and chroma reference blocks is almost the same as for luma blocks except a chroma format related adjustment.

When the coordinate of the upper left corner pixel of the current chroma block is (xc, yc), the initial horizontal and vertical locations in the reference chroma picture are

xc'=(xc≪5+mvX)∙hori_scale_fp,                                         ( 15 )

yc'=( yc≪5+mvY)∙vert_scale_fp,                                         ( 16 )

where mvX and mvY are the original luma motion vector but now should be examined with 1/32-pel accuracy.

xc' and yc' are further scaled down to keep 1/1024 pel accuracy

xc'= Signxc'∙Abs(xc')+ 1≪8≫9,                                  ( 17 )

yc'= Signyc'∙Abs(yc')+ 1≪8≫9.                                  ( 18 )

Compared to the associated luma equations, the above right shift is increased by one extra bit.

The step sizes used are the same as for luma. For a chroma pixel at (i, j) relative to the upper left corner pixel, its reference pixel’s horizontal and vertical coordinates are derived by

xc'i=xc'+ i * x_step,                                                              ( 19 )

yc'j=yc'+ j * y_step.                                                             ( 20 )

In subpel interpolation, xc'i and yc'j are also broken up into full-pel parts and fractional-pel parts:

- The full-pel parts for addressing reference block are equal to

xc'i+16≫10,                                                                 ( 21 )

yc'j+16≫10.                                                           ( 22 )

- The fractional-pel parts used to select interpolation filters are equal to

∆x=xc'i+16≫5&31,                                                      ( 23 )

∆y=yc'j+16≫5&31.                                                      ( 24 )

# Interaction with Other Coding Tools

Because of the extra complexity and memory bandwidth associated with the interaction of some coding tools with reference picture scaling, it is recommended to add the following restrictions:

- When slice_temporal_mvp_enabled_flag is equal to 1, the collocated picture used for temporal motion vector prediction shall have the same size as the current picture.

- For a coding block in the current picture, if the size of the current picture is different from the size of List0 or List1 reference pictures, BDOF and DMVR shall be turned off.

- For a coding block in the current picture using affine mode, if the size of the current picture is different from the size of List0 or List1 reference picture, the existing affine fallback mode shall be triggered.

# Experimental Results

The adaptive resolution change proposed in the contribution is compared to the resolution change using I pictures. To simulate  adaptive resolution change, the initial 96 frames of 4K sequences are encoded. The first 32 frames are encoded at the original resolution, the second 32 frames are encoded at a quarter resolution and the last 32 frames are changed back to the original resolution.

Figure 2 shows the frame size and luma PSNR of DaylightRoad2 encoded at QP 37 with low delay B configuration. The bit rate jump caused by inserting the I picture at frame 33 has been completely repressed in the adaptive resolution change case. This is very significant especially when we want to use adaptive resolution change as a tool to reduce bandwidth consumption. In the opposite scenario where the picture resolution is restored to its original value at frame 65, the adaptive resolution change can also significantly mitigate the bit rate hike compared to using the I picture.

Figure 2 An example of adaptive resolution change.

The BD rate saving of ARC compared to using Intra refresh in low delay B configuration is reported in the table below.

|  |  | Y | U | V |
| --- | --- | --- | --- | --- |
| Class A1 | Tango2 | -4.88% | 6.93% | 5.62% |
|  | FoodMarket4 | -4.94% | -5.06% | -4.79% |
|  | Campfire | -0.33% | -1.85% | -2.73% |
| Class A2 | CatRobot | -13.63% | -10.88% | -9.80% |
|  | DaylightRoad2 | -8.03% | -6.86% | -11.90% |
|  | ParkRunning3 | -1.02% | -5.93% | -6.57% |

# References

- “Requirements for a Future Video Coding Standard v5”, ISO/IEC JTC1/SC29/WG11 N17074, July 2017.

# Patent rights declaration(s)

Broadcom may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).