| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0304-v1 |
| --- | --- |

| Title: | Non-CE9: Simplification of BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Kato Kiyofumi Abe Tadamasa Toma | Tel: Email: | +81-80-3083-1171 kato.yusuke003@jp.panasonic.com abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution simplified BDOF in VTM5.0 by removing 98% multiplications operations. In the proposed method, multiplication of gradient values in calculating correlation terms is replaced with the sign function. Simulation results reportedly shows 0.02% luma BD-rate loss in RA and 1% decoding time reduction.

Introduction

In the existing BDOF of VTM5, many multiplications are required to calculate the optical flow.

In the BDOF process, firstly, the gradient images are calculated using the pred images of two reference blocks. Secondly, five types of correlations are calculated per 4x4 subblock, and, the optical flow is derived. Finally, the BDOFoffset is calculated per pixel and prediction samples are generated. In these processes, the multiplication occurs when calculating correlations and BDOFoffset. Especially, calculation of five types of correlations requires a large number of multiplications (See table1).

・ Correlation (per 4x4 sub block unit)

: pred image from MC

s1=i,j∊ΩIx1+Ix0×Ix1+Ix0   (1)           I0, I1

: gradient image

s2=[i,j]∊ΩIy1+Iy0×Iy1+Iy0   (2)           Ix0, Ix1,Iy0, Iy1

: 6x6 area for each 4x4 sub block

s3=i,j∊Ω(-Ix1+Ix0×I0-I1)   (3)         Ω

s4=[i,j]∊Ω(-Iy1+Iy0×I0-I1)   (4)

s5=[i,j]∊ΩIx1+Ix0×Iy1+Iy0  (5)

vx=s3s1

vy=s4-vx×s5s2

・BDOFoffset(per sample unit)

BDOFoffset=vx×Ix0-Ix1/2 +(vy×(Iy0-Iy1))/2

From the above two steps, total number of multiplication is:

(5×6×6 + 1) × (the number of 4x4 sub block in CU) + 2×(the number of samples in CU)

Note:
If the multiplication results of overlapped samples are reused, the number of multiplication of correlation part can be 5 × (Width+2) × (Height+2) instead of 5×6×6×(the number of 4x4 sub block in CU). But it requires additional memory of 5 × (Width+2) × (Height+2). Therefore, we estimated as above.

Proposed method

Multiplication of gradient values in calculating correlation terms in BDOF of VTM5.0 is replaced with the sign function. Proposed algorithm is as follows:

・Correlation (per 4x4 sub block unit)

(6)

(7)

s1=[i,j]∊ΩabsIx1+Ix0

s2=[i,j]∊ΩabsIy1+Iy0

no multiplication

(8)

s3=[i,j]∊Ω(-signIx1+Ix0×I0-I1)

(9)

s4=[i,j]∊Ω(-signIy1+Iy0×I0-I1)

(10)

s5=[i,j]∊ΩsignIy1+Iy0×Ix1+Ix0

vx=s3s1

vy=s4-vx×s5s2

signZ= 1 (Z>0)0 (Z=0)-1 (Z<0)

・BDOFoffset(per sample unit)

BDOFoffset=vx×Ix0-Ix1/2 +(vy×(Iy0-Iy1))/2

In the proposed method, total number of multiplication is:

3 × (the number of 4 × 4 sub block in CU)+ 2 × (the number of samples in CU)

Mathematical explanation of this proposal

The following briefly shows how the proposed new correlation definitions are derived.

At first, consider the original definition of cross-correlations from (1) to (5).

Then horizontal component of optical flow is calculated as follows:

vx=s3s1= [i,j]∊Ω(-Ix1+Ix0×I0-I1)[i,j]∊ΩIx1+Ix0×Ix1+Ix0

Suppose that values absIx1+Ix0 for all pixel positions in Ω are same. Then, above formula is transformed as follows:

i,j∊ΩIx1+Ix0×I1-I0i,j∊ΩIx1+Ix0×Ix1+Ix0

= [i,j]∊ΩsignIx1+Ix0×abs(Ix1+Ix0)×I1-I0[i,j]∊Ωabs(Ix1+Ix0)×absIx1+Ix0

= abs(Ix1+Ix0)×[i,j]∊ΩsignIx1+Ix0×I1-I0abs(Ix1+Ix0)×[i,j]∊ΩabsIx1+Ix0

= abs(Ix1+Ix0)×[i,j]∊ΩsignIx1+Ix0×I1-I0abs(Ix1+Ix0)×[i,j]∊ΩabsIx1+Ix0

= [i,j]∊Ω(-signIx1+Ix0×I0-I1)[i,j]∊ΩabsIx1+Ix0

If equations (6) and (9) are used as new definitions of cross correlations, vx obtained from these new definitions matches the right side of the above equation.

This logic is also applied to the vertical component of opticalflow vy. In deriving vx and vy, CCLM like LUT is used for division process (in this proposal, 6 bit table is used).

Complexity analysis of proposed BDOF

Table1 gives a comparison of proposed BDOF’s complexity regarding multiplication operations with BDOF in VTM5.0

Table 1: Comparison of the number of multiplications of proposed BDOF with BDOF in VTM5.0

| VTM5.0 | Proposed method |
| --- | --- |
| W; H; Correlation; BDOFoffset; Total. 128; 128; 185,344; 32,768; 218,112. 128; 64; 92,672; 16,384; 109,056. 64; 128; 92,672; 16,384; 109,056. 128; 32; 46,336; 8,192; 54,528. 32; 128; 46,336; 8,192; 54,528. 128; 16; 23,168; 4,096; 27,264. 16; 128; 23,168; 4,096; 27,264. 128; 8; 11,584; 2,048; 13,632. 8; 128; 11,584; 2,048; 13,632. 64; 64; 46,336; 8,192; 54,528. 64; 32; 23,168; 4,096; 27,264. 32; 64; 23,168; 4,096; 27,264. 64; 16; 11,584; 2,048; 13,632. 16; 64; 11,584; 2,048; 13,632. 64; 8; 5,792; 1,024; 6,816. 8; 64; 5,792; 1,024; 6,816. 32; 32; 11,584; 2,048; 13,632. 32; 16; 5,792; 1,024; 6,816. 16; 32; 5,792; 1,024; 6,816. 32; 8; 2,896; 512; 3,408. 8; 32; 2,896; 512; 3,408. 16; 16; 2,896; 512; 3,408. 16; 8; 1,448; 256; 1,704. 8; 8; 724; 128; 852. | W; H; Correlation; BDOFoffset; Total. 128; 128; 3,072; 32,768; 35,840. 128; 64; 1,536; 16,384; 17,920. 64; 128; 1,536; 16,384; 17,920. 128; 32; 768; 8,192; 8,960. 32; 128; 768; 8,192; 8,960. 128; 16; 384; 4,096; 4,480. 16; 128; 384; 4,096; 4,480. 128; 8; 192; 2,048; 2,240. 8; 128; 192; 2,048; 2,240. 64; 64; 768; 8,192; 8,960. 64; 32; 384; 4,096; 4,480. 32; 64; 384; 4,096; 4,480. 64; 16; 192; 2,048; 2,240. 16; 64; 192; 2,048; 2,240. 64; 8; 96; 1,024; 1,120. 8; 64; 96; 1,024; 1,120. 32; 32; 192; 2,048; 2,240. 32; 16; 96; 1,024; 1,120. 16; 32; 96; 1,024; 1,120. 32; 8; 48; 512; 560. 8; 32; 48; 512; 560. 16; 16; 48; 512; 560. 16; 8; 24; 256; 280. 8; 8; 12; 128; 140. |

Comparing the number of multiplications in VTM5.0 with the number of multiplications in proposal, the number of correlation multiplications is reduced by 98% and the total number of multiplications in BDOF process is reduced to about 1/7.

- The number of multiplications of BDOF (VTM5.0 and proposed method) compared to the number of MC multiplications (additional information)

In order to show that the proposal method contributes to the reducing multiplication of BDOF prediction process, Table2 gives a comparison of proposed BDOF’s multiplication operations and BDOF in VTM5.0 with MC’s multiplication operations.

Table 2: Comparison of the number of multiplications of prooposed BDOF and BDOF in VTM5.0 with MC multiplications

| W | H | MC | BDOF (Total) |  |
| --- | --- | --- | --- | --- |
|  |  |  | VTM5.0 | proposal |
| 128 | 128 | 538,624 | 218,112 | 35,840 |
| 128 | 64 | 276,480 | 109,056 | 17,920 |
| 64 | 128 | 269,312 | 109,056 | 17,920 |
| 128 | 32 | 145,408 | 54,528 | 8,960 |
| 32 | 128 | 134,656 | 54,528 | 8,960 |
| 128 | 16 | 79,872 | 27,264 | 4,480 |
| 16 | 128 | 67,328 | 27,264 | 4,480 |
| 128 | 8 | 47,104 | 13,632 | 2,240 |
| 8 | 128 | 33,664 | 13,632 | 2,240 |
| 64 | 64 | 138,240 | 54,528 | 8,960 |
| 64 | 32 | 72,704 | 27,264 | 4,480 |
| 32 | 64 | 69,120 | 27,264 | 4,480 |
| 64 | 16 | 39,936 | 13,632 | 2,240 |
| 16 | 64 | 34,560 | 13,632 | 2,240 |
| 64 | 8 | 23,552 | 6,816 | 1,120 |
| 8 | 64 | 17,280 | 6,816 | 1,120 |
| 32 | 32 | 36,352 | 13,632 | 2,240 |
| 32 | 16 | 19,968 | 6,816 | 1,120 |
| 16 | 32 | 18,176 | 6,816 | 1,120 |
| 32 | 8 | 11,776 | 3,408 | 560 |
| 8 | 32 | 9,088 | 3,408 | 560 |
| 16 | 16 | 9,984 | 3,408 | 560 |
| 16 | 8 | 5,888 | 1,704 | 280 |
| 8 | 8 | 2,944 | 852 | 140 |

Comparing the number of multiplications in MC with the number of multiplications in BDOF of VTM5.0, the number of multiplications in BDOF is relatively large. Besides, the size of bit generated in the MC multiplication process is 16 (=10+6), while the sizes of bits generated in the calculation of cross correlation and bdofoffset in BDOF is 24 (=11+13) and 17 (=11+6), respectively. Both of the number of multiplications and the worst size of bit can be largely reduced by this proposed method.

Simulation results

Table3 show the simulation result of the proposed method. This simulation is conducted using VTM-5.0 under common test conditions of [1]. It is sufficient to provide the results of RA only since BDOF uses two blocks from forward and backward reference pictures. The BD-rate loss in RA is 0.02% , and the DecT is 99%.

Table 3: simulation results of proposed BDOF

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.12% | 0.01% | 100% | 100% |
| Class A2 | 0.03% | -0.04% | -0.05% | 100% | 99% |
| Class B | 0.02% | 0.00% | -0.03% | 99% | 99% |
| Class C | 0.01% | -0.02% | -0.11% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.04% | -0.05% | 99% | 99% |
| Class D | 0.09% | 0.06% | 0.03% | 100% | 97% |
| Class F | 0.00% | -0.02% | -0.03% | 100% | 100% |

Conclusion

In this contribution, a simplified BDOF is proposed to reduce the number of multiplications. About 98% multiplication operations are removed with respect to correlation calculation, and total BDOF multiplication operations are reduced to about 1/7. BD-rate loss for RA is 0.02%, which is negligible with the benefit of drastically reduced multiplications and DecT is reduced by 1%. We suggest the proposed BDOF should be adopted into the test model.

Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

- J.Li, C.-W.Kuo, C.S.Lim “CE9: Simplification of BDOF’s optical flow parameter derivation (Test 9.2.5)”, JVET-N0198, JVET 14th Meeting: Geneva, CH, 19–27 March 2019

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).