| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N1001-v8 |
| --- | --- |

| Title: | Versatile Video Coding (Draft 5) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text of video coding specification |  |  |
| Author(s) or Contact(s): | Benjamin Bross Jianle Chen Shan Liu | Email: | firstname.lastname@hhi.fraunhofer.de jianle.chen@huawei.com shanl@tencent.com |
| Source: | Editors |  |  |

_____________________________

- Abstract

This document describes the suggested specification changes on top of version 8 of JVET-N1001. The newly added parts are highlighted in yellow and the text to be removed is marked as red with strikethrough.

  - Mathematical functions

The following mathematical functions are defined:

Abs( x ) =  x;x  >=  0-x;x < 0		(5-1)

Asin( x )	the trigonometric inverse sine function, operating on an argument x that is
in the range of −1.0 to 1.0, inclusive, with an output value in the range of 
−π÷2 to π÷2, inclusive, in units of radians		(5-2)

Atan( x )	the trigonometric inverse tangent function, operating on an argument x, with
an output value in the range of −π÷2 to π÷2, inclusive, in units of radians	(5-3)

Atan2( y, x ) =  Atan yx ;x > 0Atan yx +πAtan yx -π;;x < 0  &&  y  >=  0x < 0  &&  y  <  0+π2-π2;;x = =  0  &&  y  >=  0otherwise	(5-4)

Ceil( x )	the smallest integer greater than or equal to x.	(5-5)

Clip1Y( x ) = Clip3( 0, ( 1  <<  BitDepthY ) − 1, x )		(5-6)

Clip1C( x ) = Clip3( 0, ( 1  <<  BitDepthC ) − 1, x )		(5-7)

Clip3( x, y, z ) =  x;z < xy;z > yz;otherwise		(5-8)

ClipH( o, W, x ) =   x  + o;x  < 0x -o;x  > W-1x;otherwise		(5-9)

Cos( x )	the trigonometric cosine function operating on an argument x in units of radians.	(5-10)

Floor( x )	the largest integer less than or equal to x.		(5-11)

GetCurrMsb( a, b, c, d ) =  c+d;b-a  >=  d / 2  c-d;a-b  >  d / 2c;otherwise	(5-12)

Ln( x )	the natural logarithm of x (the base-e logarithm, where e is the natural logarithm base constant 2.718 281 828...).		(5-13)

Log2( x )	the base-2 logarithm of x.		(5-14)

Log10( x ) the base-10 logarithm of x.		(5-15)

Min( x, y ) =   x;x <= yy;x > y		(5-16)

Max( x, y ) =   x;x >= yy;x < y		(5-17)

Round( x ) = Sign( x ) * Floor( Abs( x ) + 0.5 )		(5-18)

Sign( x ) = 1;x > 00;x  = =  0-1;x  <  0		(5-19)

Sin( x )	the trigonometric sine function operating on an argument x in units of radians	(5-20)

Sqrt( x ) = x		(5-21)

Swap( x, y ) = ( y, x )		(5-22)

Tan( x )	the trigonometric tangent function operating on an argument x in units of radians	(5-23)

The following mathematical division function is defined:

Division( num, denom )

{

x = Floor( Log2( denom ) )

normDiff = ( ( denom << 6 ) >> x ) & 63

x += ( normDiff  !=  0 )  ?  6 : 5

offsetctl = ( num < 0 ) ? −1 : 1

a = ( num * ( divSigTable[ normDiff ] | 32 ) + offsetctl * 2x − 1 ) >> x

}

where divSigTable[ ] is specified as follows:

divSigTable[ ] = {  0, 31, 30, 29, 28, 27, 27, 26, 25, 24, 23, 23, 22, 21, 21, 20, 19, 19, 18,

17, 17, 16, 16, 15, 15, 14, 14, 13, 13, 12, 12, 11, 11, 10, 10, 9, 9, 9, 8,

8, 7, 7, 7, 6, 6, 6, 5, 5, 5, 4, 4, 4, 3, 3, 3, 2, 2, 2, 2, 1, 1, 1, 1, 0 }

      - Bidirectional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

- the bidirectional optical flow utilization flags bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( nCbW >> 2 ) − 1, yIdx = 0..( nCbH >> 2 ) − 1.

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable bitDepth is set equal to BitDepthY.

- The variable shift1 is set to equal to Max( 6, bitDepth − 6 ).

- The variable shift2 is set to equal to Max( 4, bitDepth − 8 ).

- The variable shift3 is set to equal to Max( 1, bitDepth − 11 ).

- The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to 1 << Max( 5, bitDepth − 7 ).

For xIdx = 0..( nCbW >> 2 ) − 1 and yIdx = 0..( nCbH >> 2 ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx << 2) + 1 and ySb is set equal to ( yIdx << 2 ) + 1.

- If bdofUtilizationFlag[ xIdx ][ yIdx ] is equal to FALSE, for x = xSb − 1..xSb + 2, y = ySb − 1.. ySb + 2, the prediction sample values of the current subblock are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 +	(8-766)
	predSamplesL1[ x + 1 ][ y + 1 ] )  >>  shift4 )

- Otherwise (bdofUtilizationFlag[ xIdx ][ yIdx ] is equal to TRUE), the prediction sample values of the current subblock are derived as follows:

  - For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply:

- The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows:

hx = Clip3( 1, nCbW, x )		(8-767)

vy = Clip3( 1, nCbH, y )		(8-768)

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  (predSamplesL0[ hx + 1 ][vy] − predSampleL0[ hx − 1 ][ vy ] ) >> shift1	(8-769)

gradientVL0[ x ][ y ]  =   (predSampleL0[ hx ][ vy + 1 ] − predSampleL0[ hx ][vy − 1 ] ) >> shift1	(8-770)

gradientHL1[ x ][ y ]  =  (predSamplesL1[ hx + 1 ][vy] − predSampleL1[ hx − 1 ][ vy ] ) >> shift1	(8-771)

gradientVL1[ x ][ y ]  =   (predSampleL1[ hx ][ vy + 1 ] − predSampleL1[ hx ][vy − 1 ] ) >> shift1	(8-772)

- The variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 )	(8-773)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3	(8-774)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3	(8-775)

  - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows:

sGx2 = ij( tempH[ xSb + i ][ ySb + j ] * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-776)

sGy2 = ij( tempV[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-777)

sGxGy = ij( tempH[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j  −1..4	(8-778)

sGxGym = sGxGy >> 12		(8-779)

sGxGys =  sGxGy & ( ( 1 << 12 ) − 1 )		(8-780)

sGxdI = ij( −tempH[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-781)

sGydI = ij( −tempV[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-782)

sGx2 = ij Abs( tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-776)

sGy2 = ij Abs( tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-777)

sGxGy = ij( Sign( tempV[ xSb + i ][ ySb + j ] ) * tempH[ xSb + i ][ ySb + j ] ) with i, j  −1..4	(8-778)

sGxGym = sGxGy >> 12		(8-779)

sGxGys =  sGxGy & ( ( 1 << 12 ) − 1 )		(8-780)

sGxdI = ij( − Sign( tempH[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-781)

sGydI = ij( − Sign( tempV[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-782)

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres,	(8-783)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − 	(8-784)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGx2 ) ) )  :  0

vx = Division( ( sGydI << 3 ), sGx2 )

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, vx )  :  0

vy = Division( ( ( sGydI << 3 ) − ( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ), sGy2 )

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, vy )  :  0

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

bdofOffset = Round( ( vx * ( gradientHL01[ x + 1 ][ y + 1 ] – gradientHL10[ x + 1 ][ y + 1 ] ) ) >> 1 ) 
	+ Round( ( vy * (gradientVL01[ x + 1 ][ y + 1 ] – gradientVL10[ x + 1 ][ y + 1 ] ) ) >> 1 )

[Ed. (JC): Round() operation is defined for float input. The Round() operation seems redundant here since the input is an integer value. To be confirmed by the proponent]

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ]  + offset4 +	(8-786)
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )

ISO/IEC VVC

ITU-T Rec. H.VVC

