| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0306-v1 |
| --- | --- |

| Title: | AHG17: Entry point offsets without start code emulation |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Tadamasa Toma Virginie Drugeon | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution proposes to modify the syntax structure for signalling entry point offsets. An encoder needs to know the number of bits in each subset of the current slice in order to write entry point offsets into the slice header. Some real time encoders achieve it by updating this syntax field after finishing encoding the slice. This contribution proposes two items to simplify this encoding operation: (1) byte align before entry point offset field for easier updating during encoding; (2) separate entry points by one bit equal to one and divide into upper and lower bits to avoid start code emulation without the need for an emulation prevention byte.

Introduction

Table1 shows the slice header syntax related to entry point offset in VTM5.0. entry_point_offset_minus1 has a length of 1 to 32 bits, and indicates the first byte position of each subset in slice data minus one as shown in Figure1. When entropy_coding_sync_enabled_flag is equal to 1, a subset will be a luma CTU rows in the slice, otherwise, a subset will be a brick in the slice. Therefore, an encoder needs to know the number of bits in each subset to write this information into the slice header.

Table1: slice header syntax in VTM5.0

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( entropy_coding_sync_enabled_flag ) |  |
| num_entry_point_offsets | ue(v) |
| if( NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

Figure1: entry_point_offset

Problems

In the case of a real time encoder, it is difficult to know the number of bits in each subset when encoding a slice header. Some encoders solve this issue using dummy bytes and updating as shown in Figure2. At first, the encoder outputs a bitstream with a slice header including dummy entry_point_offset values to the bitstream buffer. The encoder later updates the dummy entry_point_offset field with the actual value after finishing encoding each subset. Finally, the updated bitstream is output from the bitstream buffer.

Figure2: encoding operation of entry_point_offset

Considering this operation, there are two problems.

- The entry_point_offset field is not byte aligned. Therefore, an encoder needs to update this field by bit unit operation.

- If a large number of consecutive 0 occurs in the entry_point_offset field, a byte equal to 0x03 (emulation prevention byte) may be inserted thereby extending the field length. As a result, this field would overlap and overwrite the data area of subset0.

This problem also exists in HEVC, but it will be a more significant issue in VVC because there are more use cases that use entry_point_offset (all the use cases using bricks).

Proposal

This contribution proposes to introduce two modifications in the syntax for signaling entry point offsets.

- byte align before entry point offset field for easier updating during encoding.

- separate entry points by one bit equal to one and divide into upper and lower bits to avoid start code emulation without the need for an emulation prevention byte.

Figure3 shows a detailed explanation for modification (2). entry_point_offset_minus1 is divided into upper bits and lower bits if the length of the syntax element exceeds 21, and separation bits equal to one are inserted between each offset.

Figure3: modification of entry_point_offset_minus1

Suggestion of specification text changes

The suggested specification changes are described on top of version 8 of JVET-N1001[1]. The newly added parts are highlighted in yellow and the text to be removed is marked as red.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( entropy_coding_sync_enabled_flag ) |  |
| num_entry_point_offsets | ue(v) |
| if( NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| entry_point_alignment_zero_bit | f(1) |
| for( i = 0; i < NumEntryPoints; i++ ) { |  |
| entry_point_offset_minus1[ i ] | u(v) |
| entry_point_separation_one_bit | f(1) |
| entry_point_offset_minus1_upperbits[ i ] | u(v) |
| if( offset_len_minus1 > 20 ) { |  |
| entry_point_separation_one_bit | f(1) |
| entry_point_offset_minus1_lowerbits[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

num_entry_point_offsets is used to specify the variable NumEntryPoints, which specifies the number of entry points in the current slice as follows:

NumEntryPoints = entropy_coding_sync_enabled_flag ? num_entry_point_offsets :
			NumBricksInCurrSlice − 1		(7-62)

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset_minus1[ i ] the total length, in bits, of the entry_point_offset_minus1_upperbits[ i ] and the entry_point_offset_minus1_lowerbits[ i ] syntax elements. The value of offset_len_minus1 shall be in the range of 0 to 31, inclusive.

entry_point_alignment_zero_bits shall be equal to 0.

entry_point_separation_one_bit shall be equal to 1.

entry_point_offset_minus1_upperbits[ i ] and entry_point_offset_minus1_lowerbits[ i ] are used to derive EntryPointOffsetMinus1[ i ] as follows:

if( offset_len_minus1 > 20 )

EntryPointOffsetMinus1[ i ] = entry_point_offset_minus1_upperbits[ i ] <<

(offset_len_minus1 – 20) | entry_point_offset_minus1_lowerbits[ i ]
else

EntryPointOffsetMinus1[ i ] = entry_point_offset_minus1_upperbits[ i ]

EntryPointOffsetMinus1[ i ] plus 1 specifies the i-th entry point offset in bytes, and is represented by offset_len_minus1 plus 1 bits. The slice data that follow the slice header consists of NumEntryPoints + 1 subsets, with subset index values ranging from 0 to NumEntryPoints, inclusive. The first byte of the slice data is considered byte 0. When present, emulation prevention bytes that appear in the slice data portion of the coded slice NAL unit are counted as part of the slice data for purposes of subset identification. Subset 0 consists of bytes 0 to EntryPointOffsetMinus1[ 0 ], inclusive, of the coded slice data, subset k, with k in the range of 1 to NumEntryPoints − 1, inclusive, consists of bytes firstByte[ k ] to lastByte[ k ], inclusive, of the coded slice data with firstByte[ k ] and lastByte[ k ] defined as:

firstByte[ k ] =n=1k( EntryPointOffsetMinus1[ n-1 ]+1)	(7-63)

lastByte[ k ] = firstByte[ k ] + EntryPointOffsetMinus1[ k ] 	(7-64)

The last subset (with subset index equal to NumEntryPoints) consists of the remaining bytes of the coded slice data.

When entropy_coding_sync_enabled_flag is equal to 0, each subset shall consist of all coded bits of all CTUs in the slice that are within the same brick, and the number of subsets (i.e., the value of NumEntryPoints + 1) shall be equal to the number of bricks in the slice.

When entropy_coding_sync_enabled_flag is equal to 1, each subset k with k in the range of 0 to NumEntryPoints, inclusive, shall consist of all coded bits of all CTUs in a CTU row within a brick, and the number of subsets ( i.e., the value of NumEntryPoints + 1 ) shall be equal to the total number of brick-specific luma CTU rows in the slice.

Conclusion

This contribution proposes two modifications to solve entry point offset encoding issues in real time encoders: (1) byte align before entry point offset field for easier updating during encoding; (2) separate entry points by one bit equal to one and divide into upper and lower bits to avoid start code emulation without the need for an emulation prevention byte. We believe these modifications are useful for real time encoders, and we propose their adoption into VVC.

References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, JVET 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).