| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0307-v1 |
| --- | --- |

| Title: | Non-CE9/3: Unification of division processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Yusuke Kato Tadamasa Toma | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com kato.yusuke003@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

VVC has several block level division processing in some tools, but each tool uses different division method. This contribution proposes to define a common division method and apply it to these tools (We identified these tools to CCLM, BDOF, and DMVR). The common division method is performed by multiplication and shift processing using size 64 LUT. Simulation results reportedly show that the proposed method can unify and simplify division processing with -0.02%, -0.01%, and -0.00% BD-rate for AI, RA, and LDB on VTM-5.0.

Introduction

VVC has several block level division processing in some tools [1]. We identified these tools to CCLM, BDOF, and DMVR as show in Table1. The division processing of each tool is specified using different methods individually. Therefore, a decoder has to implement several kinds of division methods for these tools. This contribution proposes to define a common division method and apply it to these tools.

Table1: Block level division processing in VVC

| Tool | Explanation | Equation | Implementation |
| --- | --- | --- | --- |
| CCLM | parameter a calculation (1 time per CU) | a=maxC-minCmaxY-minY | Size 16 LUT method using the difference between numerator and denominator magnitude. |
| BDOF | vx and vy calculation (2 times per 4x4 sub block) | vx=-sGxdIsGx2 vy=sGydI-vx×sGxGysGy2s2 | Round denominator to power of 2 and perform using only bit shift. |
| DMVR | parametric MV refinement (2 times per CU) | dMvX=sad[3]-sad[5]sad[3]+sad[5]-sad[4]×2 dMvY=sad[1]-sad[7]sad[1]+sad[7]-sad[4]×2 | Repeat the comparison of numerator and denominator and assign to -8 to +8 values. |

Proposal

Definition of common division processing

This contribution proposes the common division processing as follow. This division method is based on the existing specification of CCLM, but size 64 LUT is introduced instead of size 16 LUT. Because of using the size 64 LUT, the equation can be simpler than that of CCLM, and also can keep enough accuracy in the case of higher bit depth input images. The size 64 LUT requires additional memory but the impact may be negligible for a hardware.

Equation: nume / denom

x = Floor ( Log2 ( denom ) );

normDiff = ( ( denom << 6 ) >> x ) & 63;

x += ( normDiff  !=  0 ) ? 6 − frac : 5 − frac;

a = ( nume * ( divTable64 [ normDiff ] | 32 ) + offsetctl * 2x − 1 ) >> x;

divTable64 [ ]

= { 0, 31, 30, 29, 28, 27, 27, 26, 25, 24, 23, 23, 22, 21, 21, 20, 19, 19, 18, 17, 17, 16, 16, 15, 15, 14, 14,

13, 13, 12, 12, 11, 11, 10, 10, 9, 9, 9, 8, 8, 7, 7, 7, 6, 6, 6, 5, 5, 5, 4, 4, 4, 3, 3, 3, 2, 2, 2, 2, 1, 1, 1, 1, 0 }

frac: control parameter of fractional precision

= 5  // CCLM

= 0  // BDOF, DMVR

offsetctl: control parameter of rounding offset

= 1  // CCLM

= 0  // BDOF, DMVR

Apply to CCLM division

CCLM parameter consists of a, b, and k. The division processing is used for the calculation of parameter a. The followings are VTM5.0 methods and proposed methods of CCLM parameters derivation processing. LUT size is increased from 16 bytes to 64 bytes, but the equation can be simpler than VTM5.0 method. Additionally, the cording performance is slightly better than VTM5.0 method.

VTM5.0 method

Equation: a = diffC / diffY

x = Floor ( Log2 ( diffY ) );

normDiff = ( ( diffY << 4 ) >> x ) & 15;

x += ( normDiff  !=  0 ) ? 1 : 0;

y = Floor ( Log2 ( Abs ( diffC ) ) ) + 1;

a = ( diffC * ( divTable16 [ normDiff ] | 8 ) + 2y − 1 ) >> y;

k = ( ( 3 + x − y ) < 1 ) ? 1 : 3 + x − y;

a = ( ( 3 + x − y ) < 1 ) ? Sign ( a ) * 15 : a;

b = minC − ( ( a * minY ) >> k );

divTable16 [ ]

= { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }

Proposed method

Equation: a = diffC / diffY

common division

x = Floor ( Log2 ( diffY ) );

normDiff = ( ( diffY << 6 ) >> x ) & 63;

x += ( normDiff  !=  0 ) ? 1 : 0;

a = ( diffC * ( divTable64 [ normDiff ] | 32 ) + 2x − 1 ) >> x;

k = 5;

a = Clip3 ( −15 << (k − 1), 15 << (k − 1), a );

b = minC − ( ( a * minY ) >> k );

Apply to BDOF division

In BDOF, the refinement MV of Vx and Vy are derived using gradient images and prediction images. The division processing is used for the calculation of Vx and Vy. The followings are VTM5.0 methods and proposed methods of Vx derivation processing. It is common to Vy derivation as well. The equation will be slightly complex than VTM5.0 method, however, some additional gain can be expected since the accuracy of the calculation is improved.

VTM5.0 method

Equation: Vx = sGxdI / sGx2

Vx = (sGxdI << 3) >> Floor (Log2( sGx2 ));

Proposed method

Equation: Vx = sGxdI / sGx2

common division

x = Floor ( Log2 ( sGx2 ) );

normDiff = ( ( sGx2 << 6 ) >> x ) & 63;

x += ( normDiff  !=  0 ) ? 6 : 5;

Vx = ( sGxdI * ( divTable64 [normDiff] | 32 )) >> x;

Apply to DMVR division

In DMVR, parametric motion vector refinement is performed as fractional unit delta motion derivation. The division processing is used for the calculation of dMvX and dMvY of fractional unit delta motion. The followings are VTM5.0 methods and proposed methods of dMvX derivation processing. It is common to dMvY derivation as well. It can be much simpler than VTM5.0 method, but the coding loss is negligible.

VTM5.0 method

Equation: dMvX = ((sad[3] − sad[5]) << 3) / (sad[3] + sad[5] − (sad[4] << 1))

nume = ( sad[3] − sad[5] ) << 4;

denom = (( sad[3] + sad[5] ) − ( sad[4] << 1 )) << 3;

if (denom == 0) dMvX = 0;

else {

if (sad[3] = = sad[4] )  dMvC = −8;

else if (sad[5] = = sad[4] )  dMvC = 8;

else {

signNume = ( nume < 0 ) ? 1 : 0;

nume = abs( nume );

quotient = 0; counter = 3;

while ( counter > 0 ) {

counter = counter − 1; quotient = quotient << 1;

if ( nume >= denom ) {

nume = nume − denom; quotient = quotient + 1;

}

denom = (denom >> 1);

}

dMvX = signNume ? − quotient : quotient;

}

}

Proposed method

Equation: dMvX = ((sad[3] − sad[5]) << 3) / (sad[3] + sad[5] − (sad[4] << 1))

nume = ( sad[3] − sad[5] ) << 3;

denom = ( sad[3] + sad[5] ) − ( sad[4] << 1 );

if (denom == 0) dMvX = 0;

else {

common division

x = Floor ( Log2 ( denom ) );

normDiff = ( ( denom << 6 ) >> x ) & 63;

x += ( normDiff  !=  0 ) ? 6 : 5;

dMvX = ( nume * ( divTable64 [normDiff] | 32 )) >> x;

}

Simulation results

Table1 shows the simulation results of the proposed method. The simulation is conducted using VTM-5.0 with the test condition described in JVET-N1010[2]. These results show that the coding performance of the proposed method has -0.02%, -0.01%, and -0.00% BD-rate for AI, RA, and LDB. It shows that the proposed method can unify and simplify division processing without any coding loss.

Table1: Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.30% | -0.02% | 99% | 102% |
| Class A2 | -0.02% | -0.12% | 0.00% | 101% | 101% |
| Class B | 0.00% | -0.06% | 0.01% | 99% | 101% |
| Class C | 0.00% | -0.09% | 0.02% | 100% | 101% |
| Class E | -0.01% | -0.06% | -0.04% | 99% | 101% |
| Overall | -0.01% | -0.12% | 0.00% | 100% | 101% |
| Class D | -0.01% | 0.07% | 0.05% | 100% | 107% |
| Class F | -0.04% | -0.01% | 0.13% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.65% | 0.06% | 101% | 101% |
| Class A2 | -0.01% | -0.19% | -0.12% | 100% | 101% |
| Class B | 0.00% | -0.06% | -0.19% | 100% | 101% |
| Class C | 0.00% | -0.11% | -0.21% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.22% | -0.13% | 100% | 101% |
| Class D | 0.15% | -0.12% | 0.17% | 101% | 100% |
| Class F | 0.04% | -0.14% | 0.10% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.09% | -0.11% | 100% | 99% |
| Class C | 0.02% | 0.05% | 0.07% | 100% | 100% |
| Class E | 0.00% | -0.03% | 1.17% | 100% | 100% |
| Overall | 0.00% | 0.04% | 0.27% | 100% | 100% |
| Class D | 0.03% | -0.18% | 1.82% | 100% | 103% |
| Class F | -0.11% | 0.17% | -0.16% | 99% | 101% |

Conclusion

VVC has several block level division processing in some tools, but each tool uses different division method. This contribution proposes to define a common division method and apply it to these tools (We identified these tools to CCLM, BDOF, and DMVR). The common division method is performed by multiplication and shift processing using size 64 LUT. Simulation results reportedly show that the proposed method can unify and simplify division processing with -0.02%, -0.01%, and -0.00% BD-rate for AI, RA, and LDB on VTM-5.0. We propose to adopt it into VVC.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, JVET 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

[2]	F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, JVET 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).