| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0309 |
| --- | --- |

| Title: | Non-CE4: Merge mode signalling overhead reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geonjung Ko, Dongcheol Kim, Jason Jung, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 greg.ko@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method to reduce merge mode signalling overhead. The current VTM has 5 merge modes. Regular merge mode is signalled first among the merge modes and each mode except the regular merge mode has conditions to be enabled. In the proposed method, if all of modes that follows a certain mode cannot be enabled, the signalling for the certain mode is not parsed. The proposed method can be applied to ciip_flag, merge_subblock_flag, mmvd_merge_flag and/or regular_merge_flag. The proposed method was tested under the common test conditions with triangle merge mode disabled. Test 1 that utilizes the proposed method on ciip_flag and merge_subblock_flag reportedly shows -0.19%/-0.18%/-0.17% (Y/U/V) average BD-rate change with RA configurations and -0.19%/-0.17%/-0.08% (Y/U/V) average BD-rate change with LDB configurations. Test 2 that utilizes the proposed method on ciip_flag, merge_subblock_flag and mmvd_merge_flag reports -0.20%/-0.22%/-0.20% (Y/U/V) average BD-rate change with RA configurations and -0.22%/-0.32%/-0.36% (Y/U/V) average BD-rate change with LDB configurations.

# Introduction

Merge data syntax indicates which merge mode is used in the current coding unit [1]. There are 5 modes in the merge mode and the signalling order is as follows:

- Regular Merge mode

- MMVD

- Subblock merge mode

- CIIP

- Triangle merge mode

Also, each mode except regular merge mode has enabling conditions.

- MMVD condition is sps_mmvd_enabled_flag.

- Subblock merge mode conditions are ( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ).

- CIIP conditions are ( sps_ciip_enabled_flag && cu_skip_flag == 0 && cbWidth*cbHeight >= 64 && cbWidth < 128 && cbHeight < 128 ).

- Triangle merge mode conditions are ( sps_triangle_enabled_flag && slice_type == B && MaxNumTriangleMergeCand >= 2 && cbWidth*cbHeight >= 64 ).

# Proposed method

Since there are conditions to be met to use a certain merge mode, the proposed method can reduce the redundant signalling based on those conditions.

In the proposed method, if regular_merge_flag, mmvd_merge_flag and merge_subblock_flag are all 0, CIIP conditions are met, and one of triangle merge mode conditions is false, there is no need to parse ciip_flag and ciip_flag can be inferred to be equal to 1.

Similarly, if regular_merge_flag and mmvd_merge_flag are all 0, subblock merge conditions are met, one of CIIP conditions is false, and one of triangle merge mode conditions is false, merge_subblock_flag is not parsed and it is inferred to be equal to 1.

Furthermore, when regular_merge_flag is 0 and MMVD condition is met, if one of subblock merge mode conditions, one of CIIP conditions and one of triangle merge mode conditions are false, it is inferred that the current block uses MMVD.

If MMVD condition, one of subblock merge mode conditions, one of CIIP conditions and one of triangle merge mode conditions are false, regular_merge_flag is not explicitly signalled and it is inferred to be equal to 1.

# Experimental results

The proposed method was implemented on top of VTM-5.0 and tested under the common test conditions [2] with triangle merge mode disabled.

- Anchor: VTM-5.0 with triangle merge mode disabled

- Test: Proposed method with triangle merge mode disabled

The proposed method can be applied to each mode flag exclusively. Test 1 and Test 2 use the proposed method on mmvd_merge_flag, merge_subblock_flag and ciip_flag. The proposed method on regular_merge_flag was also implemented, but it has no impact on BD-rate under these experimental settings.

## Test 1: Proposed method on ciip_flag and merge_subblock_flag

In Test 1, the proposed method was applied to ciip_flag and merge_subblock_flag. Table 1 and Table 2 summarize experimental results for random access and low delay B configurations, respectively.

Table 1. Test 1 results with RA configurations

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 w/ TPM disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.18% | -0.28% | 100% | 100% |
| Class A2 | -0.16% | -0.24% | -0.18% | 100% | 100% |
| Class B | -0.18% | -0.18% | -0.18% | 100% | 100% |
| Class C | -0.19% | -0.15% | -0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.18% | -0.17% | 100% | 100% |
| Class D | -0.15% | -0.08% | -0.11% | 100% | 100% |
| Class F | -0.13% | -0.10% | -0.19% | 100% | 100% |

Table 2. Test 1 results with LDB configurations

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 w/ TPM disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.04% | -0.29% | 100% | 100% |
| Class C | -0.18% | -0.41% | 0.02% | 99% | 100% |
| Class E | -0.31% | -0.06% | 0.14% | 100% | 101% |
| Overall | -0.19% | -0.17% | -0.08% | 100% | 100% |
| Class D | -0.20% | -0.49% | -0.22% | 99% | 100% |
| Class F | 0.02% | 0.00% | -0.57% | 100% | 100% |

## Test 2: Test 1 + Proposed method on mmvd_merge_flag

In Test 2, the proposed method was applied to ciip_flag, merge_subblock_flag, and mmvd_merge_flag. Table 3 and Table 4 summarize experimental results for random access and low delay B configurations, respectively.

Table 3. Test 2 results with RA configurations

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 w/ TPM disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.28% | -0.28% | 100% | 99% |
| Class A2 | -0.19% | -0.29% | -0.23% | 100% | 99% |
| Class B | -0.18% | -0.16% | -0.12% | 100% | 99% |
| Class C | -0.22% | -0.18% | -0.20% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.22% | -0.20% | 100% | 99% |
| Class D | -0.19% | -0.39% | -0.25% | 100% | 99% |
| Class F | -0.14% | -0.12% | -0.19% | 100% | 99% |

Table 4. Test 2 results with LDB configurations

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 w/ TPM disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.35% | -0.60% | 99% | 101% |
| Class C | -0.20% | -0.43% | -0.30% | 99% | 102% |
| Class E | -0.35% | -0.14% | -0.04% | 99% | 104% |
| Overall | -0.22% | -0.32% | -0.36% | 99% | 102% |
| Class D | -0.28% | -0.59% | -0.85% | 99% | 102% |
| Class F | -0.03% | -0.90% | -1.29% | 99% | 101% |

# Conclusion

This contribution proposes to reduce merge mode signalling overhead based on enabling conditions of each merge mode. The proposed method achieves improvement on coding efficiency. Test 1 shows -0.19% average luma BD-rate change with RA configurations. Test 2 shows -0.20% average luma BD-rate change with RA configurations. It is suggested to adopt the proposed method in the next version of VVC draft text and the reference software.

# Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001, 14th meeting, Geneva, CH, 19–27 Mar. 2019.

[2] F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th meeting, Geneva, CH, 19–27 Mar. 2019.

# Proposed specification text

Yellow highlight: Proposed method on ciip_flag and merge_subblock_flag (Test 1)

Gray highlight: Proposed method on mmvd_merge_flag (Test 2 – Test 1)

Green highlight: Proposed method on regular_merge_flag

7.3.7.7	Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( sps_mmvd_enabled_flag ¦ ¦ cbWidth * cbHeight != 32 ) && ( sps_mmvd_enabled_flag ¦¦ ( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) ¦¦ (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] == 0 && cbWidth < 128 && cbHeight < 128 ) ¦¦ (sps_triangle_enabled_flag && slice_type == B && MaxNumTriangleMergeCand >= 2 )) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag && cbWidth * cbHeight != 32 && ( ( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) ¦¦ (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] == 0 && cbWidth < 128 && cbHeight < 128 ) ¦¦ (sps_triangle_enabled_flag && slice_type == B && MaxNumTriangleMergeCand >= 2 ))) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] == 0 && cbWidth < 128 && cbHeight < 128 ) ¦¦ (sps_triangle_enabled_flag && slice_type == B && MaxNumTriangleMergeCand >= 2) ) ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 && sps_triangle_enabled_flag && slice_type == B && MaxNumTriangleMergeCand >= 2) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.8.7	Merge data semantics

regular_merge_flag[ x0 ][ y0 ] equal to 1 specifies that regular merge mode is used to generate the inter prediction parameters of the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When regular_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_mmvd_enabled_flag is equal to 0.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- cbWidth*cbHeight is equal to 32.

- Otherwise if all the following conditions are true, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- sps_mmvd_enabled_flag is equal to 0.

- One of the following conditions is true:

  - MaxNumSubblockMergeCand is equal to 0.

  - cbWidth is less than 8.

  - cbHeight is less than 8.

- One of the following conditions is true:

  - sps_ciip_enabled_flag is equal to 0.

  - cu_skip_flag[ x0 ][ y0 ] is equal to 1.

  - cbWidth is equal to 128.

  - cbHeight is equal to 128.

- One of the following conditions is true:

  - sps_triangle_enabled_flag is equal to 0.

  - slice_type is not equal to B.

  - MaxNumTriangleMergeCand is equal to 0.

- Otherwise, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

mmvd_merge_flag[ x0 ][ y0 ] equal to 1 specifies that merge mode with motion vector difference is used to generate the inter prediction parameters of the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When mmvd_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_mmvd_enabled_flag is equal to 1.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- cbWidth*cbHeight is equal to 32.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise if all the following conditions are true, mmvd_merge _flag[ x0 ][ y0 ] is inferred to be equal to 1:

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- sps_mmvd_enabled_flag is equal to 1.

- One of the following conditions is true:

  - MaxNumSubblockMergeCand is equal to 0.

  - cbWidth is less than 8.

  - cbHeight is less than 8.

- One of the following conditions is true:

  - sps_ciip_enabled_flag is equal to 0.

  - cu_skip_flag[ x0 ][ y0 ] is equal to 1.

  - cbWidth is equal to 128.

  - cbHeight is equal to 128.

- One of the following conditions is true:

  - sps_triangle_enabled_flag is equal to 0.

  - slice_type is not equal to B.

  - MaxNumTriangleMergeCand is equal to 0.

- Otherwise, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

// …

merge_subblock_flag[ x0 ][ y0 ] specifies whether the subblock-based inter prediction parameters for the current coding unit are inferred from neighbouring blocks. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When merge_subblock_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- MaxNumSubblockMergeCand is greater than 0.

- cbWidth is greater than or equal to 8.

- cbHeight is greater than or equal to 8.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- mmvd_merge_flag[ x0 ][ y0 ] is equal to 0.

- One of the following conditions is true:

  - sps_ciip_enabled_flag is equal to 0.

  - cu_skip_flag[ x0 ][ y0 ] is equal to 1.

  - cbWidth is equal to 128.

  - cbHeight is equal to 128.

- One of the following conditions is true:

  - sps_triangle_enabled_flag is equal to 0.

  - slice_type is not equal to B.

  - MaxNumTriangleMergeCand is equal to 0.

- Otherwise, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 0.

merge_subblock_idx[ x0 ][ y0 ] specifies the merging candidate index of the subblock-based merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_subblock_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

ciip_flag[ x0 ][ y0 ] specifies whether the combined inter-picture merge and intra-picture prediction is applied for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When ciip_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- sps_ciip_enabled_flag is equal to 1.

- cu_skip_flag[ x0 ][ y0 ] is equal to 0.

- cbWidth*cbHeight is greater than or equal to 64.

- cbWidth is less than 128.

- cbHeight is less than 128.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- mmvd_merge_flag[ x0 ][ y0 ] is equal to 0.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- One of the following conditions is true:

  - sps_triangle_enabled_flag is equal to 0.

  - slice_type is not equal to B.

  - MaxNumTriangleMergeCand is equal to 0.

- Otherwise, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When ciip_flag[ x0 ][ y0 ] is equal to 1, the variable IntraPredModeY[ x ][ y ] with x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to INTRA_PLANAR.

The variable MergeTriangleFlag[ x0 ][ y0 ], which specifies whether triangular shape based motion compensation is used to generate the prediction samples of the current coding unit, when decoding a B slice. is derived as follows:

- If all the following conditions are true, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 1:

- sps_triangle_enabled_flag is equal to 1.

- slice_type is equal to B.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- MaxNumTriangleMergeCand is greater than or equal to 2.

- cbWidth * cbHeight is greater than or equal to 64.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- mmvd_merge_flag[ x0 ][ y0 ] is equal to 0.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- ciip_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 0.