# Draft Specification Changes

## Test 1: 1xN, 2xN removal + 32 samples per PU enhancement

#### 7.3.7.5 Coding unit syntax

…

…

…

if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) )

intra_subpartitions_mode_flag[ x0 ][ y0 ]

if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY && cbWidth != 4 )

intra_subpartitions_split_flag[ x0 ][ y0 ]

#### 7.4.8.5 Coding unit semantics

…

…

…

intra_subpartitions_split_flag[ x0 ][ y0 ] specifies whether the intra subpartitions split type is horizontal or vertical. When intra_subpartitions_split_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If cbHeight is greater than MaxTbSizeY, intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 0.

- Otherwise, if cbWidth is greater than MaxTbSizeY, intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise, if cbWidth is equal to 4, intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 0.

The variable IntraSubPartitionsSplitType specifies the type of split used for the current luma coding block as illustrated in Table 7-9. IntraSubPartitionsSplitType is derived as follows:

- If intra_subpartitions_mode_flag[ x0 ][ y0 ] is equal to 0, IntraSubPartitionsSplitType is set equal to 0.

- Otherwise, the IntraSubPartitionsSplitType is set equal to 1 + intra_subpartitions_split_flag[ x0 ][ y0 ].

Table 7-9 – Name association to IntraSubPartitionsSplitType

| IntraSubPartitionsSplitType | Name of IntraSubPartitionsSplitType |
| --- | --- |
| 0 | ISP_NO_SPLIT |
| 1 | ISP_HOR_SPLIT |
| 2 | ISP_VER_SPLIT |

The variable NumIntraSubPartitions specifies the number of transform block subpartitions into which an intra luma coding block is divided. NumIntraSubPartitions is derived as follows:

- If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, NumIntraSubPartitions is set equal to 1.

- Otherwise, if one of the following conditions is true, NumIntraSubPartitions is set equal to 2:

  - cbWidth is equal to 4 and cbHeight is equal to 8,

  - cbWidth is equal to 8 and cbHeight is equal to 4.

  - cbWidth is equal to 8, cbHeight is larger than or equal to 8, and IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT.

- Otherwise, NumIntraSubPartitions is set equal to 4.

#### 8.4.5.1 General decoding process for intra blocks

…

…

…

- Otherwise, the following ordered steps apply:

- The variables nW, nH, numPartsX and numPartsY are derived as follows:

nW = IntraSubPartitionsSplitType = = ISP_VER_SPLIT  ?  nTbW / NumIntraSubPartitions  :  nTbW	(8-53)

nH = IntraSubPartitionsSplitType = = ISP_HOR_SPLIT  ?  nTbH / NumIntraSubPartitions  :  nTbH	(8-54)

numPartsX = IntraSubPartitionsSplitType = = ISP_VER_SPLIT  ?  NumIntraSubPartitions  :  1	(8-55)

numPartsY = IntraSubPartitionsSplitType = = ISP_HOR_SPLIT  ?  NumIntraSubPartitions  :  1	(8-56)

- doubleTUsForPU is set to TRUE, if IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and one of the following condidtions is true. Otherwise, doubleTUsForPU is set to FALSE.

  - nTbW is equal to 4 and nTbH is equal to 8 or 16,

  - nTbW is equal to 8 and nTbH is equal to 4.

  - nTbW is equal to 8, nTbH is equal to 8, and IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT.

- If doubleTUsForPU is equal to TRUE:

For xPartIdx = 0..numPartsX − 1 and yPartIdx = 0..numPartsY − 1, the following applies:

- isFirstTUinPU is set to TRUE, if one of the following conditions is true. Otherwise, isFirstTUinPU is set to FALSE.

  - IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT and xPartIdx % 2 is equal to 0

  - IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT and yPartIdx % 2 is equal to 0

- If isFirstTUinPU is equal to TRUE,

  - nPbW = IntraSubPartitionsSplitType = = ISP_VER_SPLIT  ?  nW * 2  :  nW 		(8-57)

  - nPbH = IntraSubPartitionsSplitType = = ISP_HOR_SPLIT  ?  nH * 2  :  nH 		(8-58)

  - The intra sample prediction process as specified in clause 8.4.5.2 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xTb0 + nW * xPartIdx, yTb0 + nH * yPartIdx ), the intra prediction mode predModeIntra, the transform block width nTbW and height nTbH set equal to nPbW and nPbH, the coding block width nCbW and height nCbH set equal to nTbW and nTbH, and the variable cIdx as inputs, and the output is an (nTbW)x(nTbH) array predSamples.

  - Take the predicted sample array of predSamples[ x ][ y ], with x = 0..nW – 1, y = 0..nH – 1, as the input to the next steps.

- Otherwise (isFirstTUinPU is equal to FALSE):

  - If IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT, take the predicted sample array of predSamples[ x ][ y ], with x = nW..nW * 2 – 1, y = 0..nH – 1, as the input to the next steps.

  - Otherwise (IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT), take the predicted sample array of predSamples[ x ][ y ], with x = 0..nW – 1, y = nH..nH * 2 – 1, as the input to the next steps.

- The scaling and transformation process as specified in clause 8.7.2 is invoked with the luma location ( xTbY, yTbY ) set equal to ( xTbY + nW * xPartIdx, yTbY + nH * yPartIdx ), the variable cIdx, the transform width nTbW and the transform height nTbH set equal to nW and nH as inputs, and the output is an (nTbW)x(nTbH) array resSamples.

- The picture reconstruction process for a colour component as specified in clause 8.7.5 is invoked with the transform block location ( xTbComp, yTbComp ) set equal to ( xTb0 + nW * xPartIdx, yTb0 + nH * yPartIdx ), the transform block width nTbW, the transform block height nTbH set equal to nW and nH, the variable cIdx, the (nTbW)x(nTbH) array predSamples, and the (nTbW)x(nTbH) array resSamples as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- Otherwise (doubleTUsForPU is equal to FALSE):

For xPartIdx = 0..numPartsX − 1 and yPartIdx = 0..numPartsY − 1, the following applies:

- The intra sample prediction process as specified in clause 8.4.5.2 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xTb0 + nW * xPartIdx, yTb0 + nH * yPartIdx ), the intra prediction mode predModeIntra, the transform block width nTbW and height nTbH set equal to nW and nH, the coding block width nCbW and height nCbH set equal to nTbW and nTbH, and the variable cIdx as inputs, and the output is an (nTbW)x(nTbH) array predSamples.

- The scaling and transformation process as specified in clause 8.7.2 is invoked with the luma location ( xTbY, yTbY ) set equal to ( xTbY + nW * xPartIdx, yTbY + nH * yPartIdx ), the variable cIdx, the transform width nTbW and the transform height nTbH set equal to nW and nH as inputs, and the output is an (nTbW)x(nTbH) array resSamples.

- The picture reconstruction process for a colour component as specified in clause 8.7.5 is invoked with the transform block location ( xTbComp, yTbComp ) set equal to ( xTb0 + nW * xPartIdx, yTb0 + nH * yPartIdx ), the transform block width nTbW, the transform block height nTbH set equal to nW and nH, the variable cIdx, the (nTbW)x(nTbH) array predSamples, and the (nTbW)x(nTbH) array resSamples as inputs, and the output is a modified reconstructed picture before in-loop filtering.

## Test 2: 1xN, 2xN removal + Nx2 removal

#### 7.3.7.5 Coding unit syntax

…

…

…

if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) )

intra_subpartitions_mode_flag[ x0 ][ y0 ]

if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY && cbWidth != 4 && !( cbWidth = = 8 && cbHeight = = 4 ) )

intra_subpartitions_split_flag[ x0 ][ y0 ]

#### 7.4.8.5 Coding unit semantics

…

…

…

intra_subpartitions_split_flag[ x0 ][ y0 ] specifies whether the intra subpartitions split type is horizontal or vertical. When intra_subpartitions_split_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If cbHeight is greater than MaxTbSizeY, intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 0.

- Otherwise, if cbWidth is greater than MaxTbSizeY, intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise, if cbWidth is equal to 4, intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 0.

- Otherwise, if cbWidth is equal to 8 and cbHeight is equal to 4, intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 1.

The variable IntraSubPartitionsSplitType specifies the type of split used for the current luma coding block as illustrated in Table 7-9. IntraSubPartitionsSplitType is derived as follows:

- If intra_subpartitions_mode_flag[ x0 ][ y0 ] is equal to 0, IntraSubPartitionsSplitType is set equal to 0.

- Otherwise, the IntraSubPartitionsSplitType is set equal to 1 + intra_subpartitions_split_flag[ x0 ][ y0 ].

Table 7-9 – Name association to IntraSubPartitionsSplitType

| IntraSubPartitionsSplitType | Name of IntraSubPartitionsSplitType |
| --- | --- |
| 0 | ISP_NO_SPLIT |
| 1 | ISP_HOR_SPLIT |
| 2 | ISP_VER_SPLIT |

The variable NumIntraSubPartitions specifies the number of transform block subpartitions into which an intra luma coding block is divided. NumIntraSubPartitions is derived as follows:

- If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, NumIntraSubPartitions is set equal to 1.

- Otherwise, if one of the following conditions is true, NumIntraSubPartitions is set equal to 2:

  - cbWidth is larger than or equal to 4, cbHeight is equal to 8, and IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT.

  - cbWidth is equal to 8, cbHeight is larger than or equal to 4, and IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT.

  - Otherwise, NumIntraSubPartitions is set equal to 4.