| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0312-v2 |
| --- | --- |

| Title: | Non-CE3: Simplifications of MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin | Email: | PoHanLin@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

This contribution proposes several simplification methods of matrix weighted intra prediction (MIP). The proposed modifications reduce the complexity of intra mode derivation processes and the parsing process with a negligible gain loss.

Introduction

MIP method is newly intra prediction technique added in VTM5.0. The MIP applies a matrix multiplication and a vector addition to predict the current block. In the current design, the MPM derivation methods of both MIP and conventional intra prediction need a mapping function to map the mode of one to the other. In MIP, three mapping tables are needed due to three different sets used for the different block size and shape. In conventional intra prediction, one mapping table is needed. Also, an MPM derivation process which is different from the conventional one is applied for the MIP.

Proposed modification

The proposed modifications simplify the mode derivation method described in the following.

- In the neighboring mode derivation process of conventional intra prediction, if a neighboring block is MIP predicted, the neighboring mode is set as Planar mode.

- In the neighboring mode derivation process of MIP, if a neighboring block is predicted by conventional intra prediction, the neighboring mode is set as unavailable.

- In the DM mode derivation process of chroma block, if the corresponding luma block is MIP predicted, the DM mode is set as Planar mode.

- The MPM list in MIP is removed and the MIP modes are truncated binary coded.

Simulation results

The following experiments were conducted using the VTM-5.0 software with the VTM configuration and the JVET common test conditions [1] to collect the BD-rate, encoding time and decoding time of the proposed techniques in the All Intra and Random Access configurations. In this contribution, three tests are conducted.

Test 1: Modification 1 + 2

Test 2: Modification 3

Test 3: Modification 4

Table 1: Simulation results of test 1 (Modification 1 + 2)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  |  | EncT |  | DecT |
| Class A1 | 0.00% | 0.04% |  | 0.10% |  | 100% | 100% |  |
| Class A2 | 0.03% | -0.01% |  | -0.02% |  | 100% | 100% |  |
| Class B | 0.01% | 0.01% |  | 0.05% |  | 100% | 100% |  |
| Class C | 0.00% | -0.01% |  | 0.07% |  | 100% | 100% |  |
| Class E | 0.02% | -0.07% |  | -0.08% |  | 100% | 100% |  |
| Overall | 0.01% | 0.00% |  | 0.03% |  | 100% | 100% |  |
| Class D | -0.01% | 0.04% |  | 0.02% |  | 100% | 100% |  |
| Class F | 0.00% | 0.00% |  | 0.04% |  | 99% | 100% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  |  | EncT |  | DecT |
| Class A1 | 0.01% |  | 0.00% |  | 0.14% |  |  | 100% |  | 100% |
| Class A2 | 0.05% |  | 0.19% |  | 0.12% |  |  | 100% |  | 100% |
| Class B | 0.03% |  | 0.03% |  | 0.08% |  |  | 100% |  | 100% |
| Class C | -0.05% |  | 0.03% |  | -0.07% |  |  | 100% |  | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% |  | 0.06% |  | 0.06% |  |  | 100% |  | 100% |
| Class D | 0.03% |  | 0.09% |  | 0.10% |  |  | 100% |  | 100% |
| Class F | -0.01% |  | -0.09% |  | 0.20% |  |  | 100% |  | 100% |

Table 2: Simulation results of test 2 (Modification 3)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  |  | EncT |  | DecT |
| Class A1 | -0.01% |  | 0.06% |  | 0.04% |  |  | 99% |  | 100% |
| Class A2 | 0.03% |  | 0.02% |  | -0.02% |  |  | 100% |  | 100% |
| Class B | 0.01% |  | -0.04% |  | 0.09% |  |  | 100% |  | 100% |
| Class C | 0.00% |  | -0.12% |  | 0.05% |  |  | 100% |  | 100% |
| Class E | 0.03% |  | 0.01% |  | 0.03% |  |  | 100% |  | 100% |
| Overall | 0.01% |  | -0.02% |  | 0.04% |  |  | 100% |  | 100% |
| Class D | -0.03% |  | 0.19% |  | 0.00% |  |  | 100% |  | 99% |
| Class F | -0.02% |  | -0.02% |  | 0.08% |  |  | 100% |  | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  |  | EncT |  | DecT |
| Class A1 | 0.01% | -0.02% |  | 0.22% |  | 100% | 100% |  |
| Class A2 | 0.07% | 0.21% |  | 0.06% |  | 100% | 100% |  |
| Class B | 0.01% | 0.04% |  | 0.01% |  | 100% | 100% |  |
| Class C | -0.02% | 0.15% |  | 0.01% |  | 100% | 100% |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.09% |  | 0.06% |  | 100% | 100% |  |
| Class D | 0.00% | 0.07% |  | 0.00% |  | 100% | 100% |  |
| Class F | 0.00% | 0.13% |  | 0.16% |  | 100% | 100% |  |

Table 3: Simulation results of test 3 (Modification 4)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | 0.08% | 99% | 100% |
| Class A2 | 0.02% | -0.07% | -0.04% | 99% | 100% |
| Class B | 0.01% | -0.01% | 0.00% | 99% | 100% |
| Class C | 0.00% | -0.01% | 0.06% | 99% | 99% |
| Class E | 0.02% | 0.00% | 0.01% | 99% | 100% |
| Overall | 0.01% | -0.01% | 0.02% | 99% | 100% |
| Class D | -0.01% | 0.15% | -0.03% | 99% | 99% |
| Class F | 0.01% | 0.00% | 0.07% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.10% | 100% | 100% |
| Class A2 | 0.01% | 0.06% | 0.00% | 100% | 100% |
| Class B | 0.01% | 0.06% | -0.07% | 100% | 100% |
| Class C | -0.02% | -0.10% | -0.16% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.05% | 100% | 100% |
| Class D | 0.01% | 0.22% | 0.17% | 100% | 100% |
| Class F | -0.01% | -0.03% | -0.03% | 100% | 100% |

Conclusions

The proposed modifications simplify the mode derivation process of DM, conventional MPM and MIP MPM. And the simulation results show negligible impact on BD-rate and encoding/decoding time. It is suggested to adopt the proposed modification as part of VVC.

Reference

- F. Bossen, J. Boyce, X. Li ,V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, Apr. 2019, Geneve, CH.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).