| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0313 |
| --- | --- |

| Title: | CE4-related: On conditions for enabling PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chiao Yang Po-Han Lin | Tel: Email: | yuciaoyang@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

Prediction refinement with optical flow (PROF) [1] is a method to enhance the prediction signal of affine mode by using optical flow and tested in CE4 [2]. In the optical flow equation, the prediction refinement is calculated by the sample motion vector difference and spatial gradient. This contribution proposes a method to disabling PROF on the affine blocks which may not benefit of applying PROF. The result shows no BD rate change on Test 1 compare with CE4-2.1.a and negligible BD rate difference on Test 2 compare with CE4-2.1.b.

Introduction

PROF is a method to enhance the prediction signal of affine mode. In the current design of CE4-2.1 software, there are several conditions on PROF in decoder. If one of the following conditions is true, PROF is disabled.

Condition 1: Control point motion vectors are all equal.

Condition 2: The size of reference block bounding box exceed a pre-defined threshold [3].

If PROF is enabled, the following steps are applied:

Step 1) The sample motion vector difference ΔV(i, j) can be derived by the following equations

ΔVx(i, j) = ai + bjΔVy(i, j) = di + ej

Where a, b, d, e are the affine parameters, and (i, j) is the sample location relative to the center of an affine sub-block.

ΔV(i, j) is rounded by the following equation before calculated prediction refinement, the shift is related to the position precision, affine parameter precision and ΔV precision.

ΔV(i, j) = Round(ΔV(i, j)) = (ΔV(i, j) + (1<<(shift-1) + ΔV(i, j)>=0 ? -1 : 0)) >> shift

Step 2) The spatial gradients Gx(i, j) and Gy(i, j) are calculated by applying a 3 tap filter in the sub-block.

Step 3) Prediction refinement (ΔI) is derived by the following equation and the final prediction (I’) can be generated by adding the refinement to the sub-block prediction (I).

ΔI(i, j) = Gx(i, j) × ΔVx(i, j) + Gy(i, j) × ΔVy(i j)

I’(i, j) = I(i, j) + ΔI(i, j)

Proposed Conditions for Disabling PROF

On some PROF enabled blocks, each of sample motion vector differences may be rounded to zero of an affine block. In this situation, the prediction refinement will be zero for all samples. This contribution proposes to conditionally skip the PROF of an affine block that have small ΔV.

According to the characteristics of the affine model of an affine sub-block, the maximum value of ΔV appears in the 4 corners in an affine sub-block. If the value ΔV(i, j) at the 4 corners are all equal to zero after rounding, means all the ΔV(i, j) inside the sub-block are rounded to zero, which causes ΔV(i, j) is zero for all the samples. Thus, the PROF process of this affine sub-block may be unnecessary.

Fig. 1 shows coordinate system of a 4x4 sub-block with fractional interpolation, and the coordinate of 4 corners relative to center of sub-block are (-6, -6), (6, -6), (-6, 6), (6, 6), respectively. The ΔV of the 4 corners are list as follows.

ΔVx-6, -6= -6a + -6bΔVy(-6, -6) = -6d + -6e

ΔVx6, -6= 6a + -6bΔVy(6, -6) = 6d + -6e

ΔVx-6, 6= -6a + 6bΔVy(-6, 6) = -6d + 6e

ΔVx6, 6= 6a + 6bΔVy(6, 6) = 6d + 6e

To determine the maximum ΔV of the 4 corners, the 4 equations can be simplified by an absolute operation as follows and compared with a rounding threshold to decide whether to apply PROF.

ΔmaxVx= abs( 6a ) + abs( 6b )ΔmaxVy = abs( 6d ) + abs( 6e )

From the rounding function, the ΔV is rounded to zero if abs( ΔV ) is less than or equal to abs( 1<<( shift-1 ) ). Thus, if none of ΔmaxVx and ΔmaxVy is greater than abs( 1<<( shift-1 ) ), the PROF can be skipped in the affine block. The condition 1 also can be removed because if control point motion vectors are all equal, it ensures that ΔmaxV is zero.

Fig. 1 Coordinate system of a 4x4 block.

Experimental result

Table 1 shows the probability of disabling PROF by conditions. And If one of the following conditions is true, PROF is disabled.

Condition 1: Control point motion vectors are all equal.

Condition 2: The size of reference block bounding box exceed a pre-defined threshold.

Condition 3 (Proposed): None of ΔmaxVx and ΔmaxVy is greater than the rounding threshold.

Test 1: Apply Condition 3 on top of CE4-2.1.a and remove Condition 1.

Test 2: Apply Condition 3 on top of CE4-2.1.b and remove Condition 1.

The test condition described in JVET-N1010 [4] is used in the simulation. Table 2 shows the simulation results of Test 1, and the coding performance is unchanged compare with CE4-2.1.a but skip around 34.87% PROF process on affine blocks. Table 3. shows the simulation results of Test 2, and the coding performance is negligible difference compare with CE4-2.1.b, because some of the bi-prediction affine blocks with ΔV is equal to zero are force to use 6-tap interpolation filter when PROF is enabled in test CE4-2.1.b, but PROF can be disabled on these bi-prediction affine blocks in Test 2 and use 8-tap interpolation filter follows the current design in VTM-5.0.

Table 1. Probability of Disabling PROF of Affine Luma Blocks.

|  | Disabling Probability |  |  |
| --- | --- | --- | --- |
|  | Condition 1 | Condition 2 | Condition 3 |
| CE4-2.1.a RA | 23.6598% | 0.5819% | 0.0000% |
| CE4-2.1.b RA | 25.1501% | 0.6250% | 0.0000% |
| Test 1 RA | 0.0000% | 0.5819% | 34.8713% |
| Test 2 RA | 0.0000% | 0.6156% | 33.8086% |
| CE4-2.1.a LB | 21.1411% | 0.6298% | 0.0000% |
| CE4-2.1.b LB | 21.7222% | 0.5716% | 0.0000% |
| Test 1 LB | 0.0000% | 0.6298% | 33.9609% |
| Test 2 LB | 0.0000% | 0.6154% | 30.3716% |

Table 2. Summary of experimental results (Test 1 over CE4-2.1.a)

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1.a |  |  |  |  | Over CE4-2.1.a |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 3. Summary of experimental results (Test 2 over CE4-2.1.b)

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1.b |  |  |  |  | Over CE4-2.1.b |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.03% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | -0.08% | 0.05% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.02% | -0.04% | 100% | 100% | 0.03% | 0.01% | 0.06% | 100% | 99% |
| Class C | -0.02% | 0.02% | -0.02% | 100% | 100% | -0.03% | -0.18% | 0.30% | 100% | 100% |
| Class E |  |  |  |  |  | 0.06% | -0.47% | -0.62% | 100% | 99% |
| Overall | -0.01% | 0.00% | -0.02% | 100% | 100% | 0.02% | -0.17% | -0.03% | 100% | 99% |
| Class D | -0.01% | 0.01% | -0.01% | 100% | 100% | 0.03% | -0.44% | 0.28% | 100% | 99% |
| Class F | -0.01% | 0.03% | -0.04% | 100% | 100% | -0.02% | -0.01% | 0.37% | 100% | 99% |

Conclusion

This contribution proposes conditionally disabling PROF on the affine blocks which may not benefit of applying PROF. Experimental result show that the conditions can skip around 34% PROF process of affine mode without coding performance change on top of CE4-2.1.a. It is suggested to adopt proposed condition into the next VVC working draft if PROF has been adopted.

Reference

- J. Luo, Y. He, “CE2-related: Prediction refinement with optical flow for affine mode,” JVET-N0236, Geneva, CH, March 2019.

- H. Yang, C.-C. Chen, “Description of Core Experiment 4 (CE4): Inter prediction,” JVET-N1024, Geneva, CH, March 2019.

- M. Zhou, “CE2: On restriction of memory bandwidth consumption of affine mode (CE2-4.8),” JVET-N0068, Geneva, CH, March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March 2019.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).