| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0314-v2 |
| --- | --- |

| Title: | Non-CE4: Modification of MV storage in triangle merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin | Email: | PoHanLin@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

This contribution proposes a simplification method of the MV storage method of triangle merge mode. The proposed modification simplifies the process of MV storage in triangle merge mode. The simulation result shows a small gain loss and negligible encoding/decoding time change compared with VTM5.0.

Introduction

Triangle merge mode allows a merge CU split into two blocks with diagonal or anti-diagonal partition. The detail of triangle merge mode is described in [1]. Each sub-block is predicted by uni-directional inter prediction and the two predictors are blended. The two partition types of triangle merge mode are described in Figure 1.

Fig. 1. Partition types of triangle merge mode

To generate the triangle merge mode predictor, each triangular block is inter-predicted using its own motion information. After the two predictors are generated, blending process is applied to combine the two prediction signals. A position-dependent weighting is used to blend the samples around the partition edge. In VTM5.0, {7/8, 6/8, 5/8, 4/8, 3/8, 2/8, 1/8} and {6/8, 4/8, 2/8} weightings are used for luma and chroma component.

The motion field storage of each 4x4 unit inside the CU depends on the position of the unit. Denote Mv1 and Mv2 as the motion vector for partition 1 and partition 2. If a unit is located in the non-weighted area of partition 1, Mv1 is stored, if a unit is located in the non-weighted area of partition 2, then Mv2 is stored. If a unit is located in the weighted area, a bi-directional prediction motion vector derivation method described in the following is processed.

- If Mv1 and Mv2 are from different reference lists, Mv1 and Mv2 are combined to a bi-prediction motion vector.

- Otherwise, if the reference picture of Mv1/Mv2 appears in the other list, the reference index is converted to the picture in the other list. Then the two Mv are combined to a bi-prediction motion vector.

- Otherwise, only Mv1 is stored.

Proposed modification

The motion vector storage process of the non-weighted area is kept the same. And the motion vector storage process of the weighted area is simplified by the following:

- If Mv1 and Mv2 are from different reference lists, Mv1 and Mv2 are combined to a bi-prediction motion vector.

- Otherwise, only Mv1 is stored.

By the modification, the derivation process and the specification could be simplified. The process to map the reference index to the other list is also removed.

Simulation results

The following experiments were conducted using the VTM-5.0 software with the VTM configuration and the JVET common test conditions [2] to collect the BD-rate, encoding time and decoding time of the proposed techniques in the RA and LB configurations. The proposed method has a BD-rate performance of 0%/-0.02%/-0.04% for Y/Cb/Cr with negligible encoding/decoding time change for the RA configuration, and BD-rate performance of 0.08%/0.14%/0.01% for Y/Cb/Cr with negligible encoding/decoding time change for the LB configuration.

Table 1: Simulation results of the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | 0.03% | 100% | 100% |
| Class A2 | 0.01% | -0.01% | 0.06% | 100% | 100% |
| Class B | 0.01% | 0.06% | -0.01% | 100% | 100% |
| Class C | -0.03% | -0.04% | -0.20% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.04% | 100% | 100% |
| Class D | 0.00% | 0.12% | 0.26% | 100% | 101% |
| Class F | 0.00% | 0.05% | 0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.02% | 0.20% | 100% | 100% |
| Class C | 0.10% | 0.15% | 0.15% | 101% | 100% |
| Class E | 0.06% | 0.38% | -0.51% | 101% | 100% |
| Overall | 0.08% | 0.14% | 0.01% | 100% | 100% |
| Class D | 0.13% | -0.18% | 0.35% | 100% | 100% |
| Class F | -0.04% | 0.02% | 0.08% | 100% | 100% |

Conclusions

This contribution proposes a simplification method of the MV storage method of triangle merge mode. And the simulation results show small impact on BD-rate and negligible encoding/decoding time. It is suggested to adopt the proposed modification as part of VVC.

Reference

- J. Chen, Y. Ye and S. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5)”, JVET-N1002, the 14th JVET meeting, Apr. 2019, Geneve, CH.

- F. Bossen, J. Boyce, X. Li ,V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, Apr. 2019, Geneve, CH.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).