| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0315-v1 |
| --- | --- |

| Title: | CE8-related: Intra prediction mode alignment for BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current design, Block-based quantized residual domain DPCM uses a special MPM (Planar mode in VVC draft 5) as the stored intra prediction mode, although it is performed horizontally or vertically. It is proposed to align its stored intra mode with the actual intra mode employed in BDPCM. It is asserted that with the proposed method, the MPM list construction process could be simplified since the checking of BDPCM mode of neighboring blocks is avoided. The simulations result reports that the proposed modification can achieve -0.18%/-0.18%/-0.04% BD-Rate changes under AI/RA/LDB configurations versus the VTM-5.0 anchor on class TGM. It is also reported that the proposed modification can achieve -0.34%/-0.26%/0.00% BD-Rate changes under AI/RA/LDB configurations versus the VTM-5.0 anchor on class TGM1080p 444 sequences.

# Introduction

Block-based quantized residual domain DPCM (BDPCM) was proposed in the last meeting and adopted into the VVC draft [1]. The intra prediction is performed for the entire block by sample copying along prediction direction (horizontal or vertical prediction) like the normal intra prediction. The residual is quantized and the delta between the quantized residual and its predictor (horizontal or vertical) quantized value is coded.

After coding the current CU, the BDPCM always uses a specific MPM (always be Planar mode in VTM-5.0) as the stored intra prediction mode which can be used as an MPM candidate by the successive PUs. This special treatment, however complicates the text and also comprises the coding efficiency.

# Proposed Method

In this document, we propose to align the stored intra mode in BDPCM with its actual intra mode. For example, if the BDPCM use the vertical intra prediction, the stored intra mode will be VER_IDX. Similarly, if the BDPCM use the horizontal intra prediction, the stored intra mode will be HOR_IDX.

# Simulation Results

The proposed scheme is implemented on top of VTM5. The simulation follows the CE 8 test condition [2]. The anchor is VTM-5.0 and the test is the proposed method with BDPCM always on.

Two sets of simulations have been tested, as follows:

1) Test #1: Test conditions of 4:2:0 sequences in CE8. Anchor is VTM-5.0 under CTC. The test is the proposed with BDPCM always on.

2) Test #2: Test conditions of 4:4:4 sequences in CE8. BDPCM, IBC and HashME are all enabled in both the anchor and the test.

Results are summarized in the following tables.

Table 1. Test #1 AI results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.05% | 0.04% | -0.06% | 105% | 101% |
| Class A2 | -0.02% | -0.03% | -0.01% | 104% | 100% |
| Class B | -0.02% | -0.09% | 0.01% | 105% | 102% |
| Class C | -0.07% | -0.17% | 0.02% | 104% | 102% |
| Class E | -0.11% | -0.12% | -0.14% | 105% | 100% |
| Overall | -0.05% | -0.08% | -0.03% | 105% | 101% |
| Class D | -0.10% | -0.04% | -0.16% | 105% | 95% |
| Class F | -0.12% | 0.04% | -0.08% | 102% | 97% |
| TGM | -0.18% | -0.06% | -0.07% | 101% | 97% |

Table 2. Test #1 RA results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.01% | -0.21% | 0.12% | 103% | 102% |
| Class A2 | 0.03% | 0.14% | 0.01% | 102% | 102% |
| Class B | 0.00% | 0.02% | -0.16% | 102% | 102% |
| Class C | -0.07% | -0.22% | -0.14% | 102% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.06% | 102% | 103% |
| Class D | -0.04% | 0.00% | 0.17% | 102% | 107% |
| Class F | -0.09% | 0.08% | 0.10% | 103% | 103% |
| TGM | -0.18% | -0.02% | -0.13% | 103% | 102% |

Table 3. Test #1 LDB results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.35% | 0.33% | 102% | 100% |
| Class C | 0.01% | -0.24% | -0.20% | 102% | 105% |
| Class E | -0.13% | 0.26% | 1.19% | 102% | 100% |
| Overall | -0.03% | 0.13% | 0.37% | 102% | 102% |
| Class D | 0.06% | -0.03% | 0.73% | 101% | 103% |
| Class F | 0.02% | -0.21% | -0.44% | 102% | 96% |
| TGM | -0.04% | 0.01% | 0.04% | 99% | 100% |

Table 4. Test #2 AI results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.34% | -0.21% | -0.23% | 102% | 103% |
| TGM 720p | -0.17% | -0.17% | -0.10% | 100% | 98% |
| Animation | -0.04% | 0.01% | 0.01% | 101% | 99% |
| Mixed content | -0.32% | -0.12% | -0.15% | 100% | 98% |
| Camera-Captured content | 0.00% | 0.02% | -0.03% | 100% | 102% |
| Overall | -0.21% | -0.12% | -0.12% | 101% | 100% |

Table 5. Test #2 RA results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.26% | -0.13% | -0.15% | 100% | 100% |
| TGM 720p | -0.12% | -0.13% | -0.03% | 100% | 102% |
| Animation | -0.02% | -0.01% | 0.00% | 100% | 100% |
| Mixed content | -0.03% | 0.01% | 0.11% | 99% | 100% |
| Camera-Captured content | 0.04% | 0.06% | -0.17% | 100% | 100% |
| Overall | -0.10% | -0.06% | -0.05% | 100% | 101% |

Table 6. Test #2 LDB results

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.00% | -0.07% | -0.04% | 100% | 98% |
| TGM 720p | 0.04% | -0.22% | -0.02% | 100% | 98% |
| Animation | 0.01% | 0.04% | -0.01% | 101% | 100% |
| Mixed content | -0.06% | 0.13% | -0.04% | 98% | 100% |
| Camera-Captured content | 0.03% | -0.04% | -0.04% | 101% | 102% |
| Overall | 0.00% | -0.05% | -0.03% | 100% | 99% |

# Conclusions

This document presented a method on BDPCM which aligns the stored intra mode with the actual intra mode employed by BDPCM so that the checking of BDPCM mode of neighboring blocks is avoided. We recommend adopting the proposed modification into the next draft of VVC.

# Text Changes

The changes on top of the draft provided by JVET- N1001-v8 [1] are highlighted in grey. Deleted texts are marked with strikethrough.

8.4.1 General decoding process for coding units coded in intra prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs.

When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the decoding process for luma samples is specified as follows:

- If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( cbHeight * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ),	(8-6)
		with i = 0..cbWidth − 1, j = 0..cbHeight − 1

- Otherwise, the following applies:

- The variable MipSizeId[ x ][ y ] for x = xCb..xCb + cbWidth − 1 and  y = yCb..yCb + cbHeight − 1 is derived as follows:

- If both cbWidth and cbHeight are equal to 4, MipSizeId[ x ][ y ] is set equal to 0.

- Otherwise, if both cbWidth and cbHeight are less than or equal to 8, MipSizeId[ x ][ y ] is set equal to 1.

- Otherwise, MipSizeId[ x ][ y ] is set equal to 2.

- The luma intra prediction mode is derived as follows:

- If intra_mip_flag[ xCb ][ yCb ] is equal to 1, the derivation process for matrix-based intra prediction mode as specified in clause 8.4.2 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input.

- Otherwise, the derivation process for the luma intra prediction mode as specified in clause 8.4.3 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input.

- The variable predModeIntra is derived as follows:

- If BdpcmFlag[ xCb ][ yCb ] is equal to 1, predModeIntra is derived as follows:

predModeIntra = BdpcmDir[ xCb ][ yCb ]  ?  50  :  18	(8-7)

- Otherwise, predModeIntra is set equal to IntraPredModeY[ xCb ][ yCb ].

- The general decoding process for intra blocks as specified in clause 8.4.5.1 is invoked with the sample location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight, predModeIntra, and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before in-loop filtering.

8.4.3 Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-3 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-3 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived as follows:

- If BdpcmFlag[ xCb ][ yCb ] is equal to 1, predModeIntra is derived as follows:

IntraPredModeY[ xCb ][ yCb ] = BdpcmDir[xCb][yCb] ? INTRA_ANGULAR50 : INTRA_ANGULAR18

- Otherwise, if BdpcmFlag[ xCb ][ yCb ] is equal to 1 or intra_luma_not_planar_flag[ xCb ][ yCb ] is equal to 0, IntraPredModeY[ xCb ][ yCb ] is set equal to INTRA_PLANAR.

- Otherwise (intra_luma_not_planar_flag[ xCb ][ yCb ] is equal to 1), the following ordered steps apply:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and ciip_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is derived as follows:

- If intra_mip_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is derived using IntraPredModeY[ xNbX ][ yNbX ] and MipSizeId[ xCb ][ yCb ] as specified in Table 8-4.

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Mar. 2019

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-N1028, Mar. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).