| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0317 |
| --- | --- |

| Title: | CE3-related: MPM List Modification for Zero Reference Line |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng Fang, Dong Jiang, Jucai Lin, and Feiyang Zeng | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com fang_cheng1@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution tests the MPM modification Method I and Method II proposed in JVET-N0157 on VTM5.0. The proposed methods maintain almost the same MPM structure as VTM5.0, but give the priority to the DC mode under certain conditions.

Experimental results of both methods are shown in below, the anchor is VTM5.0 under AI configuration:

- Method I: The average BD-rate gains of the first experiment are -0.03%, 0.00%, 0.00% for Y, U, V

- Method II: The average BD-rate gains of the second experiment are x%, x%, x% for Y, U, V

Introduction

In VTM5.0[1], the proposal JVET-N0185 was adopted, which proposed a method using a unified 6-MPM list in the intra mode coding, regardless of whether multiple reference line (MRL) or intra sub-partition (ISP) coding mode is applied or not.

Suppose the mode of the left is denoted as Left and the mode of the above block is denoted as Above, the unified MPM list is constructed as follows:

- When a neighboring block is not available, its intra mode is set to Planar by default.

- If both modes Left and Above are non-angular modes:

  - MPM list  {Planar, DC, V, H, V-4, V+4}

- If one of modes Left and Above is angular mode, and the other is non-angular:

  - Set a mode Max as the larger mode in Left and Above

  - MPM list  {Planar, Max, DC, Max -1, Max +1, Max -2}

- If Left and Above are both angular and they are different:

  - Set a mode Max as the larger mode in Left and Above

  - if the difference of mode Left and Above is in the range of 2 to 62, inclusive

    - MPM list  {Planar, Left, Above, DC, Max -1, Max +1}

  - Otherwise

    - MPM list  {Planar, Left, Above, DC, Max -2, Max +2}

- If Left and Above are both angular and they are the same:

  - MPM list  {Planar, Left, Left -1, Left +1, DC, Left -2}

Algorithm description

The proposed methods change the MPM list order for the case that If one neighboring mode is DC and the other neighboring mode is an angular mode a for zero reference line (non-ISP), the two methods are as follows:

Method I. mpm[0] = DC_IDX mpm[1] = a mpm[2] = Planar_IDX mpm[3] = 2 + ( ( maxAB + 61 ) % 64 ) mpm[4] = 2 + ( ( maxAB − 1 ) % 64 ) mpm[5] = 2 + ( ( maxAB + 60 ) % 64 ) Method II. mpm[0] = DC_IDX mpm[1] = Planar_IDX mpm[2] = a mpm[3] = 2 + ( ( maxAB + 61 ) % 64 ) mpm[4] = 2 + ( ( maxAB − 1 ) % 64 ) mpm[5] = 2 + ( ( maxAB + 60 ) % 64 )

Experimental results

According to the methods mentioned above, there are two tests:

- Test 1: Method I

- Test 2: Method II

All the experiments are implemented on top of VTM5.0 software. The simulations are performed on Linux cluster using VTM AI configurations under the common test condition described in JVET-M1010 [2].

Table 1 and Table 2 show the experimental results of Test 1 and Test 2 respectively.

Table 1 Coding performance of Test 1 over VTM-5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.04% | 0.00% | 0.82 | 0.80 |
| Class A2 | 0.03% | 0.02% | -0.04% | 0.80 | 0.84 |
| Class B | 0.03% | -0.03% | 0.01% | 0.87 | 0.79 |
| Class C | 0.01% | -0.02% | -0.02% | 0.88 | 0.84 |
| Class E | 0.04% | 0.13% | 0.07% | 0.78 | 0.74 |
| Overall | 0.03% | 0.00% | 0.00% | 0.84 | 0.80 |
| Class D | 0.00% | -0.05% | -0.08% | 0.93 | 0.87 |
| Class F |  |  |  |  |  |

Table 2 Coding performance of Test 2 over VTM-5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Conclusion

In this contribution, two modifications are proposed to modify the MPM list. When one neighboring mode is DC and the other neighboring mode is an angular mode, it is proposed to give the priority to the DC mode. It is proposed to adopt this proposal into VTM.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Dahua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).