| Combined Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0318 |
| --- | --- |

| Title: | Non-CE3:Intra Combined Predict |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feiyang Zeng Dong Jian Jucai Lin Cheng Fang | Email: | zeng_feiyang@dahuatech.com jiang_dong@dahuatech.com lin_jucai@dahuatech.com fang_cheng1@dahuatech.com |
| Source: | Dahua(ZHEJIANG DAHUA TECHNOLOGY CO.,LTD) |  |  |

_____________________________

- Abstract

This contribution based on the existing intra prediction methods. It obtain the best intra prediction mode of current block and prediction mode of neighbor coded block，the two prediction modes are used to perform weighted prediction to current block. If the rdcost value obtained by the combined prediction method is less than the rdcost value obtained by the best prediction mode, current block adopts combined prediction method, otherwise, adopts existing intra prediction modes.

Algorithm description

This method includes two steps:

- After determining the best prediction mode “modecur” of current block, get the prediction mode “modeL” of the L block. If the following conditions are satisfied:

- modecur != modeL;

- Both width and height of current block is greater than 16;

- isp is not used in the current block;

The current block is predicted by using modecur and modeL respectively, and the prediction values predcur and predL are obtained. The weighted prediction values of current block are as follows:

predcom=3*predcur+predL≫2

Figure1. Positions of current block and neighbouring blocks

- The rdcost obtained from the best prediction model is compared to the rdcost obtained from the combined prediction method. If the rdcost of combined prediction is smaller, flag CombinePred will be set to true, current block prediction using combined prediction method. Otherwise, flag CombinePred will be set to false, current block prediction using existing intra prediction methods.

In order to indicate whether to adopt combined prediction for the current block, all intra blocks which satisfy the conditions in Step 1 are marked CombinePred.

Experimental results

The proposed modification is implemented on VTM-5.0 reference software. The simulations are performed on Linux cluster using VTM AI configurations under the common test condition described in JVET-N1010 [2].

Table1. Performance of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | 0.21% | 0.21% | 106% | 104% |
| Class A2 | 0.05% | 0.05% | 0.04% | 102% | 101% |
| Class B | 0.04% | 0.02% | 0.06% | 106% | 105% |
| Class C | 0.00% | -0.04% | -0.02% | 102% | 102% |
| Class E | 0.07% | 0.08% | 0.04% | 102% | 104% |
| Overall | 0.06% | 0.05% | 0.06% | 104% | 103% |
| Class D | 0.00% | 0.03% | 0.03% | 100% | 101% |
| Class F |  |  |  |  |  |

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Dahua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).