| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0319r1 |
| --- | --- |

| Title: | Adaptive resolution change (ARC) in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala Madhu P Krishnan Wei Dai | Email: | Pankajtva at gmail dot com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

ARC has received increased attention in the past three meeting cycles, and numerous documents consider ARC for VVC. This contribution focuses on an ARC solution that supports any resampling ratio. The solution requires an additional flag in the PPS to notify a change in resolution and two additional parameters for the resampled height and width. The resampling filters used in the proposed solution are a 9-tap downsampling filter (complexity of 7-tap), and 4-tap upsampling filters with 32 phases, which is also part of the existing design of VVC. Gains of about 0.4dB in PSNR (using phase 0 of the upsampling filter) are reported without encoding, when compared to filters described in JVET-M0135, on a subset of test data. An average BD-rate gain of about 2.35% is reported for the luma(Y) channel with compression.

# Introduction

The concept of ARC was originally part of H.263 Annex P [2] , and was introduced in concept for HEVC by JCTVC-F158 [3]; the acronym “ARC” itself, which at the time stood for Adaptive Resolution Coding (where “Coding” is now “Change”), was actually first used in [8]. JCTVC-F158 discussed the utility of adapting resolution within a single stream without introducing an IDR frame. The document JVET-M0135 cites that contribution and uses a zero phase 5-tap downsampling filter, and suggests for upsampling the half-pel motion compensation interpolation filter coefficients, which we believe is described in VVC WD [4] Table 8-14 for scaling operations.  In this contribution FastVDO details: a)  capabilities of the proposed ARC solution; b) high level syntax to signal the resolution change and c) introduces a zero phase 9-tap downsampling filter and 4-tap upsampling filters with 32 phases,which is also listed in VVC WD Table 8-12 & 8-15: interpolation filters.

## Signalling of ARC

The capabilities of the proposed ARC solution are:

- Support any resampling ratio

- Support spatial resampling of all picture types (intra or inter).

- Support reference picture resampling depending on the spatial ratio of the reference picture to the current picture that is encoded.

- Support motion vector scaling, so that motion vectors pointing to reference pictures with different sizes can be used for prediction.

The proposed changes to the PPS are:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| adaptive_resolution_flag | u(1) |
| If(adaptive_resolution_flag) { |  |
| resampled_pic_width_in_luma_samples | ue(v) |
| resampled_pic_height_in_luma_samples | ue(v) |
| } |  |
| if( !single_tile_in_pic_flag ) { |  |
| … |  |
| } |  |

## Description of filters

JVET-M0135 lists the following filters for scaling by a factor of 2. In fact, the downsampling filter is explicitly given, but for the upsampling, they refer to “additional samples at odd grid positions are generated using the half-pel motion compensation interpolation filter coefficients in the latest VVC WD.” While that is descriptive though not very explicit, we infer (perhaps inaccurately) that this refers to the 0-phase filter in Table 8-14, which is for 1/16th pel accuracy motion compensation luma interpolation, given below. In any event, for definiteness, we compare our results with that obtained with the filters below as anchor.

Anchor:

- Downsampling: [ −1  9  16  9  −1 ]/ 32; which is a 5-tap zero phase filter

- Upsampling: [-1  4  -11  40  40  -11  4  -1]/64;  an 8-tap filter from Table 8-9 in VVC WD

This appears to have a modest downsampler, but an expensive upsampler. By contrast, FastVDO will use a slightly more complicated 9-tap downsampler (which really has the complexity of a 7-tap, due to the two zeros) and 4-tap upsampling filters, with 32-phases (Table 8-12 & 8-15), to shift the phase and align the pixel positions.

FastVDO:

- Downsampling: [1 0 -8 16 46 16 -8 0 1] / 64    (complexity of 7-tap)

- Upsampling:

| Fractional sample position p | Upsampling filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | p [ 0 ] | p [ 1 ] | p [ 2 ] | p [ 3 ] |
| 1 | −1 | 63 | 2 | 0 |
| 2 | −2 | 62 | 4 | 0 |
| 3 | -2 | 60 | 7 | −1 |
| 4 | −2 | 58 | 10 | −2 |
| 5 | −3 | 57 | 12 | −2 |
| 6 | −4 | 56 | 14 | −2 |
| 7 | −4 | 55 | 15 | −2 |
| 8 | −4 | 54 | 16 | −2 |
| 9 | −5 | 53 | 18 | −2 |
| 10 | −6 | 52 | 20 | −2 |
| 11 | −6 | 49 | 24 | −3 |
| 12 | −6 | 46 | 28 | −4 |
| 13 | −5 | 44 | 29 | −4 |
| 14 | −4 | 42 | 30 | −4 |
| 15 | −4 | 39 | 33 | −4 |
| 16 | −4 | 36 | 36 | −4 |
| 17 | −4 | 33 | 39 | −4 |
| 18 | −4 | 30 | 42 | −4 |
| 19 | −4 | 29 | 44 | −5 |
| 20 | −4 | 28 | 46 | −6 |
| 21 | −3 | 24 | 49 | −6 |
| 22 | −2 | 20 | 52 | −6 |
| 23 | −2 | 18 | 53 | −5 |
| 24 | −2 | 16 | 54 | −4 |
| 25 | −2 | 15 | 55 | −4 |
| 26 | −2 | 14 | 56 | −4 |
| 27 | −2 | 12 | 57 | −3 |
| 28 | −2 | 10 | 58 | −2 |
| 29 | −1 | 7 | 60 | −2 |
| 30 | 0 | 4 | 62 | −2 |
| 31 | 0 | 2 | 63 | −1 |

Table 1. 4-tap upsampler with 32 phases

For comparison with the anchor we will be using the 0-phase filter in Table 1 which is essentially [-4 36 36 -4]/64 or [-1 9 9 -1] / 16.. Also note that this filter is quite close to the simplest possible interpolator: the two-tap filter [1 1]/2. A tantalizing question, not studied yet, is whether we can get good results even with the simplest possible upsampler.  Importantly, note that since both sets of filters use upsamplers already in the draft specification, this is in part a comparison of downsamplers. See references [5-8] for more discussion.

## Simulation Results

One second clips from seven sequences (from the JVET test sequences list) were used for the filter study. This will actually suffice since in fact we are using All-Intra coding, and the gain are seen at every frame, as will be shown. Each of the sequences were downsampled by a factor of 2 using the JVET-M0135 and our filters. We report results for both cases: no compression, and with compression, using All Intra coding. The downsampled sequences were then encoded at QP 22,27,32 and 37 using VTM 4.0 with the All Intra Main10 profile. The binaries were then decoded and upsampled to generate the reconstructed videos. PSNR comparisons were made for all sequences at the source resolution with the original sequences as reference. A study was also conducted to evaluate the performance of the filters without encoding. Table 1 describes the results of the filter study without encoding between the downscaling and upscaling process. An average gain of ~0.4dB can be observed for the luma (Y) with the use of our filters. Furthermore, the gains increase with resolution, up to ~0.8dB for one 4K 10-bit sequence (“ParkRunning”). Table 2 plots the frame by frame gain for 1 second of the BasketballDrive sequence without compression. As described above the gain is consistent. Tables 3 and 4 do the same for encoding at Qp 22. Table 5 describes the results of the filter study with encoding with QP=22,27,32 & 37 introduced between the downscaling and upscaling process. An average BD-rate gain of ~2.3% can be observed for the luma (Y) with the use of our filters.  As an example result, Table 6 describes the Y-PSNR(dB) vs bitrate (KBps) plot for 1 second of the BasketballDrive sequence with compression. A consistent gain is observed for all bitrates. This is reassuring, in that after the initial loss due to resampling, the gain due to the better filters is essentially retained intact under compression. More extensive results and visuals will be presented at meeting.

Table 1. Filter comparison without encoding

Table 2. Frame by frame gain (dB) for BasketballDrive sequence without compression

Table 3. Filter comparison with encoding

Table 4. Frame by frame gain (dB) for BasketballDrive sequence with compression

Table 5. BD-rate gain with encoding at 4 Qp values.

Table 4. Y-PSNR(dB) vs bitrate(KBps) plot for BasketballDrive sequence with compression

## Visual Evidence

Figure 1. Visual evidence using the Camp Fire sequence, All Intra coded at Qp=22. Error in representing text/graphics using the Anchor filters,upper, which does not appear in the test filters, lower.

Figure 2. Visual evidence from the BasketballDrive sequence, All Intra coded at Qp=22, frame 38. Loss of sharpness in details (background wood tiles, number on jersey) using the Anchor filters, on the left, which improves with the test filters, on the right.

Figure 3. Example coding of catRobot sequence, All Intra, Qp=22, frame 16. Anchor filters on the left generate more aliasing artifacts than the test filters, on the right.

| 0048 | No filters proposed |
| --- | --- |
| 0052 | Down: [41,26,7,-5,-6,1], symmetric. Table 8-9, 8-6 (chroma) up, phase 0. |
| 0118 | Down: [16,10,0,-3,0,1], symmetric. Up: [-4,54,14,-2]/64. |
| 0279 | No filters proposed |
| 0532 | Down:[46,16, -8, 0, 1], symmetric. Up: Table 8-6, phase 0. |

Table 5. Proposed filters for ARC and document numbers from previous meeting cycle.

# Patent rights declaration(s)

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-M0135-v1, “On adaptive resolution change (ARC) for VVC”, Hendry, T. Davies, Y. Sun et al. , 13th Meeting: Marrakech, MA, 9–18 Jan. 2019

[2] ITU-T H.263 (01/2005): Video coding for low bit rate communication, Annex O “Temporal, SNR and Spatial Scalability mode” and Annex P “Reference Picture Resampling”

[3] JCTVC-F158, “Resolution switching for coding efficiency and resilience”, T. Davies (Cisco), JCTVC 6th Meeting, Turin, IT, 14-22 July 2011

[4] JVET-N1001v9, “Versatile Video Coding(Draft 5)”, B. Bross, J. Chen, S. Liu, 14th Meeting: Geneva, CH, 19–27 Mar. 2019

[5] JCTVC-F618, “Advanced Resampling Filters for Spatial Scalability and Chroma Sampling Applications in HEVC”, W. Dai, M. Krihnan, P. Topiwala (FastVDO LLC), JCTVC 6th Meeting, Turin, IT, 14-22 July 2011

[6] JVT-V030, “Advanced Extended Spatial Re-sampling Filters For SVC”, Trac D. Tran, L. Liu, P. Topiwala (FastVDO), JVT 22nd Meeting: Marrakech, Morocco, 13–19 January, 2007

[7] JCTVC-G862, “Advanced Resampling Filters for HEVC Applications”, W. Dai, M.Krihnan, P. Topiwala (FastVDO), JCT-VC 7th Meeting, Geneva, CH,  21-30 November, 2011.

[8] JCTVC-G264, “AHG18: Adaptive Resolution Coding,” T. Davies (Cisco), P. Topiwala (FastVDO), JCT-VC 7th Meeting, Geneva, CH,  21-30 November, 2011.