| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0320-v3 |
| --- | --- |

| Title: | Non-CE3: CCLM simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo Yanzhuo Ma Shuai Wan Xinwei Li Fuzheng Yang YuanFang Yu Yang Liu | Email: | jyhuo@mail.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn xinweili@stu.xidian.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

The current VVC derives the CCLM parameters with at most 4 neighbouring samples. In this contribution, simplified and unified CCLM is designed according to the number of available neighbouring chroma samples. Specifically, when the available neighbouring chroma samples is greater than or equal to 4, the CCLM is performed; otherwise, the prediction samples is set to the default value. In this way, the CCLM parameters are always derived from 4 neighbouring samples. The results reportedly shows 0.00%, 0.02%, and 0.02% BD-rates on Y, Cb, and Cr components in All Intra configuration and 0.00%, 0.04%, and 0.02% BD-rates on Y, Cb, and Cr components in RandomAccess configuration compared to VTM5.0.

- Introduction

In VTM5.0, the CCLM parameters are derived with at most four neighbouring chroma samples and their corresponding down-sampled luma samples.

Firstly, the available neighboring chroma samples on the top and top-right is obtained, labeled as numSampT and the number of available neighboring chroma samples on the left and left-below is labeled as numSampL.

Then, a subset neighboring samples are selected and the number of selected neighboring samples are labeled as cntT and cntL.

In current VVC, three different operations are employed according to numSampT, numSampL, cntT and cntL, as shown in Figure 1.

Figure 1 CCLM prediction process in VVC

# Proposed method

In VTM 5.0, when cntT+cntL is 2, 2 samples are selected and additional copy operations are envoked to derive 4 samples, after that, the same operations can be performed as when the number of selected sample points is 4.

We propose a simplified and unified CCLM method, as shown in Figure 2. The CCLM is performed only when the number of available neighbouring chroma samples is greater than or equal to 4, otherwise the chroma prediction is set to the default value, i.e. 1<<(BitDepthC-1). In addition, this method guarantees that there are always 4 samples for linear model parameters derivation.

Figure 2 CCLM prediction process in the proposed method

When the number of available neighbouring chroma samples is 2, this method removes several operations, including additional copy operations, comparisons, grouping, averaging, parameter derivation, and linear calculations. This method reduces the computational complexity while unify the CCLM. In this way, the CCLM parameters are always derived from 4 neighbouring samples.

# Experimental results

The proposed scheme is implemented based on VTM5.0. The coding performance is provided in Table 1.

Anchor: VTM5.0

Tested: The proposed method

Table 1. Summary of experimental results

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.02% | 99% | 99% |
| Class A2 | 0.01% | -0.03% | 0.03% | 100% | 100% |
| Class B | 0.00% | 0.00% | -0.02% | 100% | 100% |
| Class C | 0.00% | 0.11% | 0.04% | 100% | 100% |
| Class E | -0.01% | -0.01% | 0.07% | 100% | 100% |
| Overall | 0.00% | 0.02% | 0.02% | 100% | 100% |
| Class D | -0.01% | 0.15% | 0.01% | 100% | 100% |
| Class F | -0.01% | 0.00% | 0.03% | 100% | 99% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | 0.11% | 100% | 100% |
| Class A2 | 0.01% | -0.01% | 0.06% | 100% | 100% |
| Class B | 0.00% | 0.12% | -0.08% | 100% | 100% |
| Class C | -0.01% | 0.11% | 0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.02% | 100% | 100% |
| Class D | -0.01% | 0.09% | 0.42% | 100% | 100% |
| Class F | 0.02% | 0.05% | 0.21% | 100% | 100% |

# Conclusions

This contribution presents a unified CCLM method where the CCLM is not used when the number of available neighbouring chroma samples is less than 4. This method can eliminate selection operation, the linear model parameter derivation operations and additional copy operations in VTM5.0 when the number of samples is 2. At the same time, there are always 4 samples for linear model parameter derivation. The proposed method can get 0.00%, 0.02%, and 0.02% BD-rates on Y, Cb and Cr components for All Intra configuration and 0.00%, 0.04%, and 0.02% BD-rates on Y, Cb and Cr components for RandomAccess configuration. The encoding time and decoding time is the same as VTM 5.0. We recommend including the proposed method to the next version of WD and VTM.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).