| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0321 |
| --- | --- |

| Title: | Non-CE3: Removal of MIP mapping table |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo Yanzhuo Ma Shuai Wan Xinwei Li Haixin Wang Fuzheng Yang YuanFang Yu Yang Liu Ming Li | Email: | jyhuo@mail.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn xwli_xidian@stu.xidian.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com myron.li@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

In VVC, MPM list construction and DM mode derivation need to use the intra mode of neighboring blocks. The neighboring block may encode with conventional intra prediction or MIP mode. In this way, two mapping tables are needed to translate between the MIP modes and the conventional intra prediction modes. In this contribution, it is proposed to remove mapping table and replace them with a fixed mode index. This method can save the memory required to store the mapping table and save the operations of looking up the table.

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time over VTM5.0:

- Test1: Replace the MIP-to-conventional mapping table with a fixed mode index PLANAR_IDX.

  - For AI configuration: 0.01%, -0.02%, and 0.04%, with 100% EncT, 100% DecT;

  - For RA configuration: 0.01%, 0.09%, and 0.06%, with 100% EncT, 100% DecT;

- Test2: Replace the conventional-to-MIP mapping table with a fixed mode index 0.

  - For AI configuration: 0.00%, -0.04%, and -0.01%, with 99% EncT, 100% DecT;

  - For RA configuration: 0.00%, -0.09%, and -0.02%, with 100% EncT, 100% DecT;

- Introduction

The current VVC includes a Matrix-based Intra Prediction(MIP) technology. This technology generates the luma intra prediction signal out of one line of reference samples left and above a current block by a matrix vector multiplication and the addition of an offset. MIP divides Coding Units (CU) into three types according to their size.

typeW,H=0for W=H=41for maxW,H=82formaxW,H>8.

Two mapping tables between conventional intra prediction modes and MIP modes are used to signal MIP technology and adapt to other technologies:

- MIP-to-conventional mapping table

- Derivation of MPM-list: For each Coding Unit (CU), if a conventional intra prediction mode is to be applied, the index of the conventional intra prediction mode is signaled using a MPM-list with 6 MPMS. The derivation of the MPMs is performed using the intra-modes of the above and the left CU. For the MPM-list derivation, when a neighboring luma block is encountered which uses an MIP mode, this block is treated as if it was using the conventional intra prediction mode. The MIP-to-conventional mapping table is provided in Table 1.

- Derivation of DM mode: In chroma intra prediction, DM is an effective mode which directly uses the intra prediction mode of the co-located luma block. However, for a co-located luma block, if a MIP mode is applied, the MIP mode needs to be mapped into a conventional intra prediction mode according to the MIP-to-conventional mapping table as shown in Table 1.

Table 1. MIP-to- Conventional mapping table

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 0 | 0 | 1 |
| 1 | 18 | 1 | 1 |
| 2 | 18 | 0 | 1 |
| 3 | 0 | 1 | 1 |
| 4 | 18 | 0 | 18 |
| 5 | 0 | 22 | 0 |
| 6 | 12 | 18 | 1 |
| 7 | 0 | 18 | 0 |
| 8 | 18 | 1 | 1 |
| 9 | 2 | 0 | 50 |
| 10 | 18 | 1 | 0 |
| 11 | 12 | 0 |  |
| 12 | 18 | 1 |  |
| 13 | 18 | 0 |  |
| 14 | 1 | 44 |  |
| 15 | 18 | 0 |  |
| 16 | 18 | 50 |  |
| 17 | 0 | 1 |  |
| 18 | 0 | 0 |  |
| 19 | 50 |  |  |
| 20 | 0 |  |  |
| 21 | 50 |  |  |
| 22 | 0 |  |  |
| 23 | 56 |  |  |
| 24 | 0 |  |  |
| 25 | 50 |  |  |
| 26 | 66 |  |  |
| 27 | 50 |  |  |
| 28 | 56 |  |  |
| 29 | 50 |  |  |
| 30 | 50 |  |  |
| 31 | 1 |  |  |
| 32 | 50 |  |  |
| 33 | 50 |  |  |
| 34 | 50 |  |  |

- Conventional-to-MIP mapping table

- Derivation of MIPMPM-list: For each Coding Unit (CU), if an MIP mode is to be applied, the index of the MIP mode is signaled using a MIPMPM-list with 3 MPMS. The derivation of the MIPMPMs is performed using the intra-modes of the above and the left CU. However, for a left CU, if a conventional intra mode is applied and the size type is same to current PU, the conventional intra prediction mode needs to be mapped into a MIP mode according to the conventional-to-MIP mapping table as shown in Table 2; for an above CU, if it belongs to the same CTU as the current CU and a conventional intra mode is applied and the size type is same to current PU, the conventional intra prediction mode needs to be mapped into a MIP mode according to the conventional-to-MIP mapping table as shown in Table 2.

Table 2. Conventional-to-MIP mapping table

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 17 | 0 | 5 |
| 1 | 17 | 0 | 1 |
| 2, 3 | 17 | 10 | 3 |
| 4, 5 | 9 | 10 | 3 |
| 6,7 | 9 | 10 | 3 |
| 8, 9 | 9 | 10 | 3 |
| 10, 11 | 9 | 10 | 0 |
| 12, 13 | 17 | 4 | 0 |
| 14, 15 | 17 | 6 | 0 |
| 16, 17 | 17 | 7 | 4 |
| 18, 19 | 17 | 7 | 4 |
| 20, 21 | 17 | 7 | 4 |
| 22, 23 | 17 | 5 | 5 |
| 24, 25 | 17 | 5 | 1 |
| 26, 27 | 5 | 0 | 1 |
| 28, 29 | 5 | 0 | 1 |
| 30, 31 | 5 | 3 | 1 |
| 32, 33 | 5 | 3 | 1 |
| 34, 35 | 34 | 12 | 6 |
| 36, 37 | 22 | 12 | 6 |
| 38, 39 | 22 | 12 | 6 |
| 40, 41 | 22 | 12 | 6 |
| 42, 43 | 22 | 14 | 6 |
| 44, 45 | 34 | 14 | 10 |
| 46, 47 | 34 | 14 | 10 |
| 48, 49 | 34 | 16 | 9 |
| 50, 51 | 34 | 16 | 9 |
| 52, 53 | 34 | 16 | 9 |
| 54, 55 | 34 | 15 | 9 |
| 56, 57 | 34 | 13 | 9 |
| 58, 59 | 26 | 1 | 8 |
| 60, 61 | 26 | 1 | 8 |
| 62, 63 | 26 | 1 | 8 |
| 64, 65 | 26 | 1 | 8 |
| 66 | 26 | 1 | 8 |

# Proposed method

In this contribution, it is proposed to remove mapping table and replace them with a fixed mode index.

- Test1: Replace the MIP-to-conventional mapping table with a fixed mode index PLANAR_IDX.

For the MPM-list derivation and DM mode derivation, when a MIP mode needs to be mapped to a conventional intra prediction mod mode using the MIP-to-conventional mapping table shown in Table 1, the modification is to map the MIP mode to the PLANAR mode with index 0. The modified MIP-to-conventional mapping table is shown in Table 3. Since all the MIP modes are mapped to the PLANAR mode with index 0, this MIP-to-conventional mapping table can be removed.

Table 3. MIP-to-conventional mapping table

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0-10 | 0 | 0 | 0 |
| 11-18 |  |  |  |
| 19-34 |  |  |  |

- Test2: Replace the conventional-to-MIP mapping table with a fixed mode index 0.

For the MIPMPM-list derivation, when a conventional intra prediction mode needs to be mapped to a MIP mode using the conventional-to-MIP mapping table shown in Table 2, the modification is to map the conventional intra prediction mode to the MIP mode with index 0. The modified conventional-to-MIP mapping table is shown in Table 4. Since all the conventional modes are mapped to the MIP mode with index 0, this conventional-to-MIP mapping table can be removed.

Table 4. Modified conventional-to-MIP mapping table

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0-66 | 0 | 0 | 0 |

# Experimental results

The proposed scheme is implemented based on VTM5.0. The coding performance is provided in Table 5 and Table 6.

Anchor: VTM5.0

Test1: Replace the MIP-to- conventional mapping table with a fixed mode index PLANAR_IDX.

Table 5. Experimental results of Test 1

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.04% | 100% | 100% |
| Class A2 | 0.03% | 0.02% | -0.02% | 99% | 100% |
| Class B | 0.01% | -0.04% | 0.09% | 99% | 100% |
| Class C | 0.00% | -0.12% | 0.05% | 101% | 101% |
| Class E | 0.03% | 0.01% | 0.03% | 99% | 99% |
| Overall | 0.01% | -0.02% | 0.04% | 100% | 100% |
| Class D | -0.03% | 0.19% | 0.00% | 100% | 99% |
| Class F | -0.02% | -0.02% | 0.08% | 101% | 100% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.22% | 99% | 100% |
| Class A2 | 0.07% | 0.22% | 0.06% | 100% | 100% |
| Class B | 0.01% | 0.04% | 0.01% | 100% | 100% |
| Class C | -0.02% | 0.15% | 0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.06% | 100% | 100% |
| Class D | 0.00% | 0.07% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.13% | 0.16% | 100% | 100% |

Anchor: VTM5.0

Test2: Replace the conventional-to-MIP mapping table with a fixed mode index 0.

Table 6. Experimental results of Test 2

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.03% | 99% | 100% |
| Class A2 | 0.00% | -0.01% | -0.02% | 99% | 99% |
| Class B | 0.01% | -0.04% | -0.01% | 99% | 100% |
| Class C | 0.01% | -0.09% | -0.11% | 100% | 100% |
| Class E | 0.00% | -0.04% | 0.10% | 98% | 100% |
| Overall | 0.00% | -0.04% | -0.01% | 99% | 100% |
| Class D | 0.00% | 0.14% | -0.03% | 100% | 100% |
| Class F | -0.01% | 0.00% | 0.05% | 101% | 101% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | 0.13% | 100% | 100% |
| Class A2 | 0.02% | -0.07% | -0.02% | 100% | 100% |
| Class B | 0.02% | -0.02% | -0.14% | 100% | 100% |
| Class C | -0.03% | -0.19% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.09% | -0.02% | 100% | 100% |
| Class D | 0.01% | -0.20% | 0.15% | 100% | 100% |
| Class F | 0.03% | -0.06% | 0.02% | 100% | 100% |

# Conclusions

This contribution presents a simplified method to remove mapping table in MIP. It can be seen that: for test 1, which replace the MIP-to-conventional mapping table with a fixed mode index PLANAR_IDX, can get 0.01%, -0.02%, and 0.04% BD-bitrate; test 2, which replace the conventional-to-MIP mapping table with a fixed mode index 0, can get 0.00%, -0.04%, and -0.01% BD-bitrate. We propose to study this method in CE.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).